#

File: remote.doc,  Node: Top,  Next: Disclaimer, Up: (dir)

Remote.bg

a general remote control program

ver 3.2

1993-09-09

Made by:

_

Bjorn Gahm

d89-bga@nada.kth.se

******************************************************************************

INDEX

******************************************************************************

* Menu:

* Disclaimer::						1.1

* Update info::						1.2

* General remote info::					1.3

* General info::					1.4

* Getting started::					2.1

* The general remote control program:: 			2.2

* How to customize the general remote control program:: 2.3

* Tips on saving memory::				3.1

* Using alarms::					3.2

The following chapters are for people that wants to know more about how

the program works

* How to use the transmit program::			4.1

* Read a code from a remote control::			4.2

* Edit an infostring::					4.3

* Learning a new remote control::			4.4

(how to create a new infostring, this part is NOT

working well due to the speed of the HP48)

* Contents of the directories on the HP::		Appendix A

* Contents of the infostring::				Appendix B

#

File: remote.doc,  Node: Disclaimer,  Next: Update info, Prev: Top, Up: Top

*******************************************************************************

1.1 DISCLAIMER

*******************************************************************************

WARNING!!!!

Even though this program works fine with my hp48SX version E I cannot be

responsible for memory loss or even hadware damage on your HP48.

I also appologize for any errors in the language since english

is not my native language.

This package is freeware. That means that it can be copied freely and may

not be sold. If you give away a copy, please copy the whole package.

#

File: remote.doc,  Node: Update info,  Next: General remote info, Prev: Disclaimer, Up: Top

*******************************************************************************

1.2 UPDATE INFO

*******************************************************************************

Differences from 3.1.1:

* The display routines are rewritten to support the G(X) series.

* The transmission program is rewritten for the G(X) series.

* All graphics now only uses the text grob.

Differences from 3.1:

* It is possible to switch remote using left and right arrow when a remote is

displayed.

* The switching between remotes is faster.

Differences from 3.0b:

* the program is faster now (the drawing is made in sysrpl + ml)

* The learning programs has been rewritten to work more reliably.

* The infostring format is rewritten (again). But 3.0 strings still work.

* The TRANSMIT program checks the string so it wont use strings older than

3.0 since older strings will get the HP to lock up.

* The general remote program has been changed to use various datasizes

and not just maximum of 16 bits. (It was needed for Funai).

* A bug with older technics and panasonic remotes fixed (the bug was introduced

in ver 3.0).

* Pulsemodulated codes can now be sampled right. (Sony codes can now be sampled

right).

Differences from 2.0:

* It can now learn new remotes by itself. (Only if the remote uses

pulse or space modulation)

* The length of code to transmit has been increased from 48 to 63 bits

* The length of pulses and spaces has been increased (the previos versions

could not cope with canon codes)

* The infostring has been changed so old infostrings must be converted.

* All routines involving displaying info has been changed to display the

info in microseconds instead of loops.

* The TRANSMIT program now cleans the stack, so there is no need for

the DROP2 after TRANSMIT that were needed in previous versions

* There is a general remoteprogram included, to make it easier

to customize your hp to be a remote control.

#

File: remote.doc,  Node: General remote info,  Next: General info, Prev: Update info, Up: Top

*******************************************************************************

1.3 GENERAL REMOTE INFO

*******************************************************************************

Most of the remotes that uses IR to transmit signals uses same kind of coding.

The signal is binary and varies in length (both in time and bit length).

The remotes transmits pulses of light. When the remote control transmits light,

the light is modulated with a frequecy ~40kHz. This is done so that the

recieving unit can cut out other IR sources such as normal lamps.

The most common way to code the signals is to vary the length of the pulses or

the length of the spaces or alter the order between pulses and spaces.

* A pulse coded signal varies the length of the pulses:

+--+  +--+  +----+  +----+     Sony uses this kind of coding

|  |  |  |  |    |  |    |

--+  +--+  +--+    +--+    +--

0     0      1       1

* A space coded signal varies the length of the spaces:

+--+  +--+    +--+    +--+     Panasonic uses this kind of coding

|  |  |  |    |  |    |  |

--+  +--+  +----+  +----+  +--

0       1       1

* A shift coded signal alters the order of pulse space:

+--+  +--+--+     +--+         Philips uses this kind of coding

|  |  |     |     |  |

--+  +--+     +--+--+  +

0        1     0

In addition to this are headers often used. A header is a pulse that are

transmited before the actual code, and is used to activate the reciever.

The header is always the same.

+----------+    +--+  +--+

|          |    |  |  |  |

+          +----+  +--+  +......

\______________/\_________......

Header          Code

The header and code are transmitted over and over as long as a button is

pressed on the remotecontrol.

A normal repetition time is ~50ms.

The actual code is normally divided into two parts:

1. An Adress  (selects which equipment to send to, called GROUP in my docs)

2. A Command  (tells the equipment what to do, called FUNCTION in my docs)

In addition to this some remotes alters the codes between transmissions.

And this is not captured by the reading routines.

#

File: remote.doc,  Node: General info,  Next: Getting started, Prev: General remote info, Up: Top

*******************************************************************************

1.4 GENERAL INFO

*******************************************************************************

The only problem is that all the remotes I have tried have

different codings. (And I think that there are many more

codings that I haven't seen yet).

It also seems that some remotes use different timing in Europe and US.

This is the case with older Panasonic and Technics remotes.

The types of codings that the program can generate is:

* Pulse modulated (the length of the pulse is the coding

1 is longer pulse, 0 is shorter)

* Space modulated (the space between the pulses is the

coding)

* Shift modulated (1 is space+pulse and 0 pulse+space or

or vice versa).

The code can be reversed.

The codelength can vary between 1 and 63 bits.

An inverted copy can be added to the code.

The number of repetitions can be set between 0 and 15

(it is useful whe you want the hp48 transmit a signal

as a batch job, since 1 transmission of the code is most of the

time not enough for the reciever to recognize)

There is a read function to read codes from known remotes.

The carrier frequency generated by S(X) series are 32kHz.

This means that some remotes will not work good.

The G(X) series generate a carrier frequency of 38kHz.

This means that the G(X) series can produce a signal that is more

accurate than S(X) series can do.

There is a program to learn new remotes that are not supported yet.

But that part is not stable yet (I have tried that part on some remotes and

it is not working all the time). This is due to the sampling theorem.

If the signal is to be smpled correctly the HP must sample it with a rate

that is atleast two times higher than the highest frequency. That is not

possible on a HP48. The S(X) series can sample in approx 29kHz which is

less than half the required speed. Therefore cannot the HP sample the signals

good. The G(X) samples in 44 kHz.

But the signal is read and with adjustments it is possible to make a

fuctional remote. This is not an easy task though.

#

File: remote.doc,  Node: Getting started,  Next: The general remote control program, Prev: General info, Up: Top

*******************************************************************************

2.1 GETTING STARTED

*******************************************************************************

The package contains (as HP binaries) in hp48bin:

remote      - The main directory.

make        - Contains all the programs for reading codes from remotes.

learn       - Contains Programs for learning new remotes (this part is not

functioning well but is included to play around with).

transmit.gx - If you have a GX or a G then replace the TRANSMIT program in

the REMOTE dir on the HP with this code (call it TRANSMIT when

storing it in the REMOTE dir.

MtoT.gx     - This is the timing program to change in order to sample with a

GX. Download this program to the HP48G(X) and replace M->T 's

contents with this program. M->T is located in MAKE/READ/.

REMOTES     - that is a directory with some remotes in to start with.

They are divided in brands with documentation on each brand.

Download remote to the HP.

In REMOTE you find Start and REMOTES.

REMOTES is the directory containing all the remotes installed.

When the program is loaded to the HP the first time it is empty.

You have to go to the REMOTES dir on the HP and download the remotes

you want to use.

They are included in a separate directory in the package.

Once that is done you can start the program by pressing Start in the

REMOTE (main) dir on the HP.

I use this little program assinged to blue shift NXT to start the remote

program:

<<

RCLMENU

PATH

HOME REMOTE Start

EVAL

MENU

>>

It will leave the HP in the same status as it was before the REMOTE program

was started. And it makes it possible to start it from any directory.

If you want to read codes from a remote you have to download MAKE also.

It must be stored in the REMOTE dir.

#

File: remote.doc,  Node: The general remote control program,  Next: How to customize the general remote control program, Prev: Getting started, Up: Top

*******************************************************************************

2.2 THE GENERAL REMOTE CONTROL PROGRAM

*******************************************************************************

The reason for me to make this program was to make it possible to exchange

complete remotes.

Every remote with up to 72 buttons will take approx 500-800 bytes.

Once the program is started the menubar will be a selector between remotes.

Pressing one of the menu keys will customize the HP to be a remote.

Eg. Press VIDEO. (if you have JVC VIDEO installed in the REMOTES dir).

Now the display will show 8 meny rows on the screen, and only one of them is

displayed as a normal menubar (all the others is inverted).

##REW## #PLAY## ##FF### #STOP## #PAUSE# ##REC##

OPERA    CH-     CH+            SLO-    SLO+

PRGM    CANC    GOTO    DISP    CRES    SP/LP

AMON    EJECT

All the keys now have three functions:

1. If pressed unshifted it will transmit the code.

2. If pressed with orange shift left it will let you alter the code.

3. If pressed with blue shift right it will read a code from a remote.

By pressing Arrow up or Arrow down you can switch to the other menu rows.

The row that is normal is using the keys A-F to transmit the code.

To play the video press B.

To turn on the video press Arrow Down and then A.

To end the remote program just press ON.

Blue shift right and ON will switch off the HP (as normal) without leaving

the remote program.

To switch faster between remotes it is possible to press rigth and left arrow.

The program will switch to remotes in the order they are in the REMOTES

directory.

The advantage of using this program is that when you have sampled a remote

and made a directory for it, it is easy to pass it on to others since

all the information about the remote is in that directory.

That makes it easy to trade remotes with others.

I would be glad to have a copy of new working remotes so that I can keep

a database of all the remotes this program can cope with.

#

File: remote.doc,  Node: How to customize the general remote control program,  Next: How to use more than one remote in one dir, Prev: The general remote control program, Up: Top

*******************************************************************************

2.3 HOW TO CUSTOMIZE THE GENERAL REMOTE PROGRAM

*******************************************************************************

In every remote directory there must be:

A CODE var that holds the infostring. (it is a string)

A DATA var that holds the codes for the remote. (it is a string)

a MD var that holds the data for the meny. (it is a list)

I have set the default length of the codes to 16 bits. (This is to save memory)

Some remotes have codes that are longer than 16 bits, and they need a GROP var

that contains the first part of the code.

If the GROUP is needed then it is written in the documentation for that remote.

(There is a GROUP after every type of equipment)

************* ENTER CODES *****************************************************

To enter new commands just select the right menurow and press orange shift left

and the right key.

If it is one of the menukeys you will be able to alter the command name first

and then alter the code.

If you don't want to alter the command name just press ENTER, otherwise

alter the name and then press ENTER.

If it is the ENTER key row and down you will only be able to alter the keys

code.

************** READING CODES **************************************************

This assumes that you have the MAKE directory installed.

*** This option is not working on the GX at the moment. ***

To read new codes just select the right menurow and press blue shift right

and the right key.

If it is a menu row the program will let you alter the name of the command.

Once that is done the HP will turn off the screen and wait for a code from the

remote.

Try to hold the remote as close as possible to the IRport on the HP.

If you have a SX take the cover off. Try also to find exactly where the

transmitting diod is on the remotecontrol in order to put that as close as

possible to the IR port on the hp.

Press the right key on the remote.

Wait a while, then you will be able to see the old code stored and the read

code.

On the bottom row you will have the code to store. Normally you just press

enter and the code will be stored.

Be aware of one thing: If the code Read: #...h contains a binary with more

than 4 numbers check that the starting numbers are the same as the ones

in the GROUP var. If not you will not get the right codes.

************** CREATING A NEW REMOTE ******************************************

To add a new remote to this program, make a copy of one of the directories

(take the VIDEO directory or the PVIDEO directory if the code that you are

going to use is longer than 16 bits)

In every remote directory there must be:

CODE  - that holds the infostring that is to be used.

MD    - holds the list to construct the menus (to begin from scratch

store a empty list in MD).

DATA  - contains the codes (you don't have to edit this var unless you change

the Size var (below)).

The DATA var is a string with 78 entries x Size (default 2) = 156 bytes long.

The first 48 entries is for the menusystem, and the last 30 is for the keys

from enter and down. They are acutally five keys per row but DATA sees that

as if there were six key per row (easier decoding of the keys).

So if you want to get the code for key 8 it will be at (48 + 9) x Size and

it will be Size long.

Optional vars:

Size  - (if you are to use codes that are longer than 32 bits and need to

have different GROUP codes for different keys).

it should be number of bits / 8. The default Size is 2 (16 bits).

If this is change to a bigger value then the DATA var must be

changed. It is a string that must be Size * 78 long.

GROUP - if the code is longer than 16 bits but the first part of the

code is identical for all the codes for one remote.

(this is the most common vay of coding)

CONV  - is used to convert a code to the right format after it is read

by the HP.

If you are to use codes longer than 16 bits but only the end of the code

differs then use the REM program in PANASONIC VIDEO directory.

You have to alter the prefix code in the REM program.

Make shure that you store the REM progam in the right directory and NOT in

the REMOTE directory, because there is a default REM program there.

Or alter the directories that exsists.

#

File: remote.doc,  Node: Tips on saving memory,  Next: Using alarms, Prev: How to customize the general remote control program, Up: Top

*******************************************************************************

3.1 TIPS ON SAVING MEMORY

*******************************************************************************

If you don't have to learn new codes or new remotes, then you can purge the

MAKE directory. (MAKE takes approx 8kbytes of memory).

Just make shure to store the MAKE directory in the REMOTE directory when

loaded back in the HP, otherwise it will not work properly.

Every remote will take up approx 400-500 bytes.

The REMOTE directory will take up approx 3500bytes and more (depending on

how many remotes that are loaded in the system).

The only program that is used for transmission is the TRANSMIT program.

So if you want write your own programs you only need TRANSMIT and

the right infostrings.

#

File: remote.doc,  Node: Using alarms,  Next: How to use the transmit program, Prev: Tips on saving memory, Up: Top

*******************************************************************************

3.2 USING ALARMS

*******************************************************************************

I use the HP to wake me up in the morning.

It switches on the video on MTV.

To do this there is a small program in the REMOTE directory that is called

ALARM.

Use this a a executable when creating a alarm.

Just recall the program to the stack (by pressing blue shift right and ALARM).

And set the alarm in the TIME menu.

To use the alarm with other codes just alter the ALARM program before recalling

it to the stack and setting the alarm.

#

File: remote.doc,  Node: How to use the transmit program,  Next: Read a code from a remote control, Prev: Using alarms, Up: Top

*******************************************************************************

4.1 HOW TO USE THE TRANSMIT PROGRAM

*******************************************************************************

This chapter is for using the TRANSMIT program by itself.

Look in chapter Using the general remote program first.

To transmit a code just put the apropriate infostring on level 2

and the code on level 1 and press TRANSMIT.

(TRANSMIT is located in the main directory, the REMOTE directory)

Note: 	Some remotes needs more than one INFOSTRING to work properly.

Grundig uses 3 INFOSTRINGS.

The infostrings that I have managed to get so far are in the INFO

directory.

Eg to play a sony CD:

Press SONY  (SONY is in the INFO directory, a subdirectory to MAKE)

Enter #8B2h

Press TRANSMIT (TRANSMIT is in the main directory)

or in a program

<< REMOTE INFO SONY #8B2h TRANSMIT >>

WARNING WARNING WARNING!!!!

The TRANSMIT program just checks that the data is there but not if it makes

sense. If an invalid string is used the hp48 might transmit for a very long

time. The only way to stop the program then is to press the reset button, since

the program turns off the display and the interrupts.

However it can not destroy any memory becauce is does not write to the memory

except to the irled.

#

File: remote.doc,  Node: Read a code from a remote control,  Next: Edit an infostring, Prev: How to use the transmit program, Up: Top

*******************************************************************************

4.2 READ A CODE FROM A REMOTE CONTROL

*******************************************************************************

This assumes that you already have the right infostring. If you don't have that

see chapter 4.5.

To read a remote control, recall the right infosting and go to the READ

directiory in the MAKE directory.

Save the infostring in INFO.

Then press READIR (the screen will go blank).

Then hold the remote control close to the irport on the HP48

(as close as possible) and press the right button on the remote.

Now should the screen turn on again and after a half a minute

you will have a binary on level 1 that is the code.

If the code does not work the try with an another distance

between the hp and the remote control.

You might have to try several times.

It might also help to insert new batteries in the remote control.

#

File: remote.doc,  Node: Edit an infostring,  Next: Learning a new remote control, Prev: Read a code from a remote control, Up: Top

*******************************************************************************

4.3 EDIT AN INFOSTRING

*******************************************************************************

In order to view an infostring or edit it, the string must be on

level 1 on the stack.

To view, recall the string to the stack, go to the MAKE directory an press

Show.

All the commands that are used to make an infostring works in two ways.

The commands are HEADER, ONE, ZERO, LEN, SPACE, REV, Inv, Mod, REPS.

If the infostring is on level 1 on the stack then the commands will recall the

data.

If the infostring is on level 2 and the data is on level 1 then the commands

will alter the infostring and leave it on level 1.

To alter a infostring:

Put on stack:

Level 2: infostring

Level 1: data to change to

Press HEADER or ONE or ZERO or LEN ......

---------------------------------------

Read infostring:

HEADER ONE ZERO programs:

If the infostring is on level 1 on the stack then HEADER or ONE or ZERO will

reall the info for the header one or zero respectivily.

Eg.

HEADER Will produce:

_______________________      _______________________

|_______________________|    |_______________________|

|4:                     |    |4:                     |

|3:                     |    |3:                     |

|2:                     | -> |2: infostring          |

|1: infostring          |    |1: { aaa bbb n }       |

|xxx xxx xxx xxx xxx xxx|    |xxx xxx xxx xxx xxx xxx|

Where aaa is the time for the pulse in micro seconds,

bbb is the time for the space in micro seconds

and n is the number of repetitions of the header.

For sony string the list will be: { 2192 592 0 }

For ONE and ZERO n will be the order of transmission.

n=1 -> transmits pulse space

n=0 -> transmits space pulse

----------------------------------------

LEN:

Sets the length of the code.

2: infostring

1: real number

Just put the number of bits as a real and press LEN.

Eg.

2: infostring

1: 12

LEN

Will set the code length to 12 bits.

-----------------------------------------

SPACE:

Sets the space between the transmissions.

2: infostring

1: real number

Just put the length of the space as a real and press SPACE.

Eg.

2: infostring

1: 23000

SPACE

Will set the space between the transmissions to approx 23ms.

-----------------------------------------

Rev:

Sets if the code is to be reversed before transmission.

(Sets the transmission order, MSB first or LSB first)

Normally you don't have to change this parameter, but

it could make the codes more understandable if you change

the transmission order.

Eg to recall data.

Press Rev :

_______________________      _______________________

|_______________________|    |_______________________|

|4:                     |    |4:                     |

|3:                     |    |3:                     |

|2:                     | -> |2: infostring          |

|1: infostring          |    |1: n                   |

|xxx xxx xxx xxx xxx xxx|    |xxx xxx xxx xxx xxx xxx|

Where n is 1 or 0.

0 means transmit LSB first. (LSB = Least Significant Bit)

1 means transmit MSB first. (MSB = Most Significant Bit)

To alter put the number on level 1 and infostring on level 2:

Press Rev :

_______________________      _______________________

|_______________________|    |_______________________|

|4:                     |    |4:                     |

|3:                     |    |3:                     |

|2: infostring          | -> |2:                     |

|1: n                   |    |1: infostring          |

|xxx xxx xxx xxx xxx xxx|    |xxx xxx xxx xxx xxx xxx|

-----------------------------------------

Inv:

Works as Rev, but

0 means don't add an inverted copy of the code

1 means add an inverted copy of the code after the code (used by old panasonic

and technics remotes)

-----------------------------------------

Mod:

Same as above, but

0 means don't add a pluse after the code (used if the code is pulse modulated)

1 means add a pulse after the code (used if the code is space modulated)

#

File: remote.doc,  Node: Learning a new remote control,  Next: Appendix A, Prev: Edit an infostring, Up: Top

*******************************************************************************

4.4 LEARNING A NEW REMOTE

*******************************************************************************

The learning program has been improved since the previous version but it still

has problems.

It is not functioning good at all times.It has a higher probability to work

better on a G(X).

The problem is that the HP cannot sample fast enough, so the carrier frequency

that many remotes use could make the HP to decode the signal wrong.

I am trying to go around that but the fact remains that the sampling theorem

say that it can't be done since the HP48S(X) only can sample at a frequency

of 29kHz, and a G(X) can sample in approx 44kHz.

To learn a new remote, go to the READ directory in the MAKE directory.

There you will find LEARN Draw and TEST.

LEARN is the program to generate data to make a infostring.

Draw is used to draw the sampled signal on screen.

TEST will sample and draw the tree first transmissions.

Use TEST first. It will show you how the signal looks like. TEST is used

as LEARN below. TEST will draw the three first signals from the remote.

If they look the same then you should be able to generate a infostring.

One thing though, some remotes sends a stop code after the real code and that is

not shown by TEST.

Press LEARN and when the screen goes blank hold the remote control as close

as possible to the HP's irport (you should also take the cover off).

Press any button on the remote until the screen goes back on again.

Then wait a while (about a half minute).

|               |

|               |

|   Remote      |

|      Control  |

------        |

--------

-------------------

| ----------------- |       I have had best results in learning

||                 ||       new remotes when I took off the cover

||      HP48       ||       and held the remote close to the IR-port

||                 ||       and had the remote sligtly tilted to

||                 ||       the HP48.

| ----------------- |

|                   |

|                   |

|                   |

|                   |

|                   |

|                   |

|                   |

|                   |

-------------------

Now you will have a list on level 1.

The list will consist of:

{ n (h1,h2) (p1,p2) (s1,s2) (sp1,sp2) m }

Where:

n is the length of the code in bits.

(h1,h2) is the length of the header (of there are no header then this will be

missing).

(p1,p2) is the length of one codeing

(s1,s2) is the length of zero coding

(sp1,sp2) is the length between transmissions

m is the coding type 1 means space modulated, 0 means pulse modulated

All llengths are in miicro seconds.

If there are more than 4 complex numbers in the list then the sample was no

good. Then you have to try again.

I know that Bang&Olufsen will not work.

I have tried it on JVC, Sony, Funai and Canon with successful results.

When you have a list that seems right then press MKSTR.

now you will have a string that ought to work.

The list will be stored in L and the infostring will be stored in INFO.

Recall INFO and store it under a new name in the REMOTE directory

(Use the name of the brand).

Now you can do as in chapter 3.1 to learn codes.

If the infostring seems right but the codes doesnot work then the length of

the pulses might be too short.

Try to make the pulses a bit longer by altering them maually.

(using HEADER, ONE and ZERO in the MAKE directory).

I am trying to fix this timing problem.

It can also be that the remotecontrol sends controlbits before and/or after

the actual code.

This is the case with Yamaha and Grundig.

#

File: remote.doc,  Node: Appendix A,  Next: Appendix B, Prev: Learning a new remote control, Up: Top

*******************************************************************************

APPENDIX A    DIRECTORY CONTENTS

*******************************************************************************

Main directory contains:

REMOTES   - subdirectory with remotes

MAKE	  - subdirectory for editing infostrings (see below)

READ      - subdirectory for reading codes and remotes (see below)

INFO      - subdirectory with infostrings (see below)

TRANSMIT  - the transmission program (machine dependent, S(X) and

G(X) has different versions).

PRG

Put

Get

---------------------------------------------------------------

MAKE directory contains: (MAKE must be a subdirectory of REMOTE)

READ      - subdirectory for reading remote codes   (see below)

Show      - shows all the info from an infostring

HEADER    - shows or sets the header data

ONE       - shows or sets the 1 data

ZERO      - shows or sets the 0 data

LEN       - shows or sets the length

SPACE     - shows or sets the space between transmissions

REV       - toggles if the data is to be reversed

Inv       - toggles if an inverted copy is to be added

Mod       - toggels of a pulse is to be added after the code

REPS      - shows or sets the number of repetitions

SPOS      - Shows or sets from which sample to decode

Flags     - Shows or sets the flags that control of the transmit program

SCNV      - converts between time and loops for space

PCNV      - converts between time and loops for pulse

S->R

R->S

---------------------------------------------------------------

INFO directory contains: (all the infostrings that I have at the moment)

RC5       - Philips remote info

MATSU     - Panasonic and Technics info

OLDMATSU  - Panasonic and Technics info (Older equipment)

UOLDMATSU - Older Pana. and Technics US timing

PION      - Pioneer info

JVC       - JVC info

SONY      - Sony info

SONYCAM   - Sony cam info (15bit instead of 12bit datalength)

FUNAI     - Funai video info

YAMAHA    - Yamaha info

YAMAHA2

GRUNDIG   - Grundig info

GRUNDIG2

GRUNDIG3

FINLUX	  - Finlux info

FINLUX2

FINLUX3

---------------------------------------------------------------

READ directory contains: (READ must ba a subdirectory to MAKE)

READIR    - Reads a code for the infosting in INFO

TEST      - Does a test sample and draw the three first codes

Draw      - Draws the looks of a sampled signal

LEARN     - Reads the code and produces a list with the info needed

the list is stored in L

MKSTRING  - Makes a info sting of a list (in the L var, produced by LEARN)

INFO      - The infostring that are created or used by READIR

L         - The list created by LEARN

GETLEN    - Used by LEARN

T         - Used by LEARN, READIR and Draw

IRLEARN   - Mlcode used by LEARN, READIR and TEST

SAMPLE    - the string used by IRLEARN

REVERSE   - reverses the code

PPAR

#

File: remote.doc,  Node: Appendix B, Prev: Appendix A, Up: Top

*******************************************************************************

APPENDIX B    CONTENTS OF AN INFOSTRING

*******************************************************************************

To check or edit an infostring recall it to the stack and go to

MAKE directory.

Press Show to view all the information of the string.

Or press HEADER to see the header info

press ONE to see the 1 info and so on.

(Contents of an infostring see below)

Infostring: (25 chars)

1     2     3     4     5     6     7     8     9    10

|     |     |     |     |     |     |     |     |     |     |

|  |  |  |XX|  |  |  |XX|  |  |  |  |  |XX|  |  |  |XX|  |  |

| LSB | MSB | LSB | MSB |     | LSB | MSB | LSB | MSB |     |

|   Pulse   |   Space    Reps |   Pulse   |   Space   |Order|

|        Header               |           1                 |

|                             |                             |

11    12    13    14    15    16    17    18    19    20    21    22

|     |     |     |     |     |     |     |     |     |     |     |     |

|  |  |  |XX|  |  |  |XX|  |  |  |  |  |  |  |XX|  |  |  |  |  |  |  |  |

| LSB | MSB | LSB | MSB |     |     | LSB | MSB |     |     |     |     |

|   Pulse       Space    Order|Data |   Trans   |Rev  |Inv  |Add  |Repeat

|           0                 |Len  |   Space   |Data |Data |Pulse|     |

|                             |     |           |     |     |     |     |

The 3 last chars in the infostring are optional

23    24    25

|     |     |     |

|  |  |  |  |  |  |

|     |     |     |

|Sampl|   Flags   |

|Start|           |

Head: Pulse: length of pulse, if length = 0 then there are no header

Space: time to next bit

Reps: 0 = send header always  1-9 send header that number of times

1,0:  Pulse: length of pulse

Space: time to next bit

Order: 48 "0" => space pulse

: 49 "1" => pulse space

Data len   : length of data in bits

Trans space: specifies the time between the repetitions of the data

Rev Data   : 82 "R" => reversed

32 " " => not reversed

Inv Data   : 73 "I" => the data is reversed and added to the data -> data+inv(data).

32 " " => ignored

Add Pulse  : 65 "A" => a pulse is transmitted after the last bit.

32 " " => ignored

Repeat     : "0"-"?"=> the transmission is repeated (0-15) times if no key is pressed

(if the program is run as a batch job and not manually)

Sampl start : If included specifies from which sample to decode

Flags       : If included they can change the way transmit program works

Bit0:	If the interrupts should be turned off (1 = don't turn off)

Bit1:	If the interrupts should be turned on  (1 = don't turn on)

Bit2:	If the keboard should be scanned       (1 = don't scan)

Bit3:	If a space is to be at the end         (1 = make a space)

Bit4:

*******************************************************************************

Have fun.

If you successfully learns new remotes please mail me the new code so that

I can keep a database of remotes so that others don't have to sample new

remotes more than they have to.

_

Bjorn Gahm d89-bga@nada.kth.se

_

Appelstigen 2_

181 62 LIDINGO

SWEDEN