#

File: hitachi.doc,  Node: Top,  Prev: (grundig.doc),  Next: (jvc.doc),  Up: (remotes.info)

******************************************************************************

HITACHI

******************************************************************************

Version:		3.2

Date:			20 Nov 93

* Menu:

* Technical info::

* Video::

#

File: hitachi.doc,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Carrier: 		39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+                +..

Header:      |                                |                |

..+                                +----------------+

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+                +--+

Stop code:      |                                |                |  |

..+                                +----------------+  +..

16T                      8T        T

8.8ms                   4.4ms

T = 550us

Space between transmissions: 11000us

To use the codes add the command with (the address * #10000h)

For more information please read the documentation.

#

File: hitachi.doc,  Node: Video,  Prev: Technical info,  Up: Top

******************************************************************************

Video

******************************************************************************

Video1:

GROUP	#06F9

Video2:

GROUP	#0679

Code	Function

----	--------

#50	Rew

#28	Play

#90	FF

#D0	Stop

#58	Pause

#28	Rec

#E8	Operate

#A2	Index

#E2	Repeat

#0C	SP/LP

#08	Step

#80	Tracking -

#00	Tracking +

#C0	Slow -

#88	Slow

#40	Slow +

#62	Counter remain

#82	Counter reset

#22	Counter memory

#02	Osd menu

#42	Osd disp

#30	0

#B0	1

#70	2

#F0	3

#38	4

#B8	5

#78	6

#F8	7

#20	8

#A0	9

#18	Ch -

#98	Ch +