#

File: philips.doc,  Node: Top,  Prev: (panasoni.doc),  Next: (pioneer.doc),  Up: (remotes.info)

The RC5 code:

-------------

The RC5 code is an 13-bit code, it used to have a 5-bits group and a

6-bits command, and 2 stop-bits, but it was changed to 5-bits group,

7-bits command and 1 stop-bit.

Carrier: 36kHz

No header.

+----+

1 is coded:     |    |

----+    +

T    T

+----+

0 is coded: |    |

+    +----

T    T

T = 889us

Old RC5:

data length: 14bit (2sync bits, 1 control bit, 11 data bits) 24,889ms

Example to play video:

store 5 in GROUP

put 53 on stack

press RC5REMOTE

******************************************************************************

------------------------------------------------------------------------------

Commnd:	Video mode: TV1 & TV2	Video mode: TXT		Video mode: VD

Group 0 (TV1) 1 (TV2)	Group 2			Group 3

------------------------------------------------------------------------------

0	0			0			0

1	1			1			1

2	2			2			2

3	3			3			3

4	4			4			4

5	5			5			5

6	6			6			6

7	7			7			7

8	8			8			8

9	9			9			9

10	1/2/3 digs (japan:10)	step page +		=/= =/=

11	channel/program(jap:11)	step page -		*

12	standby			standby			standby

13	mute/demute		mute/demute		mute/demute

14	P.P. (personal pref.)	P.P.			P.P.

15	display			display			display

16	volume +		volume +		volume +

17	volume -		volume -		volume -

18	brightness +		brightness +		brightness +

19	brightness -		brightness -		brightness -

20	colour saturation +	colour saturation +	colour saturation +

21	colour saturation -	colour saturation -	colour saturation -

22	bass +			bass +			bass +

23	bass -			bass -			bass -

24	treble +		treble +		treble +

25	treble -		treble -		treble -

26	balance right		balance right		balance right

27	balance left		balance left		balance left

28	contrast +		enter/validation (Frnc)

29	contrast -		memory out

30	search +		sequence out		attribute on/off

31	tint/hue +

32	channel/program +	exchange

33	channel/program -	index/summary (France)	index

34	alt/channel		row zero (France)

35	1st/2nd language	1st/2nd language	1st/2nd language

36	spacial stereo on/off	spacial stereo on/off	spacial stereo on/off

37	mono/stereo (USA)

38	sleeptimer		hardcopy printer	hardcopy printer

39	tint/hue -		antiope TV mixed (Frnc)

40	RF switch/PIP select	PIP select		terminal initiation

41	vote			page hold on/off

42	timed page/channel clck	timed page (F:magazine)

43	increment (USA)		large top/lrge bot/norm	large top/lrge bot/norm

44	decrement (USA)		reveal/conceal/canc pic

45				cancel teletext picture

46	2nd controls (USA:menu)	TV/Text mixed/Subtitle	TV/Text mixed

47	PIP shift		PIP shift		view data line off

48	pause			pause			pause

49	erase			erase			erase

50	rewind			rewind			rewind

51	go to			go to			go to

52	wind			wind			wind

53	play			play			play

54	stop			stop			stop

55	recording		recording		recording

56	external 1		external 1		external 1

57	external 2		external 2		external 2

58	PIP display mode	PIP display mode

59	view data mode/advance				view data line on

60	teletext submode (J:12)

61	system standby		system standby		system standby

62	crispener on/off	news flash (France)

63	system select		system select		system select

64	surround sound		surround sound		surround sound

65	balance front		balance front		balance front

66	balance rear		balance rear		balance rear

67	sound effects		sound effects		sound effects

68	sound effects		sound effects		sound effects

69	sound effects		sound effects		sound effects

70	sound effects		sound effects		sound effects

71	sound effects		sound effects		sound effects

72	sound or menu func.	sound or menu func.	sound or menu func.

73	sound or menu func.	sound or menu func.	sound or menu func.

74	sound or menu func.	sound or menu func.	sound or menu func.

75

76

77	linear function up	linear function up	linear function up

78	linear function down	linear function down	linear function down

79	sound or menu func.	sound or menu func.	sound or menu func.

80	cursor step up		cursor step up		cursor step up

81	cursor step down	cursor step down	cursor step down

82	menu on			menu on			menu on

83	menu off		menu off		menu off

84	display A/V status	display A/V status	display A/V status

85	cursor step left	cursor step left	cursor step left

86	cursor step right	cursor step right	cursor step right

87	menu function		menu function		menu function

88	PIP on/off		PIP on/off		PIP on/off

89	PIP shift		PIP shift		PIP shift

90	PIP main swap		PIP main swap		PIP main swap

91	strobe on/off		strobe on/off		strobe on/off

92	multi strobe		multi strobe		multi strobe

93	main freezed		main freezed		main freezed

94	3/9 multi scan		3/9 multi scan		3/9 multi scan

95	PIP mode select		PIP mode select		PIP mode select

96	mosaic			mosaic			mosaic

97	solarization		solarization		solarization

98	main stored		main stored		main stored

99	PIP strobe		PIP strobe		PIP strobe

100	recall main		recall main		recall main

101	PIP freeze		PIP freeze		PIP freeze

102	PIP step up		PIP step up		PIP step up

103	PIP step down		PIP step down		PIP step down

104

105

106

107				red

108				green

109				yellow

110				cyan

111				index

112

113

114

115

116

117

118	sub mode		sub mode		sub mode

119	sub mode		sub mode		sub mode

120

121

122	cleaning up		cleaning up		cleaning up

123	connect			connect			connect

124	disconnect		disconnect		disconnect

125	cleaning up		cleaning up		cleaning up

126	cleaning up		cleaning up		cleaning up

127	cleaning up		cleaning up		cleaning up

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd:	Video mode: LV1		Video mode: VCR1 & VCR2	Video mode: Satellite

Group 4			Group 5	(VCR1) 6 (VCR2)	Group 8

------------------------------------------------------------------------------

0	0			0			0

1	1			1			1

2	2			2			2

3	3			3			3

4	4			4			4

5	5			5			5

6	6			6			6

7	7			7			7

8	8			8			8

9	9			9			9

10	picture number/time	1/2 digits/AM/PM	1/2 digits

11	channel number		channel/program mode	channel/program mode

12	standby			standby			standby

13	mute/demute		mute/demute		mute/demute

14	P.P. (personal pref.)	P.P.			P.P.

15	display			display			display

16	volume +		volume +		volume +

17	volume -		volume -		volume -

18	brightness +		brightness +		brightness +

19	brightness -		brightness -		brightness -

20	colour saturation +	colour saturation +	colour saturation +

21	colour saturation -	colour saturation -	colour saturation -

22	bass +			bass +			bass +

23	bass -			bass -			bass -

24	treble +		treble +		treble +

25	treble -		treble -		treble -

26	balance right		balance right		balance right

27	balance left		balance left		balance left

28	repeat once/TSPo	ITR (OTR) delay

29	repeat cont/TSPc	timer programming

30	next			record select

31	fast run reverse	fast run reverse

32	entry			step +			channel/program +

33	auto stop/memory	step -			channel/program -

34	slow run reverse	slow run reverse

35	audio 1			audio output select	language select/mode 1

36	audio 2			clock/channel		special stereo on/off

37	still picture		speed -			audio mixed / mode 2

38	speed +			speed +

39	speed -			Instant Time Recording

40	slow run forward	slow run forward

41	still forward		still forward

42	fast run forward	fast run forward

43	search automatic	index scan

44	scan reverse		scan reverse

45	open/close (eject)	eject

46	scan forward		scan forward		subtitle

47	play reverse		play reverse

48	pause			pause			pause

49	erase			erase			erase

50	rewind			rewind			rewind

51	go to			go to			go to

52	wind			wind			wind

53	play			play			play

54	stop			stop			stop

55	recording		recording		recording

56	external 1		external 1		external 1

57	external 2		external 2		external 2

58	clear all memory	short/long play

59	freeze segment		counter memory

60	TXT (pal)/ant.sw (ntsc)	scroll local display	TXT submode

61	system standby		system standby		system standby

62	CX on/off		RF switch

63	system select		system select		system select

64	surround sound		surround sound		surround sound

65	balance front		balance front		balance front

66	balance rear		balance rear		balance rear

67	sound effects		sound effects		sound effects

68	sound effects		sound effects		sound effects

69	sound effects		sound effects		sound effects

70	sound effects		sound effects		sound effects

71	sound effects		sound effects		sound effects

72	sound or menu func.	sound or menu func.	sound or menu func.

73	sound or menu func.	sound or menu func.	sound or menu func.

74	sound or menu func.	sound or menu func.	sound or menu func.

75				data stream start

76				data stream end

77	linear function up	linear function up	linear function up

78	linear function down	linear function down	linear function down

79	sound or menu func.	catalogue		sound or menu func.

80	cursor step up		cursor step up		cursor step up

81	cursor step down	cursor step down	cursor step down

82	menu on			menu on			menu on

83	menu off		menu off		menu off

84	display A/V status	display A/V status	display A/V status

85	cursor step left	cursor step left	cursor step left

86	cursor step right	cursor step right	cursor step right

87	menu function		acknowledge		menu function

88	PIP on/off		PIP on/off		PIP on/off

89	PIP shift		PIP shift		PIP shift

90	PIP main swap		PIP main swap		PIP main swap

91	strobe on/off		strobe on/off		strobe on/off

92	multi strobe		multi strobe		multi strobe

93	main freezed		main freezed		main freezed

94	3/9 multi scan		3/9 multi scan		3/9 multi scan

95	PIP mode select		PIP mode select		PIP mode select

96	mosaic			mosaic			mosaic

97	solarization		solarization		solarization

98	main stored		main stored		main stored

99	PIP strobe		PIP strobe		PIP strobe

100	recall main		recall main		recall main

101	PIP freeze		PIP freeze		PIP freeze

102	PIP step up		PIP step up		PIP step up

103	PIP step down		PIP step down		PIP step down

104				automatic start ID

105				write skip ID

106				skip on/off

107				write end ID

108				write FTS

109				write TOC

110

111

112				viss index next

113				viss index previous

114				mark

115				erase

116				renumber

117				blank search

118	sub mode		sub mode		TV1

119	sub mode		sub mode		sub mode

120				auto repeat

121				insert temp

122				scr loc disp

123	connect			connect			connect

124	disconnect

125

126

127

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd:	Audio mode: LV2/CDLV	Audio mode: Pre-Amp	Audio mode: Tuner

Group 12		Group 16 (#1) 19 (#2)	Group 17

------------------------------------------------------------------------------

0	0			0			0

1	1			1			1

2	2			2			2

3	3			3			3

4	4			4			4

5	5			5			5

6	6			6			6

7	7			7			7

8	8			8			8

9	9			9			9

10	picture number/time	graphic equalizer left	1/2 digits

11	chapter number		graphic equalizer right

12	standby			standby			standby

13	mute/demute		mute/demute		mute/demute

14	P.P. (personal pref.)	P.P.			P.P.

15	display			display			display

16	volume +		volume +		volume +

17	volume -		volume -		volume -

18	brightness +		brightness +		brightness +

19	brightness -		brightness -		brightness -

20	colour saturation +	colour saturation +	colour saturation +

21	colour saturation -	colour saturation -	colour saturation -

22	bass +			bass +			bass +

23	bass -			bass -			bass -

24	treble +		treble +		treble +

25	treble -		treble -		treble -

26	balance right		balance right		balance right

27	balance left		balance left		balance left

28	repeat once/TSPo	graph equal left&right

29	repeat cont/TSPc	loudness

30	next			high tone filter	search +

31	programming		low tone filter		search -

32				step +			preset +

33	auto stop/memory	step -			preset -

34	slow run reverse	signal path scroll

35	stereo left/right

36	previous

37	still reverse		surround mode		mono/stereo

38	speed +			sleeptimer

39	speed -			balance rear

40	slow run forward	balance front		RF switch

41	still forward		TPS			store execute

42	digital multi screen	timer set

43	strobe			timer up		scan forward

44	scan reverse		timer down

45	open/close (eject)	timer memo		FM

46	scan forward		acoustic memory		MW (USA: MW/FM)

47				acoustic memory select	LW

48	pause			pause			pause

49	erase			erase			erase

50	rewind			rewind			rewind

51	go to			go to			go to

52	wind			wind			wind

53	play			play			play

54	stop			stop			stop

55	recording		recording		recording

56	external 1		external 1		external 1

57	external 2		external 2		external 2

58	clear all memory	clear all memory	clear all memory

59	repeat A/B program

60	TXT (pal)/ant.sw (ntsc)	dyn range expand/compr

61	system standby		system standby		system standby

62	CX on/off

63	system select		system select		system select

64	surround sound		surround sound		surround sound

65	balance front		balance front		balance front

66	balance rear		balance rear		balance rear

67	sound effects		sound effects		sound effects

68	sound effects		sound effects		sound effects

69	sound effects		sound effects		sound effects

70	sound effects		sound effects		sound effects

71	sound effects		sound effects		sound effects

72	sound or menu func.	sound or menu func.	sound or menu func.

73	sound or menu func.	sound or menu func.	sound or menu func.

74	sound or menu func.	sound or menu func.	sound or menu func.

75

76

77	linear function up	linear function up	linear function up

78	linear function down	linear function down	linear function down

79	sound or menu func.	catalogue		sound or menu func.

80	cursor step up		cursor step up		cursor step up

81	cursor step down	cursor step down	cursor step down

82	menu on			menu on			menu on

83	menu off		menu off		menu off

84	display A/V status	display A/V status	display A/V status

85	cursor step left	cursor step left	cursor step left

86	cursor step right	cursor step right	cursor step right

87	menu function		menu function		menu function

88	PIP on/off		PIP on/off		PIP on/off

89	PIP shift		PIP shift		PIP shift

90	PIP main swap		PIP main swap		PIP main swap

91	strobe on/off		strobe on/off		strobe on/off

92	multi strobe		multi strobe		multi strobe

93	main freezed		main freezed		main freezed

94	3/9 multi scan		3/9 multi scan		3/9 multi scan

95	PIP mode select		PIP mode select		PIP mode select

96	mosaic			mosaic			mosaic

97	solarization		solarization		solarization

98	main stored		main stored		main stored

99	PIP strobe		PIP strobe		PIP strobe

100	recall main		recall main		recall main

101	PIP freeze		PIP freeze		PIP freeze

102	PIP step up		PIP step up		PIP step up

103	PIP step down		PIP step down		PIP step down

104

105

106

107

108	FTS

109

110

111

112	index next

113	index previous

114

115

116

117

118	sub mode

119	sub mode

120

121

122

123	connect			connect			connect

124	disconnect		disconnect		disconnect

125

126

127

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd:	Audio mode: CD		Audio mode: Phono	Audio mode: Recorder

Group 20		Group 21		Group 18 (#1) 23 (#2)

------------------------------------------------------------------------------

0	0			0			0

1	1			1			1

2	2			2			2

3	3			3			3

4	4			4			4

5	5			5			5

6	6			6			6

7	7			7			7

8	8			8			8

9	9			9			9

10	cursor scroll

11	display scroll

12	standby			standby			standby

13	mute/demute		mute/demute		mute/demute

14	P.P. (personal pref.)	P.P.			P.P.

15	display			display			display

16	volume +		volume +		volume +

17	volume -		volume -		volume -

18	brightness +		brightness +		brightness +

19	brightness -		brightness -		brightness -

20	colour saturation +	colour saturation +	colour saturation +

21	colour saturation -	colour saturation -	colour saturation -

22	bass +			bass +			bass +

23	bass -			bass -			bass -

24	treble +		treble +		treble +

25	treble -		treble -		treble -

26	balance right		balance right		balance right

27	balance left		balance left		balance left

28	rept once/TSPo/shuffle	repeat once		repeat once

29	repeat cont/TSPc	repeat continuously	repeat continuously

30	select +		select +		select +

31	select -		select -		select -

32	next			next			next

33	previous		previous		previous

34	index next		index next

35	index previous		index previous		last record cancel

36	play/program		stereo/mono		sequential

37	speed nominal					blank search

38	speed +

39	speed -

40							recording pause

41	store execute		store execute		store execute

42							recording blank

43	scan forward		scan forward		scan forward

44							mechanism A

45	open/close		open/close		eject

46				wave scroll		mechanism B

47							play reverse

48	pause			pause			pause

49	erase			erase			erase

50	rewind			rewind			rewind

51	go to			go to			go to

52	wind			wind			wind

53	play			play			play

54	stop			stop			stop

55	recording		recording		recording

56	external 1		external 1		external 1

57	external 2		external 2		external 2

58	clear all memory	clear all memory	clear all memory

59	repeat A/B program

60	dynamic range expand

61	system standby		system standby		system standby

62	dynamic range compress				counter memory

63	system select		system select		system select

64	surround sound		surround sound		surround sound

65	balance front		balance front		balance front

66	balance rear		balance rear		balance rear

67	sound effects		sound effects		sound effects

68	sound effects		sound effects		sound effects

69	sound effects		sound effects		sound effects

70	sound effects		sound effects		sound effects

71	sound effects		sound effects		sound effects

72	sound or menu func.	sound or menu func.	sound or menu func.

73	sound or menu func.	sound or menu func.	sound or menu func.

74	sound or menu func.	sound or menu func.	sound or menu func.

75

76

77	linear function up	linear function up	linear function up

78	linear function down	linear function down	linear function down

79	sound or menu func.	sound or menu func.	sound or menu func.

80	cursor step up		cursor step up		cursor step up

81	cursor step down	cursor step down	cursor step down

82	menu on			menu on			menu on

83	menu off		menu off		menu off

84	display A/V status	display A/V status	display A/V status

85	cursor step left	cursor step left	cursor step left

86	cursor step right	cursor step right	cursor step right

87	menu function		menu function		menu function

88	PIP on/off		PIP on/off		PIP on/off

89	PIP shift		PIP shift		PIP shift

90	PIP main swap		PIP main swap		PIP main swap

91	strobe on/off		strobe on/off		strobe on/off

92	multi strobe		multi strobe		multi strobe

93	main freezed		main freezed		main freezed

94	3/9 multi scan		3/9 multi scan		3/9 multi scan

95	PIP mode select		PIP mode select		PIP mode select

96	mosaic			mosaic			mosaic

97	solarization		solarization		solarization

98	main stored		main stored		main stored

99	PIP strobe		PIP strobe		PIP strobe

100	recall main		recall main		recall main

101	PIP freeze		PIP freeze		PIP freeze

102	PIP step up		PIP step up		PIP step up

103	PIP step down		PIP step down		PIP step down

104							automatic start ID

105							write skip ID

106							skip on/off

107							write end ID

108	FTS						write FTS

109							write TOC

110

111

112

113

114							write start ID

115							Ca/Er/SK ID

116							reformat

117							blank search

118	sub mode		sub mode		sub mode

119	sub mode		sub mode		sub mode

120

121

122

123	connect			connect			connect

124	disconnect		disconnect		disconnect

125

126

127

------------------------------------------------------------------------------

The codes and parts of the RC5REMOTE program are copied from

the article "Remote Control for HP28S" written by

Michiel Niemeijer & Eric Toonen.

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN