#

File: pioneer.doc,  Node: Top,  Prev: (philips.doc),  Next: (salora.doc),  Up: (remotes.info)

******************************************************************************

PIONEER

******************************************************************************

Version:	3.2

Date:		15 Nov 93

* Menu:

* Technical info::

* Amp::

* Tape::

* CD::

* Tuner::

* Eq::

* Timer::

* Additional info::

#

File: pioneer.doc,  Node: Technical info,  Prev: Top,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length 32 bits.

Carrier: 40kHz

+--------+      +..

Header:   |        |      |

..+        +------+

16T     8T

+--+    +--+

1 is coded:  |  |    |  |

..+  +----+  +..

T   3T  T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

T=500us approx

Example to play a pioneer CD:

\<<

PION				@ get info string

#45BAh #10000h * E817h +	@ get the data

TRANSMIT			@ transmit

\>>

#

File: pioneer.doc,  Node: Amp,  Prev: Technical info,  Next: Amp,  Up: Top

******************************************************************************

AMPLIFIER

******************************************************************************

GROUP:	#A55A

Code	Function

----	--------

#38C7	Power

#50AF	Vol +

#D02F	Vol -

#31CE	Dat/Video

#30CF	Phono

#59A6	CD direct

#7A85	Surround

#

File: pioneer.doc,  Node: Tape,  Prev: Amp,  Next: CD,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP:	#659A

Deck a	Deck b

Code	Code	Function

----	----	--------

#5AA5	#E817	Play

#9A65	#A857	Reverse Play

#DA25	#6897	Stop

#6A95	#8877	Rew

#EA15	#08F7	FF

#18E7	Pause

#28D7	Rec

#

File: pioneer.doc,  Node: CD,  Prev: Tape,  Next: Tuner,  Up: Top

******************************************************************************

CD

******************************************************************************

GROUP:	#45BA

Code	Function

----	--------

#E817	Play

#6897	Stop

#8877	Rew

#08F7	FF

#18E7	Pause

#B847	Disc

#

File: pioneer.doc,  Node: Tuner,  Prev: CD,  Next: Eq,  Up: Top

******************************************************************************

TUNER

******************************************************************************

GROUP:	#25DA

Code	Function

----	--------

#08F7	Station +

#8877	Station -

#

File: pioneer.doc,  Node: Eq,  Prev: Tuner,  Next: Timer,  Up: Top

******************************************************************************

EQ

******************************************************************************

GROUP:	#05FA

Code	Function

----	--------

#BC43	On/Off

#0CF3	Select

#

File: pioneer.doc,  Node: Timer,  Prev: Eq,  Next: Additional info,  Up: Top

******************************************************************************

TIMER

******************************************************************************

GROUP:	#659A

Code	Function

----	--------

#9867	Sleep

#738C	Wake up

#B34C	Snooze

#

File: pioneer.doc,  Node: Additinoal info,  Prev: Timer,  Up: Top

******************************************************************************

ADDITIONAL INFO

******************************************************************************

The pioneer receiver (amp/tuner) uses #A55A0000 and #25DA0000, but

not the amplifier group number you had listed in the mail you sent me.

PIONEER

0       2       8       A

0       (10)    (+10)   (ch>)   -

1       (8)     sleep   -       -

2       (4)     -       -       video select

3       -       cd      power   -

4       (2)     -       mute    -

5       vol>    balance on      -

6       (6)     (A-B)   -       (fr>)

7       vcr2(am) tape   (mono)  -

8       (1)     -       (<ch)   mute on

9       (9)     bal>    -       -

A       (5)     -       -       select

B       vdp(fm) phono   -       -

C       (3)     -       (am/fm) -

D       <vol    <bal    off     -

E       (7)     tuner   -       (<fr)

F       vcr1    -       -       -

where:  values in parentheses are for group #25DA0000

and other values are for group #A55A0000.

Tom Boldt

che043sa@sail.uwaterloo.ca

(519) 746-5163