#

File: salora.doc,  Node: Top,  Prev: (pioneer.doc),  Next: (sanyo.doc),  Up: (remotes.info)

Code length:   12bits

_

| |

Header:   | |__________

580us

_

| |

0:        | |____

200us

_

| |

1:        | |________

400us

Space:    25000us

Code is transmited once and then xored with #11110b and then

transmited as long as a key is held down.

Codes for Salora cable decoder

#A00  1

#A20  2

#A40  3

#A60  4

#A80  5

#AA0  6

#AC0  7

#AE0  8

#B00  9

#B20  0

#B40  1-

#B60  #

#B80  Ch-

#BA0  Ch+

#C60  Kan

#C40  MP

#CE0  M

#CC0  N

#8A0  fine tune -

#880  fine tune +

#840  power

#8C0  mute

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN