#

File: sharp.doc,  Node: Top,  Prev: (schneide.doc),  Next: (sony.doc),  Up: (remotes.info)

Remote control info:	SHARP

Version:		3.1.1

Date:			16 june 93

Code length:		17 bits

Carrier:		32kHz

Transmits code + code XOR #7FE0h

header:	None

one:	275us pulse + 1900us space

zero: 	275us pulse + 775us space

Space between transmissions:	43ms

*********************************

Sharp

---------------------------------

Device:	TV

GROUP:	Not needed

Code	Function

----	--------

#2141	0

#2021	1

#2041	2

#2061	3

#2081	4

#20A1	5

#20C1	6

#20E1	7

#2101	8

#2121	9

#2221	Ch+

#2241	Ch-

#22C1	Power

#25E1	10+

#2601	20+

#2621	30+

#2261	TV/video

#22E1	Mute

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN