#

File: technics.doc,  Node: Top,  Prev: (teac.doc),  Next: (yamaha.doc),  Up: (remotes.info)

Codes for Technics.

The code is 48 bit long.

Carrier:  32kHz

+--------------------+

Header:      |                    |

..+                    +--------

10T            4T

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +....

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +....

T   T  T

Space between transmissions 190T.

T = 400us approx

To use the codes add the command with (the address * #10000h)

*********************************

TECHNICS

---------------------------------

Amp:

GROUP	#40040500

Code      Function

----      --------

#0401     Vol+

#8481     Vol-

#4C49     Muting

#4346     Super Bass

#5154     Aux Digital

#191C     Dat

#797C     VCR

#4540     VDP

---------------------------------

Amp:

GROUP	#40040520

Code      Function

----      --------

#BC99     on/off

#694C     Sleep

#0326     CD

#83A6     Tape

#4366     Tuner

---------------------------------

CD:

GROUP	#40040550

Code      Function

----      --------

#5005     Play

#6035     Pause

#0055     Stop

#5207     ff

#92C7     Rew

#5401     Program

#C590     Cancel

#80D5     Open/Close

#AAFF     Time mode

#085D     1

#88DD     2

#481D     3

#C89D     4

#287D     5

#A8FD     6

#683D     7

#E8BD     8

#184D     9

#98CD     0

#2174     >10

---------------------------------

Tape:

GROUP	#40040510

Code      Function

----      --------

#A9BC     Deck 1/2

#D0C5     Play

#5045     Rew Play

#0015     Stop

#C0D5     Rew

#4055     FF

#6075     Pause

#1005     Rec

#4145     Rec mute

---------------------------------

Tuner:

GROUP	#40040520

Code      Function

----      --------

#082D     1

#88AD     2

#486D     3

#C8ED     4

#280D     5

#A88D     6

#684D     7

#E8CD     8

#183D     9

#98BD     0

---------------------------------

Phono:

GROUP	#40040570

Code      Function

----      --------

#5025     Play

#0075     Stop

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN