#

File: jvc.info,  Node: Top,  Prev: (hitachi.info),  Next: (kenwood.info),  Up: (remotes.info)

******************************************************************************

JVC

******************************************************************************

* Menu:

* Technical info::

* Video::

* Amplifier::

* Tuner::

* CD::

* TV::

* DAT::

* Micro Component System::

#

File: jvc.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:    16 bits

Carrier:        approx 35.5kHz

+--------+

Header:  |        |

+        +----------------+..

4T          8T

The header is only transmited once (for the first repetition of the code).

+--+

1 is coded:  |  |

..+  +------+..

500us 1800us

+--+

0 is coded:  |  |

..+  +--+..

500 600us

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

500us

#

File: jvc.info,  Node: Video,  Prev: Technical info, Next: Amplifier, Up: Top

******************************************************************************

Video

******************************************************************************

Group:  Not needed

xx43 is Video A

xx53 is Video B

Code	Function

----	--------

#0343	stop

#0443	eject

#0543	on

#0643	ff

#0743	rew

#0A43	off

#0B43	operate

#0C43	play

#0D43	pause

#1443	slow+

#1543	slow-

#1743	audio monitor

#1843	ch-

#1943	ch+

#2143	1

#2243	2

#2343	3

#2443	4

#2543	5

#2643	6

#2743	7

#2843	8

#2943	9

#3143	sp/lp

#3343	0

#3243	Search (Tune)

#3443	ch

#3643	cancel/skip

#3743	program

#3843	display

#3943	counter reset

#3A43	Clock set

#3B43	go-to

#3C43	Fine tune +

#3E43	Store

#9143	index

#9343	mark

#9443	erase

#9643	skip/blank

#CC43	rec

#F443	vps

#From: Howard Chu <howard@harry.lloyd.com>

#0843	slow+

#1343	TV/Video

#3543	Timer

#4B43	TV Monitor

#8E43	Sound Menu

#AD43	Picture Menu

#CD43	Index+

#CF43	Index-

You must use a different info string to use these codes; take the regular

JVC code and turn off the header part.

#AF43	Jog-

#ED43	Jog+

#

Codes for the JXS-300 A/V switcher:

#BFF3	Power

#A1F3	Input 1

#A2F3	Input 2

#A3F3	Input 3

#A4F3	Input 4

File: jvc.info,  Node: Amplifier,  Prev: Video, Next: Tuner, Up: Top

******************************************************************************

Amplifier

******************************************************************************

Group:  Not needed

Code	Function

----	--------

#1FA3	vol-

#1EA3	vol+

#1CA3	muting

#0BA3	tuner

#3CA3	phono

#3EA3	video/aux

#0C83	tape

#0383	tape stop

#

File: jvc.info,  Node: Tuner,  Prev: Amplifier, Next: CD, Up: Top

******************************************************************************

Tuner

******************************************************************************

Group:	Not needed

Code	Function

----	--------

#18A3   pre-

#19A3	pre+

#21A3	1

#22A3	2

#23A3	3

#24A3	4

#25A3	5

#26A3	6

#27A3	7

#28A3	8

#29A3	9

#20A3	0

#2AA3	10

#2FA3	+10

#

File: jvc.info,  Node: CD,  Prev: Tuner, Next: TV, Up: Top

******************************************************************************

CD

******************************************************************************

From: kwmccoy@mentor.cc.purdue.edu (Ken)

Group:  Not needed

Code	Function

----	--------

#43B3	stop

#44B3	open/close

#46B3	search f

#47B3	search r

#48B3	intro

#49B3	editing

#4CB3	play

#4DB3	pause

#54B3	display

#55B3	?

#56B3	index

#57B3	?

#58B3	skip <-

#59B3	skip ->

#5BB3	repeat all

#5CB3	A<->B

#5DB3	?

#60B3	0

#5EB3	?

#5FB3	?

#61B3	1

#62B3	2

#63B3	3

#64B3	4

#65B3	5

#66B3	6

#67B3	7

#68B3	8

#69B3	9

#6AB3	program

#6EB3	ten

#6FB3	+10

#70B3	search r

#72B3	random

#76B3	search f

#6AB3	10

#3DA3	play

#43B3	stop

#4DB3	pause

#44B3	open/close

#58B3	skip rew

#59B3	skip ff

#70B3	rew

#76B3	ff

#

File: jvc.info,  Node: TV,  Prev: CD, Next: DAT, Up: Top

******************************************************************************

TV

******************************************************************************

Group:  Not needed

Code    Function

----    --------

#0403   display

#0503   return

#0603   function

#0A03   set choice

#0B03   ANT/Cable

#0D03   AV status

#1003   channel

#1103   channel

#1203   channel

#1303   channel

#1403   channel

#1703   on/off

#1803   ch -

#1903   ch +

#1A03   off

#1C03   mute

#1E03   vol +

#1F03   vol -

#2003   zero

#2103   one

#2203   two

#2303   three

#2403   four

#2503   five

#2603   six

#2703   seven

#2803   eight

#2903   nine

#3C03   recall choice

#5A03   fun +

#5B03   fun -

#7703   ??

#7A03   menu

#7B03   fun for

#7C03   fun back

#

File: jvc.info,  Node: DAT,  Prev: TV, Next: Micro Component System, Up: Top

******************************************************************************

DAT

******************************************************************************

Device: JVC dat (name: XD-Z505)

GROUP:  Not needed

These codes were sampled by Frank Heepmann

Code	Function

-----	--------

#2193	1

#2293	2

#2393	3

#2493	4

#2593	5

#2693	6

#2793	7

#2893	8

#2993	9

#2E93	10

#2F93	+10

#0C93	play

#0393	stop & open/close

#0D93	pause

#CC93	rec (at once)

#CD93	rec (pause)

#0293	mute & sampling monitor

#1893	prgm <<

#1993	prgm >>

#0793	search <<

#0693	search >>

#3093	start id write

#1493	display mode

#2D93	reset counte

#1B93	repeat 1/all

#0A93	digital peak

#

File: jvc.info,  Node: Micro Component System,  Prev: DAT, Up: Top

******************************************************************************

Micro Component System

******************************************************************************

From: jsproat@uvsc.edu (JEREMY H SPROAT)

Here's my system:

HP48SX ROM Version E

Remote v3.2 by Bjorn Gahm

JVC UX-T1 C/J Micro Component System

Code	Glyph	Function

-----   	-----	--------

#009F	0/1	power

#7B9F	DOOR	CD door open/close

#0B9F	RADIO	radio AM/FM

#029F	V-	volume down

#019F	V+	volume up

#889F	B-	bass level down

#879F	B+	bass level up

#8A9F	T-	treble level down

#899F	T+	treble level up

#3E9F	|<|<|	CD skip backward/radio scan backward

#3D9F	|>|>|	CD skip forward/radio scan forward

#709F	|> ||	CD play/pause

#719F	[]/CL	CD clear program/stop

#779F	RAND	CD random play on/off

#789F	REPEA	CD repeat track/repeat disc

#749F	PROG	CD program tracks

#4B9F	() ||	tape record/pause recording

#4D9F	<|	tape reverse play

#419F	[]	tape stop

#4C9F	|>	tape forward play

#459F	<|<|	tape rewind/scan reverse

#449F	|>|>	tape fast forward/scan forward

#219F	1	CD track 1/radio preset 1

...	...	...

#2?9F	?	CD track ?/radio preset ?

...	...	...

#299F	9	CD track 9/radio preset 9

#2A9F	10	CD track 10/radio preset 10

#2E9F	+10	CD track +10/radio preset +10