Archive-name: AudioFAQ/pro-audio-faq

Last-modified: 1998/12/12

Version: 0.9.34

Frequently Asked Questions (FAQ) file for rec.audio.pro

Version 0.9.34

Edited and compiled by Gabe Wiener

David Josephson (david@josephson.com)

Mark Plancke (mark@soundtechrecording.com)

The most current version of this document can be found at

http://recordist.com/rap-faq/current

Many thanks to all who have contributed to the FAQ.  Individual

contributions are credited at the end of their respective sections.

The core FAQ writers are listed below.  The rubric in brackets will be

used to indicate who has written a particular section.

Brian Allen (buzallen@teleport.com) [Brian]

Scott Dorsey (kludge@netcom.com) [Scott]

Harvey Gerst (harvey@itrstudio.com) [Harvey]

Christopher Hicks (cmh@eng.cam.ac.uk) [Chris]

David Josephson (david@josephson.com) [David]

Marco Olivotto (sonica@tqs.it) [Marco]

Dick Pierce (dpierce@world.std.com) [Dick]

Mark Plancke (mark@soundtechrecording.com) [Mark]

David Rick (drick@@hach.com) [David R]

Joe Schottman (jschottm@E115074.vtacs.com) [Joe]

Gabe Wiener [Gabe]

PLEASE NOTE

Each author maintains and asserts full legal copyright on his

contribution to the FAQ.  Compilation copyright (C) 1996 by Gabe M.

Wiener.  All rights reserved.  Permission is granted for

non-commercial electronic distribution of this document. Distribution

in any other form, or distribution as part of any commercial product

requires permission. Anyone is free to link to current copies of this

FAQ on their web sites, but please use http://www.recordist.com/rap-faq

as this will often have a more recent rev than will rtfm.mit.edu.

Please do not use copies of this FAQ at your site; we want the most

recent revision to be available to anyone who asks for it.

Inquire to (david@josephson.com).

---------

TABLE OF CONTENTS FOR THE FAQ:

** - Denotes new additions or significant revisions since last

version. [MM/DD] Denotes revision date [Mark]

Section I - Netiquette

Q1.1 - What is this newsgroup for?  What topics are appropriate here

and what topics are best saved for another newsgroup?

Q1.2 - Do I have to be a "professional" to post here?

Q1.3 - I've got a question, what is the first thing I should do?  [9/98]

Q1.4 - I need to ask the group for help with selecting a piece of

equipment. What information should I provide in my message?

Q1.5 - I want to sell something.  Is this the place to do it? [9/98]

Q1.6 - What is some general Netiquette to follow? [9/98]

Q1.7 - Hey, I posted something here, and now I'm getting a bunch of

junk mail!  What gives? [9/98]

Q1.8 - Hey, I posted a simple question, and some jerk gave me a rude

answer! What gives? [9/98]

Section II - The business of audio

Q2.1 - How does one get started as a professional audio engineer?

Q2.2 - Are audio schools worth the money?  Which schools are best?

Q2.3 - What are typical rates for various professional audio services?

Section III - Audio Interconnections

Q3.1 - How are professional transmission lines and levels different

from consumer lines and levels?  What is -10 and +4?  What's a

balanced or differential line?

Q3.2 - What is meant by "impedance matching"?  How is it done?  Why is

it necessary?

Q3.3 - What is the difference between dBv, dBu, dBV, dBm, dB SPL, and

plain old dB?  Why not just use regular voltage and power measurements?

Q3.4 - Which is it for XLRs?  Pin 2 hot?  Or pin 3 hot? **[12/98]

Q3.5 - What is phantom power?  What is T-power? Can I damage my

dynamic or ribbon microphone by plugging it in to phantom power? **[12/98]

Q3.6 - How do I interconnect balanced and unbalanced components?

Q3.7 - What are ground loops and how do I avoid them?

Q3.8 - What is the "Pin 1 problem" and how do I avoid it?

Section IV - Analog tape recording

Q4.1 - What does it mean to "align" a tape machine? **[12/98]

Q4.2 - What is bias?  What is overbias? **[12/98]

Q4.3 - What is the difference between Dolby A, B, C, S, and SR?  How

does each of these systems work? What is the effect on recorded

sound of these systems? **[12/98]

Q4.4 - What is Dolby HX-Pro?

Q4.5 - How does DBX compare to Dolby?

Q4.6 - How much better are external microphone preamplifiers than

those found in my portable recorder?

Q4.7 - What is an MRL?  Where do I get one?

Section V - Digital recording and interconnection

Q5.1 - What is sampling?  What is a sampling rate?

Q5.2 - What is oversampling?

Q5.3 - What is the difference between a "1 bit" and a "multibit"

converter? What is MASH?  What is Delta/Sigma?  Should I really care?

Q5.4 - On an analog recorder, I was always taught to make sure the

signal averages around 0 VU.  But on my new DAT machine, 0 is all

the way at the top of the scale.  What's going on here?

Q5.5 - Why doesn't MiniDisc or Digital Compact Cassette sound as good

as DAT or CD?  After all, they're both digital.

Q5.6 - What is S/P-DIF?  What is AES/EBU?

Q5.7 - What is clock jitter?

Q5.8 - How long can I run AES/EBU or S/P-DIF cables?  What kind of

cable should I use?

Q5.9 - What is SCMS?  How do I defeat it?

Q5.10 - What is PCM-F1 format?

Q5.11 - How do digital recorders handle selective synchronization?

Q5.12 - How can a 44.1 kHz sampling rate be enough?

Q5.13 - Doesn't the 44.1 kHz sampling rate make it impossible to

reproduce square waves?

Q5.14 - How can a 16-bit word-length be enough?

Q5.15 - What's all this about 20- and 24-bit digital audio?  Aren't

CDs limited to 16 bits?

Section VI - Digital editing and mastering

Q6.1 - What is a digital audio workstation?

Q6.2 - How is digital editing different from analog editing?

Q6.3 - What is mastering?

Q6.4 - What is normalizing?

Q4.5 - I have a fully edited DAT that sounds just like I want it to

sound on the CD.  Is it okay to send it to the factory?

Q6.6 - What is PCM-1630?  What is PMCD?

Q6.7 - When preparing a tape for CD, how hot should the levels be?

Q6.8 - What is CD-R? Why do I care about disk-at-once or

track-at-once?

Q6.9 - Where can I get CDs manufactured?

Q6.10 - How are CD error rates measured, and what do they mean?

Section VII - Market survey.  What are my options if I want --

Q7.1 - A portable DAT machine or rack size DAT machine [9/98]

Q7.2 - A good but inexpensive compressor [9/98]

Q7.3 - An inexpensive stereo microphone [9/98]

Q7.4 - An inexpensive pair of microphones for stereo [9/98]

Q7.5 - A good microphone for recording vocals [9/98]

Q7.6 - A good microphone for recording [insert instrument here]

[9/98]

Q7.7 - A small mixer [9/98]

Q7.8 - A portable cassette machine [9/98]

Q7.9 - A computer sound card for my IBM PC or Mac [9/98]

Q7.10 - An eight-track digital recorder?

Section VIII - Sound reinforcement

Q8.1 - We have a fine church choir, but the congregation can't hear

them. How do we mic the choir?

Q8.2 - How do I 'ring out' a system? [9/98]

Q8.3 - How much power to I need for [insert venue here]?

Q8.4 - How good is the Sabine feedback eliminator?

Section IX - Sound restoration

Q9.1 - How can I play old 78s?

Q9.2 - How can I play Edison cylinders?

Q9.3 - What are "Hill and Dale" recordings, and how do I play them

back?

Q9.4 - What exactly are NoNOISE and CEDAR?  How are they used?

Q9.5 - How do noise suppression systems like NoNOISE and CEDAR work?

Q9.6 - What is forensic audio?

Section X - Recording technique, Speakers, Acoustics, Sound

Q10.1 - What are the various stereo microphone techniques? [9/98]

Q10.2 - How do I know which technique to use in a given circumstance?

[9/98]

Q10.3 - How do I soundproof a room?

Q10.4 - What is a near-field monitor?

Q10.5 - What are the differences between "studio monitors" and home

loudspeakers?

Section XI - Industry information

Q11.1 - Is there a directory of industry resources?

Q11.2 - What are the industry periodicals?

Q11.3 - What are the industry trade organizations?

Q11.4 - Are there any conventions or trade shows that deal specifically

with professional audio?

Section XII - Miscellaneous

Q12.1 - How do I modify Radio Shack PZMs?

Q12.2 - Can I produce good demos at home? [9/98]

Q12.3 - How do I remove vocals from a song?

Section XIII - Bibliography

Q13.1 - Fundamentals of Audio Technology

Q13.2 - Studio recording techniques

Q13.3 - Live recording techniques

Q13.4 - Digital audio theory and practice

Q13.5 - Acoustics

Q13.6 - Practical recording guides

Section XIV

Q14.1 - Who wrote the FAQ [9/98]

Q14.2 - How do you spell and pronounce the FAQ founder's surname?

--------

THE FAQ

Section I - Netiquette

--

Q1.1 - What is this newsgroup for?  What topics are appropriate here,

and what topics are best saved for another newsgroup? [9/98]

This newsgroup exists for the discussion of issues and topics

related to professional audio engineering.  We generally do not

discuss issues relating to home audio reproduction, though they

do occasionally come up. The rec.audio.* hierarchy of newsgroups

is as follows:

rec.audio.pro Issues pertaining to professional audio

rec.audio.marketplace Buying and trading of consumer equipment

rec.audio.tech Technical discussions about consumer audio

rec.audio.opinion Everyone's $0.02 on consumer audio

rec.audio.high-end High-end consumer audio discussions

rec.audio.misc Everything else

alt.music.4-track Deals with smaller home based recording

rec.music.makers.marketplace A music related for sale forum,

with some high end gear

Please be sure to select the right newsgroup before posting.

--

Q1.2 - Do I have to be a "professional" to post here?

No.  Anyone is welcome to post on rec.audio.pro so long as the

messages you post are endemic to the group in some way. If you

are not an audio professional, we would ask that you read this

FAQ in full before posting. You may find that some of your

essential questions about our field are answered right here.  But

if not, feel free to ask us.

--

Q1.3 - I've got a question, what is the first thing I should do?

[9/98]

Start with this FAQ!

In the case of many questions, they have already been asked, and

answered. In some case they have been beaten into the ground.  A

good starting point before asking a question is to go to

<http://www.dejanews.com> and check the archives there.  To do

so, click on power search.  Enter rec.audio.pro under the forum.

Then enter a few keywords about your question, and do a search.

[Joe]

--

Q1.4 - I need to ask the group for help with selecting a piece of

equipment. What information should I provide in my message?

If you are going to post a request for advice on buying

equipment, please provide the following information.

Your application for the equipment

What other equipment you will be using it with

Your budget for the equipment

Any specific requirements the equipment should have

There is nothing worse than messages like "Can anyone recommend a

DAT machine for me to buy?"  Sure we can.  But what do you want

to _do_ with it?  We can recommend DAT machines for $400 or for

$14,000.

--

Q1.5 - I want to sell something.  Is this the place to do it?  [9/98]

According to the rec.audio.pro charter selling is allowed.  The

unofficial mood of the group is that individuals wishing to sell

studio related gear are welcome, with moderation.  There have

been negative feelings expressed towards dealers posting their

wares. [Joe]

The generally accepted way for dealers to post for sale items is

to provide a pointer or link to their web page where the detailed

information is posted. This also applies to private parties who

have many items for sale. [Mark]

--

Q1.6 - What is some general Netiquette to follow? [9/98]

Trim what you quote.  If you want to add to something someone has

posted, remove most of the redundancy - if someone is reading the

thread they have most likely read the message you are adding to,

and don't need to see it again.  And those excess lines can be

painful to readers who have to pay by the minute for their

connection, or who have slow connections.

Don't post binaries to the newsgroup.  Many people won't be

interested, but it will slow down their connection, and irritate

them.  Also, if too many binaries are posted, this may cause some

sysadmins to remove rec.audio.pro from the available list of

newsgroups for their users.

Turn off HTML coding within your newsreader.  If you are using

Netscape or Internet explorer, you may be posting in HTML code.

To turn it off in Netscape 4, go to the edit menu, to

preferences, to Mail & Groups, to Messages, and turn off "By

default, send rich text (HTML) messages.  I expect Internet

Explorer is similar.

If you are selling something, post one ad, and let that be it.

Most news servers will keep your message for at least seven days.

Posting your ad day after day is more likely to turn off

potential buyers than to find them.  If you are going to

advertise your gear on one than one newsgroup, it is recommended

that you do so as a cross-post.  Many newsreaders will filter

the message as read after it is read on any newsgroup, so that

when the user reads another group he/she will not see your

message again and again.

Don't engage in flame wars or respond to trolls.  30 people going

back and forth between "ADAT's suck!" and "ADAT's rule!" doesn't

add anything to the group.  If you wish to discuss something, add

facts to back your opinion. And if someone posts an obviously

inflammatory message, don't bother responding, no matter how

angry or clever you might be.  That's exactly what they want.  If

you ignore them, such people go away.

It is not recommended that you post something with "please

respond via e-mail" or the like.  You are asking a large group of

people to take time out of their busy lives to discuss something

with you.  The least you can do is take the time out of yours to

check back on the newsgroup.  Beyond that, other people might

want to see the reply as well.  If you have a real reason why you

can't check the newsgroup, include that in your message to avoid

irritating people. [Joe]

--

Q1.8 - Hey, I posted something here, and now I'm getting a bunch of

junk mail!  What gives? [New 9/98]

Unfortunately, junk e-mailers (spammers) have discovered that

they can find people's e-mail addresses over Usenet, and hit them

with junk mail. The passive approach is to disguise your e-mail.

There are various ways to do this, check with your newsreader

documentation.  Be aware that this may keep you from getting the

help you want - most people won't spend hours figuring out your

disguised e-mail if it bounces back.  The active way is

attempting to get your authorities to make junk e-mail illegal. A

good starting point is <http://www.cause.com> and

<http://www.sputum.com>. [Joe]

--

Q1.9 - Hey, I posted a simple question, and some jerk gave me a rude

answer! What gives? [New 9/98]

Some of us do what we do because we don't play well with others.

After all, we spend our time sitting in a room all the time with

no natural light. And to quote Harvey Gerst "Which would you

choose; someone with a 50/50 smattering of knowledge and very

polite, or 97% accuracy and blunt? [Joe]

=====

Section II - The business of audio

--

Q2.1 - How does one get started as a professional audio engineer?

There are as many getting-started stories as there are audio

engineers.  The routes into the industry are highly dependent on

what aspect of the industry one wishes to enter.  For instance,

many engineers who work in the classical-music field have at one

time or another been classical performers.  Others enter through

their work in other musical genres, or through engineering

programs at universities or technical schools.  Without

exception, everyone in the industry has learned at least a

portion of their craft from watching those with more hands-on

experience.  Whether this comes from a formal internship or just

from sustained observation and long-term question-asking, it is

almost always universally true. [Gabe]

--

Q2.2 - Are audio schools worth the money?  Which schools are best?

An audio school will teach you the basics of the audio business,

but just like any technical school, what they teach you may not

be worth what you pay.

There are several schools of thought:

1. Audio schools are great, you get trained on the gear that is

used by top studios and costs millions of dollars, you get

taught by pros in the field and you have job placement

assistance after you graduate.

2. Going to an audio school is like wanting to learn aviation,

and when you start flight school they teach you a 747.  In the

real world, you are probably not going to have 96 channel

automated consoles on your first job.  You are not going to

mix your first live gig on a 48-channel 100,000 watt stadium

PA rig.  Better to start off on real world equipment and work

your way up to the top-of-the-line stuff.  Most recording

studios are 24-track analog or less and most PA systems are 16

channel, 3,000 watts or less. Don't buy education for

something you will never get to use after you leave the school.

3. Audio Schools are a waste of money.  Instead of spending

$18,000 for a course and having nothing to show for it but a

technical certificate (which everyone knows is no help at all

getting a job). You would be better off spending the 18 grand

on books and gear and learning by trial and error, or saving

the 18 grand altogether and learning first from reading, and

later from apprenticing. [jsaurman@cftnet.com (Jim Saurman)]

Jim summarizes the opinions pretty well.  Recognize that an

altogether different option is to attend a full four-year college

program.  Many colleges and universities offer such programs.

Examples include Peabody Conservatory, Cleveland Institute of

Music, McGill University, New York University, University of

Miami at Coral Gables, and the University of Massachusetts at

Lowell.  Without fail, graduates from these sorts of programs

earn far more respect than graduates of any technical school.

[Gabe]

--

Q2.3 - What are typical rates for various professional audio services?

Depends on what you want to have done, and where.

One can pay upwards of $300/hr for prime studio rental time in

New York. In a small community however, one might find a project

studio for $25/hr. Generally speaking, the rule is: the rarer the

service, the more it will cost.  In a community with dozens of

small 8-track studios, you won't have to pay much.  If you need

emergency audio restoration, or mastering by a top-flight pop-

music engineer, you can expect to drop many hundreds of dollars

an hour.  Like so many other things in this industry, there are

no rules, and Smith's invisible hand guides the market. [Gabe]

=====

Section III - Audio Interconnections

--

Q3.1 - How are professional transmission lines and levels different

from consumer lines and levels?  What is -10 and +4?  What's a

balanced or differential line?

Professional transmission lines differ from consumer lines in two

ways.  First, consumer lines tend to run about 14 dB lower in

level than pro lines.  Second, professional lines run in

differential, or balanced, configuration.

In a single-ended line, the signal travels down one conductor and

returns along a shield.  This is the simplest form of audio

transmission, since it is essentially the same AC circuit you

learned about in high-school physics.  The problem here is that

any noise or interference that creeps into the line will simply

get added to the signal and you'll be stuck with it.

In a differential line, there are three conductors.  A shield, a

normal "hot" lead, and a third lead called the "cold" or

"inverting" lead, which carries a 180-degree inverted copy of the

hot lead.  Any interference that creeps into the cable thus

affects both the hot and cold leads equally.  At the receiving

end, the hot and cold leads are summed using a differential

amplifier, and any interference that has entered the circuit

(called "common-mode information" since it is common to both the

hot and cold leads), gets canceled out. Differential lines are

thus better suited for long runs, or for situations where noise

or interference may be a factor.  [Gabe]

--

Q3.2 - What is meant by "impedance matching"?  How is it done?  Why is

it necessary?

We can talk about the characteristic impedance of an input, which

is to say the ratio of voltage to current that it likes to see,

or how much it loads down a source.  (You can think of this as

being an "AC resistance" and you would be mostly right, although

it's actually the absolute magnitude of the vector drawn by the

resistive and reactive load components.  Dealing with line level

signals, reactive components are going to be negligible, though).

In general, in this modern world, most equipment has a low

impedance output, going into relatively high impedance input.

This wastes some amount of power, but because electricity is

cheap and it's possible to build low-Z outputs easily today, this

is not a big deal.

With microphones, it _is_ a big deal, because the signal levels

are very low, and the drive ability poor.  As a result, we try

and get the best efficiency possible from microphones to get the

lowest noise floor.  This is often done by using transformers to

step up the voltage or step it down, to go into a higher or lower

Z load.  Transformers have some major disadvantages in that they

can be significant sources of non-linearity, but back in the days

of tubes they were the only solution.  Tubes have a very high-Z

input, and building balanced inputs with tubes requires three

devices instead of one.  As a result, all mike pre-amps would

have a 600 ohm balanced input, with a transformer, driving a pre-

amp tube.  Today, transistor circuits can be used for impedance

matching, although they are often more costly and can be noisier

in cases.

As a result of the expense, consumer equipment was built with

high-Z microphone inputs, and high-Z microphones.  This resulted

in more noise pickup problems, but was cheaper to make.

Unfortunately this still held on into the modern day of the

transistor, and a lot of high-Z consumer gear exists.  Guitar

pickups are generally high-Z devices, and require a direct box to

reduce the impedance so that they can go into a standard 600 ohm

mike pre-amp directly.

Many years ago, the techniques that were used in audio came

originally from telephone company practice.  Phone systems

operate with 150 or 600 ohm balanced lines, and adoption of this

practice into the audio industry caused those standards to be

used.  In the modern age where lines are relatively short and

transformers considered problematic, the tendency has been to

have low-Z outputs for all line level devices, driving high-Z

inputs.  While this is not the most efficient system, it is

relatively foolproof, and appears on most consumer equipment.  A

substantial amount of professional gear, however, still uses

internal balancing transformers or resistor networks to match to

a perfect 600 ohm impedance.   [Scott]

[Ed. note: Modern equipment works on principles of voltage

transfer rather than power transfer.  Thus a standard audio

circuit today is essentially a glorified voltage divider.  You

have a very low output impedance and a very high input impedance

such that the most voltage is dropped across the load.  This is

not an impedance-matched circuit in the classic sense of the

word.  Rather, it is a "bridged" or "constant voltage" impedance

match, and is the paradigm on which nearly all audio circuits

operate nowadays. -Gabe]

--

Q3.3 - What is the difference between dBv, dBu, dBV, dBm, dB SPL, and

plain old dB?  Why not just use regular voltage and power

measurements?

Our ears respond logarithmically to increases in sound pressure

level. In order to simplify the calculations of these levels, as

well as the electrical equivalents of them in audio systems, the

industry uses a logarithmic system to denote the values.

Specifically, the decibel is used to denote logarithmic level

above a given reference.  For instance, when measuring sound

pressure level, the basic reference against which we take

measurements is the threshold of hearing for the average

individual, 10^-12 W/m^2.  The formula for dB SPL then becomes:

10 Log X / 10^-12  where X is the intensity in watts per square

meter

The first people who were concerned about transmitting audio over

wires were, of course, the telephone company.  Thanks to Ma Bell

we have a bunch of other decibel measurements.  We can use the

decibel to measure electrical power as well.  In this case, the

formula is referenced to 1 milliwatt in the denominator, and the

unit is dBm.  1 milliwatt was chosen as the canonical reference

by Ma Bell.  Since P=V^2 / R, we can also express not only power

gain in dB but also voltage gain.  In this case the equation

changes a bit, since we have the ^2 exponent.  When we take the

logarithm, the exponent comes around into the coefficient, making

our voltage formula 20 log. In the voltage scenario, the

reference value becomes 0.775 V (the voltage drop across 600 ohms

that results in 1 mW of power).  The voltage measurement unit is

dBv.

The Europeans, not having any need to abide by Ma Bell's choice

for a canonical value, chose 1V as their reference, and this is

reflected as dBV instead of dBv.  To avoid confusion, the

Europeans write the American dBv as dBu.  Confused yet?  [Gabe]

--

Q3.4 - Which is it for XLRs?  Pin 2 hot?  Or pin 3 hot?

Depends on whom you ask!  Over the years, different manufacturers

have adopted varying standards of pin 2 hot and pin 3 hot (and

once in a while, pin *1* hot!).  But nowadays most manufacturers

have adopted pin 2 hot.  Still, it is worth taking the extra

minute or two to check the manual.  The AES standard is

pin 2 hot.  [Gabe]

The German spec for polarity of *any* audio connector, which became the

IEC spec, can be easily summarized as "the first pin after the ground

pin, is hot." So, if pin n is ground, pin n+1 is hot. [David]

--

Q3.5 - What is phantom power?  What is T-power? Can I damage my

dynamic or ribbon mic by plugging it in to phantom power?

Condenser microphones have internal electronics that need power

to operate. Batteries, or separate power supplies using multi-

conductor cables powered early condenser microphones. In the late

1960's, German microphone manufacturers developed 2 methods of

sending power on the same wires that carry the signal from the

microphone.

The more common of these methods is called "phantom power" and is

covered by DIN spec 45596. The positive terminal of a power

supply is connected through resistors to both signal leads of a

balanced microphone, and the negative terminal is connected to

ground. 48 volts is the preferred value, with 6800 ohm resistors

in each leg of the circuit, but lower voltages and lower resistor

values are also used. The precise value of the resistors is not

too critical, but the two resistors must be matched within 0.4%.

Phantom power has the advantage that a dynamic or ribbon mic may

be plugged in to a phantom powered microphone input and operate

without damage, and a phantom powered mic can be plugged in to

the same input and receive power. The only hazard is that in case

of a shorted microphone cable, or certain old microphones having

a grounded center tap output, current can flow through the

microphone, damaging it. It's a good idea anyway to check cables

regularly to see that there are no shorts between any of the

pins, and the few ribbon or dynamic microphones with any circuit

connection to ground can be identified and not used with phantom

power.

There is also a very slim chance with a dynamic or ribbon mic

plugged in to phantom power over a very long cable, that one side

of the circuit might make contact sooner than the other  side,

and result in a burst of current charging the capacitance of the

cable. If you want to be doubly sure there is no such

possibility, turn off all phantom power before plugging in or

unplugging any microphone. This also prevents the huge pop that

will come out of your speakers if you plug in something phantom

powered with the power on.

One other reason to be careful about plugging mics in or unplugging

them with phantom power on (if there is any sort of transformer or

magnetic device involved) is that the pulse of current mentioned

above can cause the mic's output transformer, or the preamp's

input transformer, to become magnetized. This causes a major increase

in 2nd harmonic distortion at low frequencies. Many low level signal

transformers, especially small ones with mostly steel cores, have become

magnetized over the years and contribute a lot to the muddy or dark sound

of some equipment. The techniques for demagnetizing a transformer are

varied: using a heavy-duty bulk tape eraser works (don't do this with a

dynamic or ribbon mic, or you'll demagnetize the magnet!), or you can send

a signal of a few volts at 50-100 Hz into the transformer itself. You

can easily damage the mic or preamp if you aren't careful about the

signals either of these methods may induce in the rest of the circuit.

When in doubt, remove the transformer from the circuit and demag it

by itself, and check the 2nd harmonic distortion to see whether you've

got it right. A clean core will contribute no more than 0.001% 2nd

harmonic at 50 Hz, while a magnetized one can easily reach into the

tenths of a percent. [12/98]

T-power (short for Tonaderspeisung, also called AB or parallel

power, and covered by DIN spec 45595) was developed for portable

applications, and is still common in film sound equipment. T-

power is usually 12 volts, and the power is connected across the

balanced pair through 180 ohm resistors. Only T-power mics may be

connected to T-power inputs; dynamic or ribbon mics may be

damaged and phantom powered mics will not operate properly.

[David]

--

Q3.6 - How do I interconnect balanced and unbalanced components?

First, let's define what the terms mean. The simplest audio

circuit uses a single wire to carry the signal; the return path,

which is needed for current to flow in the wire, is provided

through a ground connection, usually through a shield around the

wire.  This system, called unbalanced transmission, is very

susceptible to hum pickup and cannot be used for low level

signals, like audio, for more than a few feet. Balanced

transmission occurs when two separate and symmetrical wires are

used to carry the signal. A balanced input is sensitive only to

voltage that appears between the two input terminals; the circuit

cancels signals from one terminal to ground. The simplest way to

connect between balanced and unbalanced equipment is to use a

transformer. The signals are magnetically coupled through the

core of the transformer and either side may be balanced or

unbalanced. Good transformers are expensive, however, and there

are cheaper methods that can be used in some instances.

An unbalanced output can be connected to a balanced input. For

instance, from the unbalanced output of a CD player, connect the

center pin to pin 2 of the balanced XLR input connector, and the

ground to pins 1 and 3.  To connect the balanced output of

something to an unbalanced input requires different techniques

depending on whether the output is active balanced (each side has

a signal with respect to ground) or floating balanced (for

instance, the secondary of a transformer with no center-tap

connection). If it's an active balanced output, you can simply

use half of it; connect pin 2 to the unbalanced input, and pin 1

to ground, leaving pin 3 floating. If this doesn't work (no or

very weak signal) connect pin 3 of the output to pin 1 and ground

and leave pin 2 connected to the unbalanced input center pin.

Some active balanced outputs, particularly microphones, use the

balanced circuit to cancel distortion, so this hookup may result

in higher distortion than if a proper balanced-to-unbalanced

converter such as a differential stage or a transformer were

used. [David]

--

Q3.7 - What are ground loops and how do I avoid them?

One of the most difficult troubleshooting tasks for the audio

practitioner is finding the source of hum, buzz and other

interfering signals in the audio signal. Often these are caused

by "ground loops." This unfortunate and inaccurate term (it need

not be in the "ground" path, and the "loop" is not what causes

the problem) is poorly understood by most users of audio

equipment. A better name for this phenomenon is "shared path

coupling" because it happens when two signals share the same

conductor path and couple to each other as a result.

Another semantic problem that should be addressed early on is the

idea that "ground" is one place where all currents go. It's not,

there's nothing special about calling a signal "ground," current

still flows through any path that's available to it.

Referring to the discussion above regarding unbalanced signal

paths, recall that there must be a complete circuit from the

output of some device, through the input of another device and

back to the "return" side of the output if any current is to

flow. Current doesn't flow by itself, it must have a complete

path. If there are multiple paths over which the current might

flow, the current will be divided among them with most of the

current flowing through the path having the least resistance. Any

available path, regardless of the resistance in it, will carry

some of the current, it's not a case of all the current following

the path that has least resistance.

For example, suppose we have two units connected together through

a small piece of coaxial cable, and the units are also connected

together at the wall outlet through their grounded power cords --

the ground pins are connected to the chassis at each end. The

audio signal goes along the center of the coaxial cable, and part

of it might come back along the shield of the coax, but part will

also go through the ground wire of one unit and back through the

ground wire of the other unit. A problem arises when some other

signal is also flowing through this same return path. The other

signal might be another audio signal, video, data, or power. All

of the currents in a wire add together, and the resistance of the

wire causes a voltage to appear in proportion to the current

flowing. All of these voltages add together, so there is a little

bit of the video signal added to the audio, some of the power

signal added to the video, some of the power signal added to the

audio, etc. In rare instances, the "loop" of wire formed by the

intended ground return path. The happenstance lower resistance

return path formed by mounting hardware, power cords, etc. can

form a magnetic pickup as well, so that magnetic fields radiated

by transformers, CRT's, etc. can also induce a current in the

"loop," which makes yet another source of noise voltage.

This shared path coupling is a constant problem with unbalanced

audio systems. Lots of different methods have been tried to get

around the problem, many of them dangerous. Clipping off the

ground leads of equipment so there is no common power line path

between them simply makes any fault or leakage current follow

some other path. Back through the signal cable to some equipment

that has a ground -- perhaps through the user's body, if all the

ground pins have been removed. The only general solution to

"ground loop" coupling with unbalanced equipment is to connect

all the chassis together with a very low resistance path (copper

strap or braid, for example), on the principle that since the

resistance is so low, any leakage current will produce a

correspondingly low signal voltage. It may also be effective to

interrupt the ground path of shield conductors over signal wires;

force the return path to go through the designated common strap

while leaving the shield in place only for electrostatic

screening.

With balanced equipment, no current should be flowing in the

shield conductors, and in fact performance should be identical

with the shield left disconnected at one end (preferably the

receiver end). Therefore balanced systems should be impervious to

shared path coupling or "ground loop" problems. But in fact they

aren't, because most signals inside a given piece of equipment

are unbalanced, and there are often return paths internal to the

equipment that can be shared with return paths between other

units of equipment connected to it. Especially with mixed

digital, video and audio signals and high gain, high negative

feedback amplifier circuitry, this can be a big problem -- small

currents can create big effects -- and this brings us to the next

question.  [David]

--

Q3.8 - What is the "Pin 1 problem" and how do I avoid it?

This is a special case of "ground loop" or shared path coupling.

Recently a group led by the consultant Neil Muncy of Toronto has

discussed this in great detail and clarity. Suppose you have a

mixer, whose balanced output is connected to an amplifier's

balanced input through a correctly wired cable. Both units are

powered from the AC mains and one or both have some small amount

of AC leakage current that travels to ground through all

available ground paths -- including the shield of the cable that

connects the two units. So far so good, no harm done because the

circuit is balanced and any common mode voltage from current

flowing through the shield will be canceled by the amplifier

input. However... a small part of this leakage current also

travels through the shield of the wire going from the back panel

XLR connector to the PC board, through some "ground" traces on

the PC board, and back out through the power line ground cable.

No problem so far, except that some gain stage on that same PC

board also uses that piece of ground trace in its negative

feedback loop, and some part of that leakage signal will be added

to the signal in that gain stage. It might be video, or data, or

other audio signal(s), or (most commonly) power.

The solution to this variant of shared path coupling is the same

sort of approach that applies to other unbalanced signals: give

the leakage current a very low resistance path to follow, and

remove as many of the shared paths as possible. Within a unit of

equipment, all the XLR connectors' pin 1 terminals should be

connected to ground with very low resistance (big) wire or

traces, and preferably all of the ground connections should be

made at one point, the so-called "star ground" system. A brute

force approach is to assume that the back panel is the star

ground, and wire every connector's pin 1 solidly to the panel as

directly as possible, and lift all the ground wires but one that

go from the connectors to the circuitry. In this way, all the

external leakage currents (the "fox" to use Neil Muncy's term)

will be conducted through the back panel and out of the way.

Rather than running them through the ground traces on the PC

board where they will mix with internal low level signals in high

gain stages (the "hen house"). Individual wires can be run from

points on the circuit board that need to be at "ground" potential

to a common point on the back panel, which is designated a "zero

signal reference point" (ZSRP). Equipment that has a reputation

for being "quiet" and easy to use in many different applications

is often found to be wired this way, while equipment that is

"temperamental" if often found to be wired in such a way that

leakage currents are easily coupled to internal signal lines.

There's a simple test that can be done to check equipment

susceptibility to this problem. Connect the output, preferably

balanced and floating, of an ordinary audio oscillator to the pin

1 of any two XLR connectors on the equipment. Now operate the

equipment through its various modes, gain settings, etc. You may

be surprised to find the audio oscillator's signal appearing in

many different places in the equipment.

All of these issues are expertly explained in a single issue of

the Journal of the AES, June 1995. You can get this as a reprint

from the AES office in New York, http://www.aes.org/ [David]

=====

Section IV - Analog tape recording

--

Q4.1 - What does it mean to "align" a tape machine?

There are a number of standard adjustments on any analogue tape

machine, which can roughly be broken up into mechanical and

electronic adjustments.  The mechanical adjustments include the

head position (height, skew, and azimuth), and sometimes tape

speed.  Incorrect head height will result in poor S/N and leakage

between channels, because the tracks on the head do not match up

exactly with those on the tape. Incorrect tape skew will result

in level differences between channels and uneven head wear,

because there is more pressure on the top of the head than the

bottom (or vice versa).  Incorrect azimuth will result in loss of

high frequency response and strange skewing of the stereo image.

Tape speed error will result in tonal shifts. Although on many

machines with capstan speed controlled by crystal or line

frequency, it is not adjustable.

Electronic adjustments include level and bias adjustments for

each channel.  Some machines may have bias frequency adjustments,

equalization adjustments for playback and record emphasis, pre-

distortion adjustments, and a varied bevy of adjustments for

noise reduction systems.

Alignment is relatively simple, and the same general method

applies from the smallest cassette deck to the largest multi-

track machine. First, put a test tape on the machine.  Use a real

reference tape, from the manufacturer, from MRL

<http://www.flash.net/~mrltapes>, or a similarly legitimate lab.

DO NOT EVER use a homebrew test tape that was recorded on a

"known good" machine.  You will regret it someday.  Spend the

money and get a real test tape (and not one of the flaky ones

from RCA).

1. Speed adjustment (if necessary).  Play back a 1 KHz reference

tone and, using a frequency counter, adjust the tape speed for

proper frequency output.  There are strobe tapes available for

this as well, but with cheap frequency counters available,

this method is much easier.

2. Head height and skew adjustments.  Better see your machine's

manual on this one, because I have seen a variety of ways of

doing this. If in doubt,leave it alone.

3. Azimuth adjustment.  I find the easiest way to do this is to

take the left and right outputs and connect them to the X and

Y inputs of an oscilloscope, and play back a 10 kHz reference

tone, while adjusting the azimuth until a perfectly-diagonal

line appears. You can do this by ear if you are desperate, but

I strongly recommend the Lissajous method, which is faster and

more accurate.  On multi-track decks, use the two tracks as

close as possible to the opposite edges of the tape. Now you have

the playback head azimuth set... put a 10 kHz source into the

record input, with a blank tape on the machine, and adjust the

azimuth of the record head for the proper diagonal line from

the playback output. If the alignment is very far off to begin with,

it might be easier to do these steps first with a 1 kHz tone to

get the azimuth adjustments close, and then switch to a 10 kHz

tone for fine adjustment.

4. Playback level adjustment.  Adjust the playback meters to show 0 dB

when a 1 Khz 0 dB signal is played back.  (This assumes that your reference

tape has the same fluxivity as the tape you're going to be using.... if you

want to work at a hotter level, you may want to adjust the meters to show

a level lower than 0 dB or get a different tape... and there should be a

set of markings on the tape to tell you what levels to set for what

fluxivities).

5. Playback eq adjustment (if necessary).  This is a case of

playing back various test tones at different frequencies, and

adjusting the response curve of the deck to produce a flat

output.  You can also do this by playing back white noise and

using a third-octave spectrum analyzer of great accuracy to

adjust for flat response.  Again, this is one to check your

deck's manual for, because the actual adjustments vary from

one machine to another, and you will want to use the test tape

once again. There is a good explanation of various choices in tape

equalization at http://www.flash.net/~mrltapes/choo&u.pdf. Most commonly,

though, there is a low end response setting that is adjusted around 100 Hz

and a high end response setting that is adjusted around 15 kHz.  It's

often a good idea to look at the various levels of a stepped tone sequence,

too, just to make sure the playback levels really are flat.

Now it's time to take the reference tape off the machine, and put a

blank tape OF THE TYPE YOU ARE GOING TO BE USING on the machine. Set

up the machine or external meters to look at the repro levels coming off

the tape.

6. Bias adjustment.  There are a lot of ways to do this.  The common way to

do this is to record a high frequency to the tape, and adjust the bias until

the signal peaks, then increase the level of the bias until the signal

coming off the tape drops by a little bit.  That little bit is called the

overbias, and you should check on the data sheet of the tape you are using

and see what level of overbias is recommended for that tape.  Usually this is

done at 10 kHz for 15 ips tape.

Other folks will use a 1 KHz tone and adjust for a peak with no overbias,

and this is the method recommended in the manual for many older tape machines.

However, it's not optimal for most of the newer tape formulations.  Some other

folks record a low frequency tone and adjust for lowest modulation noise, a

method which also has adherents.  Previous copies of this FAQ recommended

adjusting for flattest response, a method I no longer recommend.

7. Record eq adjustment (if necessary).  How this is done (and whether

you want to do it after biasing the tape) depends a lot on your deck. The

idea is to adjust the controls, again usually one for low frequencies and

one for highs, so that the playback output tracks the record input. You

have already adjusted the playback eq so that it plays an alignment tape

with uniform output, now you adjust the record eq so that the record

section prints the same flat response on to the tape. It's important to

reduce the record level when doing these adjustments, so that the high

frequencies don't overload the tape.

8. Record level adjustment.  This is really nothing more than calibrating

the record VU meter. The most common way to do this is to just adjust so

that the level coming off the tape is the same as the level going on the

tape.  Adjust the meter so you get the same level set to repro and to

input monitor modes.  Other folks prefer to set the levels for a

particular distortion level, either by using a 1 KHz tone and setting so

the onset of audible distortion is at +3 dB or using a distortion analyzer,

then going back and resetting the playback EQ so that the levels match

(thereby ignoring the level set from the reference tape).

At this point, you will be pretty much set.  Whether you want to

do this all on a regular basis is a good question.  You should

definitely go through the complete procedure if you ever change

brands of tape.  Checking the mechanical parameters on a regular

basis is a good idea with some decks (like the Ampex 350), which

tend to drift.  Clean your heads every time you put a new reel

on, and demagnetize regularly.  [Scott, David]

There are other good references for aligning a tape deck available

on the web at the following URL's.

Larry Seyer - How To Align An Analog Tape Recorder

http://www.larryseyer.com/align.htm

Bill Vermillion - Magnetic Tape Recorder Alignment

http://www.arachnaut.org/service/bilver-align.html

Jay McKnight, MRL - Choosing and Using MRL Calibration Tapes

http://www.flash.net/~mrltapes/choo&u.pdf

[Mark, David]

--

Q4.2 - What is bias?  What is overbias?

With just the audio signal applied to a tape, the frequency

response is very poor.  High frequency response is much better

than low frequency, and the low frequency distortion is very

high.  In 1906, the Poulson Telegraphone managed to record an

intelligible voice on a magnetic medium, but it was not until the

1930s when this problem was solved by German engineers.

To compensate for the tape characteristic, a very high frequency

signal is applied to the tape in addition to the audio.  This is

typically in the 100 kHz range, far above the audio range.  With

the bias adjusted properly, the frequency response should be flat

across the audible range.  With too low bias, bass distortion

will be the first audible sign, but with too much bias, the high

frequency response will drop off.

The most popular method of setting optimal bias is to set the bias so

that the reproduction of a 10 kHz tone (at 15 ips) is as high as possible,

then to increase the bias until the reproduced level drops a certain amount

(called overbias).  The amount of optimal overbias differs from one tape

to another.

Incidentally, digital recording equipment takes advantage of the

very non-linearity that is a problem with analogue methods.  It

records a square wave on the tape, driving the tape into

saturation at all times, and extracts the signal from the

waveform edges.  As a result, no bias is required.  (For a good

example of the various digital recording methods, check out NASA

SP 5038, Magnetic Tape Recording.) [Scott]

[Ed. note: For those looking for an understanding of why we need

bias in the first place, here is one way to think about it.  Tape

consists of lots of small magnetic particles called domains.

These domains are exposed to a magnetic field from the record

head and oscillate in polarity as the AC signal voltage changes.

Domains, being physical objects, have inertia.  Every time the

analog signal crosses from positive to negative and back again,

the voltage passes the zero point for an instant.  At this

moment, the domain is at rest, and like any other physical

object, there is a short period of inertia before it gets moving

again.  The result is the bizarre high-frequency performance

characteristic that Scott described.  The high frequency of a

bias signal simply ensures that the domains are always kept in

motion, negating the effect of inertia at audio frequencies.  -

[Gabe]

--

Q4.3 - What is the difference between Dolby A, B, C, S, and SR?  How

do each of these systems work? How do they affect the sound?

The Dolby A, B, C, SR, and S noise reduction (NR) systems are

non-linear level-dependent companders (compressors/expanders).

They offer various amounts of noise reduction, as shown in the

table below.

Dolby   HF NR   LF NR  Number Of Active              Target

System  Effect  Effect Frequency Bands               Market     Year

------  ------  ------ ----------------------------  ---------  ----

A     10 dB   10 dB  4 fixed                       Pro audio  1967

B     10 dB   --     1 sliding (HF)                Domestic   1970

C     20 dB   --     1 sliding (HF)                Domestic   1981

SR    24 dB   10 dB  1 sliding (HF), 1 fixed (LF)  Pro audio  1986

S     24 dB   10 dB  1 sliding (HF), 1 fixed (LF)  Domestic   1990

------  ------  -----  ----------------------------  ---------  ----

The band-splitting system used with Dolby A NR is a relatively

costly technique, although it can deal with noise at all

frequencies. The single sliding band techniques used in Dolby B

and C systems are less costly, making them more suitable for

consumer tape recording applications where the dominant noise

contribution occurs at high frequencies.

The typical on-record frequency response curves for the Dolby B

NR system look something like those depicted below. The curves

for Dolby C, SR, and S are similar, but the actual response

levels and behavior at high frequencies are modified to extract

better performance form these more advanced systems.

|

0dB -|----------------------------------------------------

|

|

-10dB -|

|                    /-------------------------------

|                   /

-20dB -|------------------/

|

|

-30dB -|                      /-----------------------------

|                    /

|                  /

-40dB -|----------------/

|______________________________________________________

|                   |                                 |

20Hz                1kHz                             20kHz

The above picture attempts to show that the encoding process

provides selective boost to high frequency signals (decoding is

the exact reciprocal), and the curves correspond to the results

achieved when no musical signal is applied. The amount of boost

during the compansion depends on the signal level and its

spectral content. For a tone at -40dB at 3 KHz, the boost applied

to signals with frequencies above this would probably be the full

10dB allowed by the system. If the same tone were at a level of -

20dB, then the boost would be less, maybe about 5dB. If the tone

was at 0dB, then no boost would be supplied, as tape saturation

would be increased (beyond its normal amount).

The single band of compansion utilized with Dolby B NR reaches

sufficiently low in frequency to provide useful noise reduction

when no signal is present. Its width changes dynamically in

response to the spectral content of music signals. As an example,

when used with a solo drum note the companding system will slide

up in frequency so that the low frequency content of the drum

will be passed through at its full level. On replay, the playback

of the bass drum is allowed to pass through without modification

to its level, while the expander lowers the volume at high

frequencies above those of the bass drum, thus providing a

reduction in tape hiss where there is no musical signal. If a

guitar is now added to the music signal, the companding band

slides further up in frequency allowing the bass drum and guitar

signals through without any compansion, while still producing a

worthwhile noise reduction effect at frequencies above those of

the guitar.

The Dolby B NR system is designed to start taking effect from

300Hz, and its action increases until it reaches a maximum of

10dB upwards of 4kHz. Dolby C improves on this by taking effect

from 100Hz and providing about 15dB of NR at 400Hz, increasing to

a maximum of 20dB in the critical hiss region from 2kHz to 10kHz.

Dolby C also includes spectral skewing networks, which introduce

a roll off above 10kHz prior to the compander when in encoding

mode. This helps to reduce compander errors caused by

unpredictable cassette response above 10kHz, and an inverse boost

is added after the expander to compensate. Although this reduces

the noise reduction effect above 10kHz, the ear's sensitivity to

noise in that region is diminished, and the improved

encode/decode tracking provides important improvements in overall

system performance. An anti-saturation shelving network,

beginning at about 2kHz, also acts on the high frequencies but it

only affects the high-level signals that would cause tape

saturation. A complementary network is provided in the decode

chain to provide overall flat response.

When the tape is played back, the inverse of the above process

takes place. For an accurate decoding to occur, it is necessary

that playback takes place with no offsets in levels between

record and replay. IE. If a 400 Hz tone is recorded at 0dB (or -

20dB), then it must play back at 0dB (or -20dB). This will help

ensure correct Dolby "tracking".

Just think about it: if a -40dB tone at 8kHz was recorded with

Dolby B on, then it would actually have a level of -30dB on tape.

The same tone, if it were at a -20dB level, would have a level of

about -15dB on tape. If the sensitivity of the tape was such that

anything recorded at 0dB actually went on tape as -10dB, then you

can see that the Dolby encoded tones would actually be at a lower

level, and the system would have no way of determining this. It

assumes 0dB in = 0dB out. Hence the signal would be decoded with

the incorrect amount of de-boost.

The Dolby SR and S NR systems provide slightly more NR than Dolby

C at high frequencies, 24dB vs. 20dB, but they also achieve a

10dB NR effect at low frequencies below 200Hz as well. This is

obtained using a two-band approach, the low frequencies being

handled by a fixed-band processor, while a sliding band processor

tackles the high frequencies. This reduces the potential for

problems such as "noise pumping", caused by high-level low

frequency transient signals (bass notes from drums, double

basses, organs), raising the sound level in a cyclic fashion.

Dolby SR and S also contain the spectral skewing and anti-

saturation circuits for high-level high-frequency signals that

are implemented with Dolby C. The performance of the sliding band

is improved over that obtained with Dolby B and C NR systems by

reducing the degree of sliding that occurs in the presence of

high-frequency signals. This increases the noise reduction effect

available at frequencies below those occurring in the music

signal.

An additional benefit of the Dolby S NR system for consumers is

that the manufacturers of cassette decks who are licensed to use

the system must adhere to a range of strict performance

standards. These include an extended high frequency response,

tighter overall response tolerances, a new standard ensuring head

height accuracy, increased overload margin in the electronics,

lower wow and flutter, and a head azimuth standard. These benefit

users by enhancing the performance of cassette recorders as well

as helping to ensure that tapes recorded on one deck will play

back accurately on any other. [Witold Waldman - witold@aed.dsto.gov.au]

Improvement in signal-to-noise ratio, or any other parameter for

that matter, doesn't come without a price. In the case of Dolby

noise reduction, the calibration of record and playback levels is

critical. Without the right setup, the wrong part of the playback

transfer curve will be overlaid on the record transfer curve,

with the result that there's a strange bump in the overall

linearity of the recording. So for any of these methods, it is

essential to read and understand the Dolby setup procedure and

make sure that the calibration tone (which also uniquely

identifies the type of Dolby being used) is recorded at the

correct level. The playback unit can then be matched to that

level.

Once the levels are set correctly, the remaining sonic artifacts

have to do with the tape being pushed closer to its limits in the

extremes of the frequency range. The high frequency information

thus sometimes seems a bit more compressed (besides being

accompanied by less noise.) And, some would argue that running

the audio through another dozen or more op-amps per channel must

create sonic artifacts too. [David]

--

Q4.4 - What is Dolby HX-Pro?

HX-Pro is a scheme to reduce the level of the bias signal when

high frequency information is present in the recorded signal.

Sufficient high frequency information will act to bias the tape

itself, and by reducing the AC bias signal somewhat, additional

signal can be applied without saturating the tape.  This is a

single-ended system; it requires no decoding on playback, because

it merely permits more signal to be recorded on the tape.  In

theory it is an excellent idea, and some implementations have

lived up to the promise of the method, although some other

implementations have produced unpleasant artifacts. [Scott]

--

Q4.5 - How does DBX compare to Dolby?

[Anyone?] [Gabe]

It's easier to spell. [Scott]

--

Q4.6 - How much better are external microphone preamplifiers than

those found in my portable recorder?

Going by the rule that "external is better than internal," the

external pre-amps are likely to sound better.  Besides the issue

of electrical shielding and interaction it is simply the case

that a designer who is spending *all* his time on a project

designing only a pre-amp is likely to do a better job of it. As

opposed to a tape machine design team that has to worry how

they're going to fit the pre-amp into the box and still have

enough room for the rest of the tape machine. [Gabe]

--

Q4.7 - What is an MRL?  Where do I get one?

An MRL is a reference alignment tape from Magnetic Reference

Laboratory. These tapes, available in every conceivable tape

speed, tape width, equalization, and field strength, contain

alignment tones useful in calibrating the electronics of analog

tape machines.

These tapes can be ordered from many pro audio dealers.  If not,

you can contact MRL directly at:

Magnetic Reference Laboratory, Inc.

229 Polaris Avenue, Suite 4

Mountain View, CA 94043

Tel: (650) 965-8187

Fax: (650) 965-8548

http://www.flash.net/~mrltapes

=====

Section V - Digital recording and interconnection

--

Q5.1 - What is sampling?  What is a sampling rate?

Sampling can be (roughly) defined as the capture of a

continuously varying quantity at a precisely defined instant in

time. Most usually, signals are sampled at a set of sample-points

spaced regularly in time. Note that sampling in itself implies

nothing about the representation of sample magnitude by a number.

That process is called quantisation.

The Nyquist theorem states that in order to faithfully capture

all of the information in a signal of one-sided bandwidth B, it

must be sampled at a rate greater than 2B. A direct corollary of

this is that if we wish to sample at a rate of 2B then we must

pre-filter the signal to a one-sided bandwidth of B, otherwise it

will not be possible to accurately reconstruct the original

signal from the samples. The frequency 2B that is the minimum

sample rate to retain all of the signal information is called the

Nyquist frequency.

The spectrum of the sampled signal is the same as the spectrum of

the continuous signal except that copies (known as aliases) of

the original now appear centered on all integer multiples of the

sample rate. As an example, if a signal of 20 KHz bandwidth is

sampled at 50 KHz then alias spectra appear from 30 - 70 KHz, 80

- 120 KHz, and so on. It is because the alias spectra must not

overlap that a sample rate of greater than 2B is required.  In

digital audio we are concerned with the base-band - that is to

say the signal components which extend from 0 to B. Therefore, to

sample at the standard digital audio rate of 44.1 KHz requires

the input signal to be band-limited to the range 0 Hz to 22.05

KHz.  [Chris]

--

Q5.2 - What is oversampling?

To take distortion-less samples at 44.1kHz requires that the

analogue signal be band limited to 22.05kHz. Since the audio band

is reckoned to extend to 20kHz we require an analogue filter that

cuts off very sharply between 20kHz and 22kHz to accomplish this.

This is expensive, and suffers from all the ailments associated

with analogue electronics.

Oversampling is a technique whereby some of this filtering may be

done (relatively cheaply and easily) in the digital domain. By

sampling at a high rate (for example 4 times 44.1kHz, or

176.4kHz) the analogue filter can have a much lower slope since

its transition band is now 20kHz to 88kHz (i.e. half of 176kHz).

The samples are then passed through a digital filter with a sharp

cutoff at 20kHz, after which three of every four are discarded,

resulting in the sample stream at 44.1kHz that we require.

[Chris]

--

Q5.3 - What is the difference between a "1 bit" and a "multibit"

converter? What is MASH?  What is Delta/Sigma?  Should I really

care?

Audio data is stored on CD as 16-bit words. It is the job of the

digital to analogue converter (DAC) to convert these numbers to a

varying voltage. Many DAC chips do this by storing electric

charge in capacitors (like water in buckets) and selectively

emptying these buckets to the analogue output, thereby adding

their contents. Others sum the outputs of current or voltage

sources, but the operating principles are otherwise similar.

A multi-bit converter has sixteen buckets corresponding to the

sixteen bits of the input word, and sized 1, 2, 4, 8 ... 32768

charge units. Each word (i.e. sample) decoded from the disc is

passed directly to the DAC, and those buckets corresponding to

1's in the input word are emptied to the output.

To perform well the bucket sizes have to be accurate to within

+/- half a charge unit; for the larger buckets this represents a

tolerance tighter than 0.01%, which is difficult.  Furthermore

the image spectrum from 24kHz to 64kHz must be filtered out,

requiring a complicated, expensive filter.

Alternatively, by using some digital signal processing, the

stream of 16-bit words at 44.1kHz can be transformed to a stream

of shorter words at a higher rate. The two data streams represent

the same signal in the audio band, but the new data stream has a

lot of extra noise in it resulting from the word length

reduction. This extra noise is made to appear mostly above 20kHz

through the use of noise shaping, and the oversampling ensures

that the first image spectrum occurs at a much higher frequency

than in the multi-bit case.

This new data stream is now converted to an analogue voltage by a

DAC of short word length; subsequently, a simple analogue filter

can filter out most of the noise above 20kHz without affecting

the audio signal.

Typical configurations use 1-bit words at 11.3MHz (256 times over

sampled), and 4-bit words at 2.8MHz (64 times over sampled).  The

former requires one bucket of arbitrary size (very simple); it is

the basis of the Philips Bitstream range of converters. The

latter requires four buckets of sizes 1, 2, 4 and 8 charge units,

but the tolerance on these is relaxed to about 5%.

MASH and other PWM systems are similar to Bitstream, but they

vary the pulse width at the output of the digital signal

processor. This can be likened to using a single bucket but with

the provision to part fill it. For example, MASH allows the

bucket to be filled to eleven different depths (this is where

they get 3.5 bits from, as 2^(3.5) is approximately eleven).

Lastly it is important to note that these are all simply

different ways of performing the same function. It is easy to

make a lousy CD player based around any of these technologies; it

is rather more difficult to make an excellent one, regardless of

the DAC technology employed. Each of the conversion methods has

its advantages and disadvantages, and as ever it is the job of

the engineer to balance a multitude of parameters to design a

product that represents value for money to the consumer.  [Chris]

--

Q5.4 - On an analog recorder, I was always taught to make sure the

signal averages around 0 VU.  But on my new DAT machine, 0 is all

the way at the top of the scale.  What's going on here?

Analog recorders are operated such that the signal maintains a

nominal level that strikes a good balance between signal-to-noise

ratio and headroom.  Further, since analog distorts very gently,

you often can exceed your headroom in little bits and not really

notice it.

Digital is not nearly as forgiving.  Since digital represents

audio as numerical values, higher levels will eventually force

you to run out of numbers.  As a result, there is an absolute

ceiling as to how hot you can record.  If you record analog and

have a nominal 12 dB of headroom, you'll probably be okay if you

have one 15 dB transient that lasts for 1/10th of a second.  The

record amps _might_ overload, the tape _might_ saturate, but

you'll probably be fine.  In a digital system, that same 3 dB of

overshoot would cause you to clip hard.  It would not be subtle

or forgiving.  You would hear a definite snap as you ran out of

room and chopped the top of your waveform off.

The reality is that digital has NO HEADROOM, because there is no

margin for overshoot.  You simply must make sure that the entire

dynamic range of the signal fits within the limits of the dynamic

range of your recorder, without exception.  The only meaningful

absolute on a digital recorder, therefore, is the point at which

you will go into overload.  The result is the metering system we

now have. 0 dB represents digital ceiling, or full-scale.  The

negative numbers on the scale represents your current level

relative to the ceiling.

Thus, to return to our example, if you have a transient with 15dB

of overshoot past your nominal level, you must then place your

nominal level at a maximum of -15 dB.  0 dB on the meters is the

absolute limit of what you can record.  [Gabe]

--

Q5.5 - Why doesn't MiniDisc or Digital Compact Cassette sound as good

as DAT or CD?  After all, they're both digital.

Both MD and DCC use loss-y compression algorithms (called ATRAC

and PASC respectively); crudely, this means that the numbers

coming out of the machine are not the same as those that went in.

The algorithms use complex models of the way the ear works to

discard the information that it thinks would not be heard anyway.

For example, if a pin dropped simultaneously with a gunshot, it

may be reasonable to suggest that it isn't worth bothering to

record the sound of the pin! In fact it turns out that around 75

to 80 per cent of the data for typical music can be discarded

with surprisingly little quality loss.

However, nobody denies that there is a quality loss, particularly

after a few generations of copying. This fact and others make

both MD and DCC useful only as a consumer-delivery format. They

have very little use in the studio as a recording or (heaven

forbid!) mastering format.  [Chris]

Recent advances have made the MD much closer to the uncompressed

signal.  They have also been integrated into a number of lower

cost 4 and 8 track recording devices.  MD is now also found in a

number of radio stations.  [Joe] [9/98]

--

Q5.6 - What is S/P-DIF?  What is AES/EBU?

AES/EBU and S/P-DIF describe two similar protocols for

communicating two-channel digital audio information over a serial

link. They are slightly different in details, their basic format

is almost identical, but there are enough differences that the

two are, for all intents and purposes electrically incompatible.

Both of these digital protocols are described fully in an

international standard, IEC 958, available from the International

Electrotechnical Commission.

AES/EBU (which stands for the joint Audio Engineering

Society/European Broadcasting Union standard) is the so-called

"professional" protocol. It uses standard 3-pin XLR connectors

and 110-ohm balanced differential cables for connection (no,

standard microphone cables, not even good quality cables, won't

work, even though it seems they might) and a 5 volt, differential

signal. This is now properly called AES-3, and there is a version

called AES-3id which allows the same format over coaxial cable.

S/P-DIF (which stands for Sony/Philips Digital InterFace, a now

obsolete standard superseded by IEC 958) is the so-called

"consumer" format. It uses what appears to be standard RCA

connectors and cables, but, in fact, require 75-ohm connectors

and cables. Good quality video "patch" cables have proven

adequate (no, standard "audio" patch cords, even excellent

quality versions, have been shown not to work). The signals are

0.5 volts unbalanced.

The actual data streams are very similar.  Each sample period a

"frame" is transmitted. Each frame consists of two "subframes",

one each for left and right channels, each subframe is 32 bits

wide. In that subframe, 4 bits are used for synchronization, then

up to 24 bits are usable for audio (the "consumer mode" format is

limited to 16 bits). The remaining four bits are used for parity

(the first level of error detection), validity, user status and

channel status. 192 subframes are collected, and the 192 user

bits and 192 channel status bits are collected into separate 24 8

bit status bytes for each channel.

The channel status bytes are interesting, because they contain

the important control information and the major differences

between the two protocol formats. One bit tells whether the data

stream is professional or consumer format.  There are bits that

specify (optionally) the sample rate, de-emphasis standards,

channel usage, and other information.  The consumer format has

several bits allocated to copy protection and control: the SCMS

bits.

Now, the notion that all of this is encoded in a standard may be

reassuring, but a standard is nothing but a voluntary statement

of common industry practice. There is a lot of incompatibility

between equipment out there caused directly by subtle differences

between interpretations and implementations.  The result is that

some equipment simply refuses to talk to each other.  Even THAT

possibility is stated in the standard!  [Dick]

--

Q5.7 - What is clock jitter?

Clock jitter is a colloquialism for what engineers would readily

call time-domain distortion.  Clock jitter does not actually

change the physical content of the information being transmitted,

only the time at which it is delivered.  Depending on

circumstance, this may or may not affect the ultimate decoded

output.

Let's look at this a little more closely.  Digital audio is sent

as a set of binary digits 1's and 0's.  But that is only a

logical construct.  In order to transmit binary math

electrically, we use square waves.  Realize that although we have

two mathematical states, we have to transmit such a construct

using control voltages and comparators.

All digital audio systems start with a crystal controlled

oscillator producing a square wave signal that is used to

synchronize the entire digital audio sampling and playback

processes.  Now, for a clock, we don't really care about the fact

that the clock might be at state 1 or state 0 at any given

moment.  That doesn't give us any information. As a computer, I

can't tell if my clock has just gotten to state 1, or if it's

been sitting there for a microsecond.  Thus it isn't the states

we care about.  Instead, we care about the state changes - when

the clock shifts from one state to the other.

Now, in a perfect square wave (no such thing exists), the change

of state would be instantaneous.  BOOM...it's done.  But in

reality, it doesn't work this way.  Square waves contain high

orders of harmonics. Fourier teaches us that all complex

waveforms are made up of simpler waveforms.  Thus, as we run

through noisy electronics, long cables, inadvertent filtering

circuits, we begin to lose some of our harmonics.  When this

happens, our square wave begins to lose form.

The result of this is that our nice sharp corners become rounded.

So our state changes are no longer precisely at the edge anymore,

because there is no more edge.  The pointy edge is now all fuzzy.

It now depends on design of the electronic comparator circuit as

to when the clock state will change, as the stage change has

shifted.  The clock is, essentially, jittering.

People love to bark out "Bits is bits.  A copy of a computer file

works as well as the original."  Yes, this is true.  But these

jittering bits can create audible distortion during the digital

to analog conversion, and the industry is working hard to reduce

the amount of jitter present in digital systems.

Furthermore, emerging research is suggesting that certain types

of jitter may produce digital copies with eccentricities that

result in more jittery output on playback.  The jury is still out

on the specifics however.  Stay tuned.  [Gabe]

--

Q5.8 - What kind of cable AES/EBU or S/P-DIF cables should I use? How

long can I run them?

The best, quick answer is what cables you should NOT use!

Even though AES/EBU cables look like ordinary microphone cables,

and S/P-DIF cables look like ordinary RCA interconnects, they are

very different. Unlike microphone and audio frequency

interconnect cables, which are designed to handle signals in the

normal audio bandwidth (let's say that goes as high as 50 KHz or

more to be safe), the cables used for digital interconnects must

handle a much wider bandwidth. At 44.1 KHz, the digital protocols

are sending data at the rate of 2.8 million bits per second,

resulting in a bandwidth (because of the biphase encoding method)

of 5.6 MHz.

This is no longer audio, but falls in the realm of bandwidths

used by video. Now, considerations such as cable impedance and

termination become very important, factors that have little or no

effect below 50 kHz.

The interface requirements call for the use of 110 ohm balanced

cables for AES/EBU interconnects, and 75 ohm coaxial unbalanced

interconnects for S/P-DIF interconnects. The used of the proper

cable and the proper terminating connectors cannot be over

emphasized. I can personally testify (having, in fact, looked at

the interconnections between many different kinds of pro and

consumer digital equipment) that ordinary microphone or RCA audio

interconnects DO NOT WORK. It's not that the results sound subtly

different, it's that much of the time, it the receiving equipment

is simply unable to decode the resulting output, and simply shuts

down.

Fortunately, there is a ready solution for S/P-DIF cables. Any

store that sells high quality 75 ohm RCA video interconnect (or

"dubbing") connectors also sells high quality S/P-DIF

interconnects as well. They may not know it, but they do. This is

because the signal and band pass requirements for video and S/P-

DIF cables are the same. National chains such as Radio Shack sell

such cables, and the data seems to indicate that they are good

digital interconnects.

For AES/EBU, there are fewer, less common solutions. Companies

such as Canare make excellent cables. Professional audio

suppliers and distributors may be good sources for such cables.

If you are handy with a soldering iron, then you can purchase 110

ohm balanced shielded cable and make your own (which I have done

quite successfully). Cables such as Alpha Twinax, Carol Twin

Coaxial, Belden 9207 twin axial, and the like, all work well for

this application. Use high-quality XLR connectors (be warned that

these cables are 0.330 inches in diameter and are a VERY tight

fit in the neoprene strain relief's of many connectors: warming

them in hot water makes them pliable enough to work well).

As to how long these cables can be, it's hard to say. However, a

couple of general rules apply.

S/P-DIF was NEVER intended to be a long-haul hardware

interconnect. The relevant specifications talk of interconnect

lengths less than 10 meters (33 feet). In fact, many pieces of

equipment cannot tolerate cables even that long, due to the

excessive capacitance and possibly induced common mode

interference.

AES/EBU is more tolerant of longer runs because it is balanced

(thus more immune to interference) and it's run at a higher

signal level (5 volts instead of 0.5 volts). The standards "allow

signal transmission up to a few hundred meters in length."

The reality is that much is highly dependent upon the actual

conditions at hand. The requirements are that the received signal

fit within certain requirements of rise time/period and voltage

level, the so-called "eye diagram". In other words, regardless of

what kind of cable you use, if it can't move the voltage at the

receiver far enough soon enough, it simply isn't going to work.

Another complicating factor is that both protocols allow a degree

of multi-drop capability. This means a single transmitter can

drive several receivers (the last of which must be terminated

with the proper termination impedance). However, implementing

multi-drop puts more stringent requirements on impedance

matching. [Dick]

--

Q5.9 - What is SCMS?  How do I defeat it?

SCMS is the Serial Copy Management System, a form of copy

protection that was mandated by Federal law (the Home Recording

Rights Act). SCMS consists of a set of subcode flags that

indicate to a digital recorder whether or not the source may be

copied.  Under the HRRA, consumers are permitted to make one

digital generation, but no more. Thus when, for instance, the

consumer copies a CD onto DAT, the SCMS flag is set on the copy

and no further generations can be made.

SCMS is only mandated in consumer machines.  Any recorder sold

through professional channels, and which is intended for use in

professional applications, does not have to implement it.

There are several professional products, such as Digital Domain's

FCN-1 format converter <http://www.digido.com/>, which allow

manipulation of the SCMS flags. These units exist so that

professional engineers may adjust the subcode bits of the

recordings they produce.  [Gabe]

SCMS is now also found on some consumer grade CD burners. [Joe]

[9/98]

--

Q5.10 - What is PCM-F1 format?

In the 1980s, before the DAT era, Sony produced a set of PCM

adapters that enabled one to record digital audio using a video

cassette machine.  These units had RCA audio connections for

input and output, as well as video I/O that could be sent to, and

received from, the VCR.  At the time, these systems offered

performance far in excess of conventional analog recorders

available in the price category.

Sony released many models, including the PCM-F1, PCM-501, PCM-

601, and PCM-701.  Perhaps the most interesting is the PCM-601,

which has S/P-DIF digital I/O.  These units are highly prized

since they are the only units that can be used to make digital

transfers of F1 tapes to modern hardware.

There are some engineers who insist that, despite the clunkiness

of the format by modern DAT standards, the F1 series was the best

digital format ever developed.  To this day, it is not surprising

to see an F1 encoder on a classical recording session. [Gabe]

--

Q5.11 - How do digital recorders handle selective synchronization?

Selective Synchronization, or "Sel-Sync" as it is often called,

is the ability of a recorder to play and record simultaneously,

allowing synchronous recording of new material onto specific

tracks without erasing everything on tape.  This technique is

what makes overdubbing possible.

On an analog recorder, audio tracks are discrete entities, and

the sync head is really just a stack of individual heads, any one

of which is capable of recording or playing back.  Thus sel-sync

is a relatively simple matter of putting some heads into record

and others into repro.

In the digital world, the problem is highly complex.  First, A/D

and D/A conversion involves an acquisition delay of several

milliseconds. Second, and more importantly, digital tracks are

not discrete.  Rather, they are multiplexed together on a tape,

along with subcode and other non-audio information.  So how can

you replace one track and leave the others untouched?

The answer is a technique called "read before write" (RBW) or

"read, modify, write" (RMW) which involves a second set of heads.

The data is read from the tape and flushed into a buffer, where

it can be modified, and ultimately written back to the tape.

Thus when you "punch in" on a digital deck, you are physically

re-writing all the tracks, not just the one you're overdubbing.

You are not, however, changing the data on any track other than

the one you want to replace.  [Gabe]

--

Q5.12 - How can a 44.1 KHz sampling rate be enough to record all the

harmonics of music?  Doesn't that mean that we chop off all the

harmonics above 20 KHz?  Doesn't this affect the music?  After

all, analog systems don't filter out all the information above 20

kHz, do they?

This whole question is based on the premise that "analog systems

don't filter out all the information above 20 kHz." Indeed there

are mixers and power amplifiers and other electronic systems that

are capable of stunningly wide bandwidth, often exceeding 100

KHz, the same cannot be said for the entire analog reproduction

chain. The mechanical transducers, microphones, speaker and phono

cartridges seldom have real response far exceeding 20 KHz. In

fact, some of the most highly regarded large diaphragm condenser

microphones often used in very high quality recordings seldom

exceed 18 KHz bandwidth. Analog tape recorders rarely have

bandwidths as wide as 25 KHz, and LP reproduction systems have

similar limitations in reality.

So while it may be possible to send very high frequency

ultrasonic signals through parts of both analog and digital

reproduction chains, there are, in both technologies, fundamental

and insurmountable limits to the bandwidth that, in reality, lead

to very similar actual reproducible bandwidths in each.

Thus, one of the basic premises of the question is flawed. Analog

systems DO filter out information above 20 KHz. Further, the

frequency response and phase errors of even the very best well-

maintained analog reproduction systems have response errors far

exceeding those of even middle of the line digital equipment.

Whether one person may find those errors tolerable or even

likeable or not is a matter or personal preference that is beyond

the scope of this or any other technical discussion.

There are a variety of anecdotal tales that are advanced to

"prove" that the ear can hear far beyond what is conventionally

accepted as the 20 KHz upper limit. An upper limit that, for the

most part, applies to young people only: modern high SPL music

and noise levels has lead to a widespread deterioration in the

hearing of the adult population at large, and especially amongst

young males.

For example, there is an apocryphal story about Rupert Neve that

tells of a console channel that sounded particularly "bad". It

was later discovered that it was oscillating at some ultrasonic

frequency, like 48 KHz. Rupert Neve is rumored to have seized

upon this as "proof" that the ear can hear well beyond 20 KHz.

However, there exist an entire range of perfectly plausible

mechanisms that require NO ultrasonic acuity to detect such a

problem. For example, the existence of ANY non-linearity in the

system would result in the production of inter-modulation tones

that would fall well within the 20 KHz audio band and certainly

would make it sound awful. Even the problem that was causing the

oscillation itself could lead to massive artifacts at much lower

frequencies that would completely account for the alleged sound

of the mixer in the complete absence of a 48 KHz "whistle."

Whether 20 KHz is an adequate bandwidth is a debatable subject.

However, several important facts have to be remembered. First,

BOTH analog AND digital reproduction systems suffer from roughly

the same bandwidth limiting. Second, digital systems using

properly implemented oversampling techniques have far less severe

phase and frequency response errors within the audible band. No

analog storage and reproduction system can match the phase and

response linearity of a digital system, both at low and high

frequencies. Once those demonstrable facts are acknowledged, then

the discussion about supra-20 KHz aural delectability can

continue, knowing that, if it is demonstrated to be significant,

both systems are provably deficient. [Dick]

--

Q5.13 - Yeah, well what about square waves? I've seen square wave

tests

of digital systems that show a lot of ringing. Isn't that bad?

Square waves are a mathematically precisely defined signal. One

of the ways to describe a perfect square wave is as the sum an

infinite series of sine waves in a precise phase, harmonic and

amplitude relationship. The relation is:

1           1           1           1

F(t) = sin(wt) + -sin(3wt) + -sin(5wt) + -sin(7wt) + -sin(9wt) ...

3           5           7           9

Where t is time, w is "radian frequency", or 2 pi times

frequency.

Remember - we require an infinite number of terms to describe a

perfect square wave. If we limit the number of terms to, say, 10

terms, (such as the case with a 1 kHz square wave perfectly band

limited to 20 kHz), there simply aren't enough terms to describe

a perfect square wave. What will result is a square wave with the

highest harmonic imposed on top as "ringing." In fact, this

appearance indicates that the phase and frequency response is

perfect out to 20 kHz, and the bandwidth limiting is limiting the

number of terms in the series.

Well, what would a perfect analog system do with square waves? As

it turns out, if you take a high quality 15 IPS tape recorder,

bias and adjust it for the flattest possible frequency response

over the widest possible bandwidth, the result looks remarkably

like that of a good digital system for exactly the same reasons.

On the other hand, adjust the analog tape recorder for a square

wave response that has no ringing, but the fastest possible rise

time. Now listen to it: it sounds remarkably dull and muffled

compared to the input. Why? Because in order to achieve that

square wave response, it's necessary to severely roll off the

high-end response in order to suppress the high-frequency

components needed to achieve fastest rise time. [Dick]

--

Q5.14 - How can a 16-bit word length be enough to record all the

detail in music?  Doesn't that mean that the sound below -96 dB gets

lost in the noise? Since it is commonly understood that humans

can perceive audio that IS below the noise floor, aren't we

losing something in digital that we don't lose in analog?

You're correct in saying that human hearing is capable of

perceiving audio that is well below the noise floor (we won't say

what kind of noise floor just yet). The reason it can do this is

through a process the ear and brain employ called averaging.

If we look at a single sample in a digital system or an

instantaneous snapshot in an analog system, the resulting value

that we measure will consist of some part signal and some part

ambiguity. Regardless of the real value of the signal, the

presence of noise in the analog system or quantisation in the

digital system sets a limit on the accuracy to which we can

unambiguously know what the original signal value was. So on an

individual sample or instantaneous snapshot, there is no way that

either ear or measurement instrument can detect signals that are

buried below either the noise or the quantisation level (when

properly dithered).

However, if we look at (or listen to) much more than a single

sample, through the process of averaging, both instruments and

the ear are capable of detecting real signals below the noise

floor. Let's look at the simple case of a constant voltage that

is 1/10th the value of the noise floor. At the instantaneous or

sample point, the noise value overwhelms the signal completely.

But, as we collect more consecutive snapshots or samples, an

interesting thing begins to happen. The noise (or dither) is

random and its long-term average is, in fact, 0. But the signal

has a definite value, 1/10. Average the signal long enough, and

the average value due to the noise approaches 0, but the average

value of the signal remains constant at 1/10.

A somewhat analogous process happens with high frequency tones.

In this case the averaging effect is that of a narrow-band

filter. The spectrum of the noise (or simple dither) is

broadband, but the spectrum of the tone is very narrow band.

Place a filter centered on the tone and while we make the filter

narrower and narrower, the contribution of the noise gets less

and less, but the contribution of the signal remains the same.

Both the ear and measurement instruments are capable of averaging

and filtering, and together are capable of pulling real signals

from deep down within the noise, as long as the signals have one

of two properties. Either a period that is long compared to the

inherent sampling period of the signal in a digital system or

long compared to the reciprocal of the bandwidth in an analog

system, or a periodic signal that remains periodic for a

comparably long time.

Special measurement instrument were developed decades ago that

were capable of easily detecting real signals that were 60 dB

below the broadband noise floor. And these devices are equally

capable of detecting signals under similar conditions in properly

dithered digital systems as well.

How much the ear is capable of detecting is dependent upon many

conditions, such as the frequency and relative strength of the

tone, as well as individual factors such as aging, hearing damage

and the like.

But the same rules apply to both analog systems with noise and

digital systems with de-correlated quantisation noise. [Dick]

Q5.15 -  Q5.14 - What's all this about 20- and 24-bit digital audio?

Aren't CDs limited to 16 bits?

Yes, CDs are limited to 16 bits, but we can use >16-bit systems

to produce 16-bit CDs with higher quality than we could

otherwise.

We are able to record audio with effective 20-bit resolution

nowadays. The finest A/D converter systems have THD+N values

around -118 dB with linearity extending far below even that.

When it comes time to reduce our word-length to 16 bits, we can

use any one of a variety of noise shaping curves. The job of

which is to mix with our 24-bit audio, shift the dither spectrum

of the noise into areas where our ears are less sensitive, thus

enabling the noise component to comprise audio information at the

spectral areas where our ears are most sensitive.  See

Lipschitz's seminal papers for fuller detail on this subject.

Furthermore, we often perform DSP calculations on our audio, and

to that end it is worthwhile to carry out the arithmetic with as

much precision as we can in order to avoid rounding errors.  Most

digital mixers carry their math out to 24-bit precision at the

I/O, with significantly longer word lengths internally.  As a

result, two 16-bit signals mixed together can produce a valid 24-

bit output word.  For that matter, a 16-bit signal subjected to a

level change can produce a 24-bit output if desired (except, of

course, for a level change that is a multiple of 6 dB, as that's

just a shift left or right).

The number of noise shaping curves available today is staggering.

Sony SBM, Weiss, Meridian 618, Sonic TBM, Apogee UV-22, Prism

SNS, Lexicon, PONS, Waves, and, of course, the classic Lipschitz

curve are just a few of the multitudinous options that now exist.

[Gabe]

=====

Section VI - Digital editing and mastering

--

Q6.1 - What is a digital audio workstation?

A digital audio workstation (DAW) is one of our newest audio

buzzwords, and applies to nearly any computer system that is

meant to handle or process digital audio in some way.  For the

most part however, the term refers to computer-based nonlinear

editing systems.  These systems can comprise a $500 board that

gets thrown into a PC, or can refer to a $150,000 dedicated

digital mastering desk.  [Gabe]

--

Q6.2 - How is digital editing different from analog editing?

In the days of analog editing, one edited with a razor blade and

a diagonal splicing block.  Making a cut meant scrubbing the tape

over the head, marking it with a grease pencil, cutting, and then

taping the whole thing back together.  Analog editing

(particularly on music) was as much art as it was craft, and good

music editors were worth their weight in gold.

In many circles, analog editing has gone the way of the Edsel,

replaced by digital workstation editing.  For complex tasks, DAW-

based editing offers remarkable speed, the ability to tweak an

edit after you make it, a plethora of crossfade parameters that

can be optimized for the edit being made, and most importantly,

the ability to undo mistakes with a keystroke.  Nearly all

commercial releases are being edited digitally nowadays.  Since

satisfactory editing systems can be had for around $1,000, even

home recordists are catching onto the advantages.  More elaborate

systems can cost tens of thousands of dollars.

There are certain areas where analog editing still predominates,

however.  Radio is sometimes cited as an example, though this has

begun to change thanks to products like the Orban DSE 7000.  The

needs of radio production are often quite different from those of

music editors, and a number of products (the Orban being a fine

example) have sprung up to fill the niche.  Nonetheless, in spite

of the rapid growth of DAW's in the radio market, razor blades

are still found in daily use in radio stations.  [Gabe]

--

Q6.3 - What is mastering?

Mastering is a multifaceted term that is often misunderstood.

Back in the days of vinyl records, mastering involved the actual

cutting of the master that would be used for pressing.  This

often involved a variety of sonic adjustments so that the mixed

tape would ultimately be properly rendered on vinyl.

The age of the CD has changed the meaning of the term quite a

bit. There are now two elements often called mastering.  The

first is the eminently straightforward process of preparing a

master for pressing. As most mixdowns now occur on DAT, this

often involves the relatively simple tasks of generating the PQ

subcode necessary for CD replication. PQ subcode is the data

stream that contains information such as the number of tracks on

a disc, the location of the start points of each track, the clock

display information, and the like.  This information is created

during mastering and prepared as a PQ data burst which the

pressing plant uses to make the glass pressing master.

Mastering's more common meaning, however, is the art of making a

recording sound "commercial."  It is the last chance one has to

get the recording sounding the way it ought to.  Tasks often done

in mastering include:

? Adjustment of time between pieces

? Quality of fade-in/out

? Relation of levels between tracks (such that the listener

doesn't have to go swinging the volume control all over the

place)

? Program EQ to achieve a desired consistency

? Compression to make one's disc sound LOUDER than others on the

market

? The list goes on.

A good mastering engineer can often take a poorly-produced

recording and make it suitable for the market.  A bad one can

make a good recording sound terrible.  Some recordings are so

well produced, mixed, and edited that all they need is to be

given PQ subcode and sent right out.  Other recordings are made

by people on ego trips, who think they know everything about

recording, and who make recordings that are, technically

speaking, wretched trash.

Good mastering professionals are acquainted with many styles of

music, and know what it is that their clients hope to achieve.

They then use their tools either lightly or severely to

accomplish all the multiple steps involved in preparing a disc

for pressing. [Gabe]

--

Q6.4 - What is normalizing?

Normalizing means bringing a digital audio signal up in level

such that the highest peak in the recording is at full scale.  As

we saw in Q5.4, 0 dB represents the highest level that our

digital system can produce.  If our highest level is, for

instance, -6 dB, then the absolute signal level produced by the

player will be 6 dB lower than it could have been.  Normalizing

just maximizes the output so that the signal appears louder.

Contrary to many frequently held opinions, normalizing does NOT

improve the dynamic range of the recording in any way, since as

you bring up the signal, you also bring up the noise.  The

signal-to-noise ratio is a function of the original recording

level.  If you have a peak at -6 dB, that's 6 dB of dynamic range

you didn't use, and when you normalize it to 0 dB, your noise

floor will rise an equivalent amount.

Normalizing may help optimize the gain structure on playback,

however. Since the resultant signal will be hotter, you'll hear

less noise from your playback system.

But the most common reason for normalizing is to make one's

recordings sound, LOUDER, BRIGHTER, and have more PUNCH, since we

all know that louder recordings are better, right? :-)  [Gabe]

--

Q6.5 - I have a fully edited DAT that sounds just like I want it to

sound on the CD.  Is it okay to send it to the factory?

This is a highly case-specific question.  Some people truly have

the experience to produce DAT's on mixdown or editing that are

ready to go in every conceivable way.  Often these people can

send their tapes out for pressing without the added expense of a

mastering house.

However, if you do not have this sort of expertise, and if your

only reason for wanting to send it out immediately is because you

know that it is technically possible to press from what you have.

You would be advised to let a mastering engineer listen to and

work on your material.  A good mastering engineer will often turn

up problems and debatable issues that you didn't even know were

there.  Also, any decent mastering house will provide you with a

master on a format significantly more robust than DAT.

DAT is a fine reference tape format, but it simply is not the

sort of thing you want to be sending to a pressing plant. [Gabe]

--

Q6.6 - What is PCM-1630?  What is PMCD?

PCM-1630 is a modulation format designed to be recorded to 3/4"

videotape.  It was, for many years, the only way one could

deliver a digital program and the ancillary PQ information to the

factory for pressing.  The PCM-1630 format is still widely used

for CD production.

But PCM-1630 is now certainly an obsolete system, as there are

many new formats that are superior to it in every way.  One of

the most popular formats for pressing now is PMCD (Pre-Master

Compact Disc). This format, developed by Sonic Solutions, allows

for CD pressing masters to be written out to CD-R's that can be

sent to the factory directly.  These CD-R's contain a PQ burst

written into the lead-out of the discs.

Some plants have gone a step further and now accept regular CD-

R's, Exabyte tapes, or even DAT's for pressing.  The danger here

is that some users may think that they can prepare their own

masters without the slightest understanding of what the technical

specifications are. For instance, users preparing their own CD-Rs

must do so in one complete pass.  It is not permitted, for

instance, to stop the CD-R deck between songs, as this creates

unreadable frames that will cause the disc to be rejected at the

plant.  [Gabe]

--

Q6.7 - When preparing a tape for CD, how hot should the levels be?

Ideally you should record a digital master such that your highest

level is at 0 dB, if only to maximize the dynamic range of your

recordings.  Many people like their CDs to be loud, and thus they

will normalize anyway even if they don't hit 0 dB during

recording.

Some classical recordings are deliberately recorded with peaks

that are significantly lower than 0 dB.  This is done in order to

prevent quiet instruments such as lutes and harpsichords from

being played too loud.  If you record a quiet instrument such as

a harpsichord out to 0 dB, the listener would have to put the

volume control all the way at the bottom in order to get a

realistic level, and the inclination would be to play it at a

"normal" listening level.  By dropping the mastering level, the

listener is more likely to set the playback level appropriately.

--

Q6.8 - What is a CD-R? Why do I care about disk-at-once Vs track-at

once?

CDs were initially intended as a mass produced play-only media.

Recently, recordable CD's (CD-R) have become affordable. The cost

of adding a CD burner to a computer is now down to around $300,

so for small runs that may be an advisable route. [Joe] [9/98]

CDs are written or `burned' in the CD-R recorder in either one of

two modes, disk-at-once (DAO) and track-at-once (TAO). With disk-

at-once, the entire CD is written in one stream starting with the

table of contents. This is the preferred format for sending a CD-

R to a duplicating plant, because you can make a CD-R DAO master

disk of which the pressed CD will be a bit-for-bit clone. If you

want to record additional tracks at different times, this might

not be practical, so track-at-once operation is also offered.

This has problems too, because a short burst of digital errors

are generated at the place on the disk where the laser turned on

or off. Nearly all consumer CD players will play these CD-R's

(once they have been "finished" or the table of contents written)

but many CD pressing plants will refuse them because of the

errors. [David]

--

Q6.9 - Where can I get CDs manufactured?

A large number of CD manufactures can be found advertising in the

back pages of recording magazines. In addition, searching your

favorite web search engine should turn up a good number of

options. [Joe]

Be sure to know whether you are dealing with an actual

manufacturer or a broker. One isn't necessarily better than the

other (there are smalltime manufacturers that might not be as

careful about quality as the bigger ones, and brokers might be

able to get you a better deal with a bigger plant. But ask

questions and be sure you know what happens if the pressed CD

doesn't sound right. [David]

--

Q6.10 - How are CD error rates measured, and what do they mean?

[Forthcoming.  -Gabe]

=====

Section VII - Market survey.  What are my options if I want --

--

Q7.1 - A portable DAT machine or a rack sized DAT machine [9/98]

A fairly comprehensive survey of DAT machines has been conducted

by the DAT-Heads mailing list. Consult the homepage at:

http://www.eklektix.com/dat-heads. [Joe]

--

Q7.2 - A good but inexpensive compressor [9/98]

Two units that have received favorable notice on r.a.p. recently

are the RNC - the Really Nice Compressor, it is available from

FMR audio - http://www.fmraudio.com. For over $1000, the "Swiss

army knife" award goes to the Empirical Labs EL-8 Distressor -

http://www.wavedistribution.com. [Joe]

--

Q7.3 - An inexpensive stereo microphone [9/98]

This is a really difficult question, as it implies a universally

accepted single-point stereo technique, and there isn't one. Sony

http://www.sel.sony.com/ has a $300 unit, Shure's VP-88

http://www.shure.com/ is about three times as much and a bit

better. [David, whose company doesn't make one.]

--

Q7.4 - An inexpensive pair of microphones for stereo [9/98]

While large-diaphragm condenser mics are sometimes used for

stereo work, small-diaphragm condensers are much more common due

to their superior off-axis response and much lower cost.  For

that reason, only small-diaphragm condensers are discussed here.

Here's a good assortment of options at various price levels.

(Pricing is approximate!)  We strongly encourage buying matched

pairs or sequential serial numbers when available, particularly

on the less-expensive models.  Warning - most of these

microphones require phantom power.

Under $300 per pair.  (Frankly, it's worth spending more than

this if you possibly can.)

Audio-Technica Pro-37R. Not the flattest thing on earth, but

cheap and does the job. Requires phantom power.

Shure 849. Pretty much the same mic as the Shure Beta Green 4.0,

for less money. Battery or phantom.

Under $400 per pair.  (These mics are still a compromise, but

good for non-serious use.)

Audio-Technica ATM 33a. Resistant to overload and reasonably

quiet.  Battery or phantom.

Octava MK-012. Multiple capsules!  High mic-to-mic variability:

listen to the ones you're buying. Phantom.

Shure SM-94. Probably a lower-spec version of the SM-81.  Battery

or phantom.

Under $600 per pair.  (If you are taking other people's money,

this is the least you should pay.)

AKG "Blue Line" (391, etc.) Versatile microphone family, many

interchangeable capsule options.  Less top end than some.

Audio-Technica 4041. A clear-sounding, if somewhat bright mic,

available in matched pairs.  A fine choice.

Crown CM-700. A nice sounding mic at an excellent price.  Rather

low output: use it for close micing.

Shure SM-81. Common studio utility condenser.  Not overly bright,

reasonably quiet, very popular.

Under $1400 per pair.  (Sometimes not much under!)

AKG C460. Bright-sounding, and impervious to overload.

Tremendous on brass sections. Placement-sensitive when close-

micing.  Alternative-pattern capsules available.

Microtech-Gefell M300. Much more bottom than the Neumann Km-84

and KM-184.

Neumann KM-184. Slightly brighter-sounding successor to the much-

loved KM-84.  Good choir mic. Frequent close-micing choice, due

to good pattern control.

Sennheiser K4 family. A versatile series with interchangeable

pieces.  More common in broadcast use.

More than $2000 per pair. The sky's the limit.  Listen and

compare carefully before choosing.)

Danish Pro Audio 4000 family and other models. Very accurate

omnidirectional mics, requiring special techniques.  A few

cardioids, too.

Josephson Series Six family. Optimized for transient and phase

response.  Built to order.

Neumann KM-100 family. Generally considered dark-sounding, can be

quite lovely.

Schoeps Colette family. Longtime favorites for location

recording.  Many capsules and accessories.

Sennheiser  MKH family. The quietest small diaphragm condensers

you can buy. Low distortion, too.

Recording the Grateful Dead?

While you were comatose, Jerry Garcia died.  The Dead are no

longer touring. Get a life. If you want to record some other

band, maybe you don't need microphones. If there isn't a multi-

output distribution box behind the console, the band probably

doesn't want you making tapes. [David R]

--

Q7.5 - A good microphone for recording vocals [9/98]

This is really an unanswerable question. Go to a local studio

with a good stock of mics. Rent a few hours of studio time. Ask

them to record your voice using the following mics through a pop

filter. Sing about one minute of the same song, but don't

identify which mic your using on the track. Have the engineer

keep track of each mic and which track it is. Mics to consider:

Rode NT1

AT 4050

AKG414

AKG C3000

Shure SM-7

Sennheiser 421

Neumann TLM-103

Any Beyer ribbon mic

Coles 4038

Shure SM-57

Any others they have in the below $1,000 price range.

(If they have a U87, or other expensive mics, get recordings of

those too, as a reference.)

Have a CD-R made. Go home, hide the track sheet, and listen to

all the tracks for a week. Pick the track you thinks sounds best

have someone write down the track number, then put the CD-R away

for a week. Listen to it again and see if you still like the same

track. If you choose a different track, put the CD-R away for a

week.

After a week, listen to it again and see if you still like the

same track. When you pick the same track twice in a row, that's

the one for you. Get the track sheet and find out which mic it

is. No cheating, and you can have other people choose as well.

[Harvey]

If you're buying the mic for a specific vocalist, the best idea

is to audition everything in your price range on the voice in

question.  A microphone that works poorly for one singer may be

perfect for the next. Generally speaking, the more broadly

applicable microphones cost more, so you may be able to save some

money if it only needs to work for you.  Some entry-level

products are the AKG C3000, Audio Technica 4033,  Beyer MC834,

Equitek E-200, Electrovoice RE-1000, Langevin CR-3A and Rode NT-

2.  A number of these mics can be had new for under $500, and

even less if you find a used one.  That's money well spent if

they do your job.

If you're taking in outside work, consider the following:

AKG 414B-ULS.  The 414 is a studio stalwart, good on voices and

instruments alike.  You can use one of these on a rock vocal one

day, an acoustic guitar the next, and pair of them for a

classical concert the following night. It's fairly present

sounding, but still reasonably flat.  The combination of four

patterns, two bass roll-off's, and two pad options makes it

highly adaptable; you can expect to have one or two in the mic

locker for your entire career.  For a bit more money, there's

also the 414TL-II, but this has a tailored presence rise, which

makes it impressive on (some) vocals, at the expense of being

less broadly applicable for other purposes.  Used 414's can often

be found at prices in the $650-750 range; the current list price

is $995.

Neumann TLM-193.  First things first: this mic doesn't sound like

a U87. If that's the sound you want, find a sympathetic banker,

or a rich uncle. (Or audition the Audio-Technica 4033, which

sounds closer than you might expect.)  Judged on its own merits,

the TLM-193 sounds slightly laid back, but very, very natural,

even off-axis.  When it's important to get something good down on

tape without a lot of fuss, the 193 is a fine tool. There are no

switchable patterns, pads, or bass roll-off to mess with, you

just put it in a good spot and roll tape.  Plus, you can use it

on a half dozen different tracks on the same song without

creating terrible problems at mixdown.  The optional shock mount

is worth having since there's no bass roll-off switch.  These are

still a bit rare on the used market, and they cost $750-850 when

you find one.  The list price is about $1300.  [I'm unsure of

this price. -- DR]

Neumann has released the TLM-103, which is also single pattern

microphone like the TLM-193. This microphone utilizes a capsule

derived from half of a U87 capsule and comes pretty close to

sounding like a U87, some people even like them better. These

retail for under $1000. [Mark, David]

Audio-Technica 4050.  If the two mics above are out of the

question, this one is worth a listen.  For substantially less

money, it's got three patterns, pad and roll-off switches, and a

nifty shock mount.  An Audio-Technica doesn't have the same rate-

card appeal as a Neumann or AKG, but your average struggling

garage band is going to think it's pretty darn cool, and they'll

be right.  The main things to watch out for are off-axis

coloration and excessive sibilance on some singers.  Place it

carefully, and you can get good results.  These cost $600-700

new, less used.

Q7.6 - A good microphone for recording [insert instrument here] [9/98]

Miking a drum kit?

[Someone check this!  This is not my area of expertise! -- DR]

If you need to economize when miking drums, the best way is to

use fewer microphones.  With careful placement, you can often get

by with four: snare, kick, and a pair of overheads.    The

canonical mic for snare drum is the Shure SM-57.  They're cheap,

which is good, because drummers are always breaking them.  For

overheads, a pair of condenser microphones is almost compulsory.

See the discussion above for ideas.

That leaves the kick drum mic. What you choose for kick drum

depends on whether you can afford to dedicate a mic to that use,

or whether your choice must serve other purposes as well. If

miking drum kits is a small part of your studio routine, it may

make sense to buy a more broadly applicable microphone, even if

it costs a bit more.  One excellent choice would be a Sennheiser

MD-421, which can also be used for voice-overs, female singers,

miking guitar cabinets, and much more.  For a bit more money, an

Electrovoice RE-20 is another good choice with similar

applications.  Both of these microphones require more

experimentation with placement than a "single-use" kick mic, but

the pay back is a broader tonal palette.  If you own a large-

diaphragm condenser mic, you might be interested to know that

these sometimes work great on kick.  Use caution: begin by

placing it well outside the drum shell, and move it only as close

as you need to in order to get the desired sound. You'll probably

need to pad the output.

If a dedicated kick drum mic makes sense, one very common choice

is the AKG D-112.  These cost $200-250 and work with a minimum of

fuss.  If that's too much money, the Audio-Technica ATM-25 and

Pro-25 are serviceable; both use the same diaphragm, but the Pro-

25 has a less-rugged case. [David R]

The following is Mr. Boisen's opinions on his top 10 mics as

always your milage may vary YMMV. [Mark]

Boisen's Top 10 Mic List

It seems like I might save some on-line response time, and

address a lot of common questions by sharing this list of my

favorite mics. You will notice that the "legendary" Neumann and

AKG tube mics are not included here, nor is there anything

particularly esoteric. This is not a comment on anything other

than the fact that I haven't worked up the courage to pay more

than $2000 for a single piece of gear (yet). I like the most

expensive toys, I like to read about them and I like to hear them

used well on record. But I LOVE great-sounding gear that I can

actually afford to own. I assume a lot of you do as well - hope

you enjoy this.

PLEASE NOTE: 1) I don't sell mics, audio gear, and I don't

endorse or work for any microphone companies. I am a studio

owner/ engineer and freelance reviewer for Electronic Musician

magazine. 2) These mics are generally used with class A and/or

tube pre-amps and recorded on analog tape - your results may vary

dependant upon equipment and personal taste. 3) Prices are based

on what I see on the Internet used, and at Bay Area stores new

over the last couple of years.

AKG C 414 B-ULS (multi-pattern large diaphragm condenser). The

smooth response, extended highs and lows, and versatile features

of this mic, combined with its longevity and accessible price,

have made it a universal favorite. Sometimes it is too bright and

scratchy around 6-8 kHz, and after countless struggles with it on

horns I finally had the sense to give up and buy a U 87. But when

you hear a good one, the airiness and transient response is

outstanding. These days I rely on the 414 almost exclusively for

bass amps, and will regularly pull it out for jazz kick (in

Omni), percussion, and congas. Truly a classic and a great buy.

$500-$700 used, $900 new.

AKG C 3000 (multi-pattern large diaphragm condenser) This is my

nominee for best large diaphragm condenser under $300. I have

used them extensively for live-to-DAT stereo recording, as well

as in the studio for overheads on bright/loud kits, vocals,

acoustic guitar, strings, et al. Many people have bought pairs on

my recommendation, and all of them are quite satisfied. The 414B-

ULS offers more features and superior electronics, while the

C3000 actually has better internal shock mounting and seems to be

less fizzy. Other than a little extra "edge" around 3 KHz I have

never had any complaints regarding this mic. $275-300 new.

Coles 4038 (ribbon). I am a big fan of ribbon mics, and this one

is my favorite for all-around use. A trombonist friend talked me

into buying one after Steve Albini used it on his horn. It's

unbeatable for low brass, cello and violin, acoustic bass (unless

you have an RCA 44), and drum room ambience, with tons of

transparent lows and an ultra-smooth upper midrange. I've never

tried it as an overhead mic, but have gotten uniformly pleasing

results on bass/ guitar/ organ amps, all kinds of saxes, and even

some vocals. May not be bright enough for some folks, or will

take some getting used to if you've been raised on ADAT's and

crispy condensers. $1000 new.

Electro-Voice RE 20 (large diaphragm dynamic). Comparison tests

in my recording classes revealed this "workhorse" mic to be very

substantial and warm on electric guitar, with just the right edge

and presence for jazz and blues styles. It also excels on

voiceovers, jazz kick drum, works great for running into a guitar

amp for crunchy alt. vocals, and actually produces a pleasing,

musical tone on horns for live sound work. The large diaphragm,

open pickup pattern and unusual porting make this much more than

just your average dynamic. $200-400 used, $500 new.

Lawson L 47 MP (multi-pattern large diaphragm tube condenser). My

"first choice" mic for any vocal, any reed instrument, any

electric guitar or organ overdub, and any source that would

benefit from the thick low mids, creamy highs, and richness that

only a tube mic can deliver. The continuously-variable pattern

control is fun to use, and gives amazingly varied results (which

may delight or confound you). And vocalists always seem to give

110% when you put this "golden goliath" in front of them. I've

already raved too much about this mic online; see the Feb. 98

issue of Electronic Musician for additional plusses and minuses.

$2000 new, direct from http://www.lawsonmicrophones.com.

Neumann U 87 (multi-pattern large diaphragm condenser). If I had

to pick one mic to use for the rest of my life, this would be it.

The U 87 sounds good on everything, great on most things, and

always delivers a "big" sound with generous low end, airiness,

minimal off-axis coloration, and realistic ambience. Mine are

late 70's, very smooth and full in the mids, and my only negative

comments are that they can sometimes make a vocal too warm or

muddy around 300 Hz, or get a little scratchy due to the slight

presence boost above 5 kHz. $1500-$1800 for the mic used, $2700-

3000 for a complete system new.

Neumann TLM 193 (cardioid large diaphragm condenser). This mic

uses the same smaller diameter capsule as the U 89/ TLM 170, and

offers a more neutral sound than the U 87 - it is often dark

and/or not "zippy" enough in the highs for some people. I find

its response at a distance of 2-3 feet to be very flattering, and

like the transparent lows and unique quality it gives on

potentially harsh or scratchy sources. The 193 has given me

excellent results on vocals, sax/ horn sections (including the

edgy Rova Sax Quartet), trumpet, scratchy "ethnic" string

instruments, resonator guitar and acoustic harmonica. $1400 new.

Oktava MC 012 (small diaphragm condenser, interchangeable

capsules). A great "bargain" mic; the 012 (cardioid capsule)

became my overhead mic of choice after a comparison with the AKG

451. It's also excellent on acoustic guitar, and has come in very

handy for soft-spoken vocalists who can't cut through on a U 87

or Lawson. Has all the highs and snappy transient response that

small diaphragms are known for, plus full warm lows; be warned

that prominent mids around 800 Hz can be a problem when miking

cymbals (cardioid capsule). Oktava quality control ranges from

awful to non-existent, so good luck finding a matched pair. $250

new.

Sennheiser 441 (dynamic). Really a super dynamic mic, with a

sweet tonal response that rivals many condensers. It can be a

little too bright at times, with an abundance of 5-6 kHz

presence, but is never harsh and has worked particularly well for

me on "live" session vocals, guitars (acoustic and electric), and

brass (wonderful on tuba!). My favorite application has been with

highly produced "in-your-face" male vocals; the ample proximity

effect, linked with crisp highs and a midrange dip, delivers a

huge, imposing sound when worked close, and the mic is especially

resistant to popping. Lots of onboard eq options too! $300-400

used, $600-700 new.

Shure Beta 58A (dynamic). A lot of vocalists love this mic, and I

used to employ it a lot, although it's somewhat neglected around

my studio these days. I will still pull it out for snare drums,

guitar amps, "live" session vocals (i.e. a blues session,

typically loud, with everyone in the same room), and it's very

good on trumpet when the Coles or Lawson is too sensitive or

dull-sounding. Basically the Beta 58A can fill in for a

Sennheiser 441, offering a more pronounced midrange, and less

"fizz" on high-presence sources. It also makes a nice change from

the SM 57 on guitar amps, as does the earlier Beta 58 model. $75-

$100 used, $175 new.

HONORABLE MENTION/ FAQ's and IFAQ's (infrequently asked

questions)

Neumann TLM 103. An outstanding mic for the money, but I'm

getting bugged by the flimsy plastic stand mount and the lack of

a -10dB pad. FAQ: Does it sound as good as a U 87? Not to my

ears... See Electronic Musician Sept. '98 issue for more

comments.

Oktava MK 219. Another very useful large diaphragm mic in the

under $300 range. The unique "vintage" frequency response is

noticeably rolled-off below 150 Hz and above 12 kHz, packing lots

of hard-hitting midrange on vocals, snare, and electric guitar

tracks. IFAQ: I've heard that Oktava "exported" their mic designs

from prominent German manufacturers after WW II. Any truth to

this rumor?

[While some of the Lomo mic capsules look like they are copies of

various old Neumann's, the Oktava 219/319 capsule is entirely

different - David]

Sennheiser MD 504/ 604. Superb tom mics; small, beefy, affordable

and kind of cute. Good for guitars and low brass also. IFAQ: Why

has Sennheiser/ Neumann started making microphones out of

plastic? Has the world gone mad?

Any RCA ribbon mic/ Beyer 160/ Altec 639b. Because ribbon mics

rule!

FAQ: What is the best microphone out there? Someone on R.A.P.

nailed this question by responding "What kind of ice cream is the

best?" Some people like vanilla, I usually choose Cherry Garcia,

but tomorrow I may be in the mood for chocolate - the best is

what works best for your needs, budget and ears. Happy listening!

Myles Boisen (mylesboise@aol.com)

--

Q7.7 - A small mixer [9/98]

The Mackie 1202 (available in the basic 1202 or 1202vlz

variations) is commonly cited by readers of RAP as a great small

mixer. For a larger format, the Allen & Heath MixWizard series

has been recommended by many people. [Joe]

--

Q7.8 - A portable cassette machine [9/98]

About the only company seriously still in the portable cassette

recorder business is Sony. The TCD-D5M seems to still be in

production, as the is WM-D6. See http://www.sony.sel.com/ for the

latest. [David]

--

Q7.9 - A computer sound card for my IBM PC or Mac [9/98]

This answer will become obsolete very quickly. For years, sound

cards were considered as an add-on for games and funny noises,

and the audio quality was about what would be expected (despite

the phenomenal and unattainable specs claimed by the makers).

There is a web site of info on conventional sound cards at

http://www.larrysworld.com/soundcard.htm. However, nearly all of

these are designed for non-critical applications. If you want to

do digital sound editing on your PC, you really want to transfer

in and out of the computer digitally, without having to convert

back and forth to analog. New entries in this product category

are coming all the time, with expansion to cover 24 bit samples

nd/or 96 kHz sampling rates. The RME cards, made in Germany and

sold in the US by Sek'd, the Sonorus cards and the Zefiro ZA-2

are all suitable. [David]

--

Q7.10 - An eight-track digital recorder?

=====

Section VIII - Sound reinforcement

--

Q8.1 - We have a fine church choir, but the congregation can't hear

them. How do we mic the choir?

--

Q8.2 - How do I 'ring out' a system? [9/98]

Ringing out a 'system' simply means creating an equal opportunity

environment for frequency. Hence this environment is where

feedback at any given frequency is no more likely to feedback

than any other frequency, (i.e. flat).  In a 'rung out' system

the response of the system should be flat verses the response of

the microphone that is primarily driving the system with

consideration to the various problems that a room can contribute.

This is primarily true in monitors where often the mix is

centered on a particular microphone and less true at front of

house where you don't want to base your mix off the response of a

SM58.  Front of house will want to be truly 'flat' to be rung

out. Any microphone characteristics, which are undesirable (like

many common live mics), can be dealt with on the channel EQ's.

When 'ringing' out a system, whether front of house or a monitor

mix, the best place to start is with your voice.  Using a

microphone which you are used to the sound of, listen to your

voice over the system and using the equalization available, use

at least a third octave, remove any obvious frequencies that

sound out of balance.  A good way to do this, especially if you

are not use to identifying frequencies, is to 'bump' up each band

and get a feel for what it sounds like and then make the decision

whether or not it is in balance with the rest. When adjusting the

EQ be careful not to remove too much of any frequencies. Abuse of

the EQ can cause many more problems than it solves regarding

feedback, as well as create problems with the overall balance and

gain structure.  In general if the speakers and system in general

are of high quality and the room is not too awkward you probably

wont be pulling any more than 4 or 5 db out of 6 or so bands.  If

you notice your pulling out more than 10 bands with 6+ db, it

might be best to flatten and start over.

Many times a RTA can come in handy for doing this.  Often after

starting off with the technique above I will use a RTA to check

for any problem areas.  Sometimes just your voice alone won't

make you aware of certain acoustical problems of a room.  Also

keep in mind that the response of a system, and a room, is going

to change with the sound pressure level.  When your listening to

the system turn it up a bit, get closer to the volume that the

show will be performed at.  Spending time adjusting the system

prior to show time can reduce many headaches during the show.

One way I often judge whether the system is 'rung out' is to turn

it up with a live microphone hooked up a bit past the typical

maximum level. When the system just starts to feedback listen to

the frequencies that ring.  If it is apparent that there is more

than one frequency feeding back at the same time then you at a

good point.  This indicates that there is not a single frequency

in the bandwidth which will feedback before any others, this is

the goal of ringing out the system.  The more full bandwidth the

ring the less likely that a particular frequency will feedback

later on.

Getting use to doing this is one of the keys to being good at it.

Try and keep in mind what various types of speakers sound like as

you encounter them and what areas of the bandwidth they are

problematic in. This will help you when time is tight. [Brian]

--

Q8.3 - How much power to I need for [insert venue here]?

--

Q8.4 - How good is the Sabine feedback eliminator?

=====

Section IX - Sound restoration

--

Q9.1 - How can I play old 78s?

First rule of thumb:  DO NOT PLAY THEM WITH AN LP STYLUS!

The grooves on 78s are gigantic compared to the microgroove LPs.

As a result, specialized styli are needed for proper playback of

78s. Also, the RIAA equalization curve normally used for LPs has

no relation to the frequencies that were equalized on 78

recordings.

The easiest stylus to obtain is the Shure V15 with the 78 pickup.

This is a good, though not great, 78 stylus and will do an okay

job at playing most discs.

The serious 78 collector will want to obtain not only a

collection of 78 styli for the various discs in their collection

(groove sizes varied, and most serious collectors own a handful

of styli), but also a pre-amp with variable equalization curves.

One supplier of all this apparatus is Audio-78 Archival Supplies

at (415) 457-7878.  [Gabe]

--

Q9.2 - How can I play Edison cylinders?

Edison cylinders are best played back with Audio 78's adapter.

You remove the horn and reproducer element from your cylinder

machine, install their electric reproducer, and connect it up to

a phono pre-amp, sans RIAA. [Gabe]

--

Q9.3 - What are "Hill and Dale" recordings, and how do I play them

back?

Hill & Dale recordings are discs where the grooves move

vertically instead of horizontally.  Edison, for instance, cut

his discs this way.  In order to play Edison discs, one needs a

special glass ball stylus that is 3.7-4.0 mil wide.  This is

available from Audio 78, among other places.

Also, since the information is vertical instead of horizontal,

one must rewire a stereo phono cartridge to reject the normal

horizontal information and reproduce only the normally discarded

vertical information.  This is easily accomplished by wiring a

stereo cartridge in mono, summing the channels, with one channel

out of phase.  In other words, connect the cartridge as follows:

______________

|         ____  to pre-amp

+ L -     + R -    |

|   |         |    |

|   |_________|    |

|__________________

One caveat: if your cartridge has one lug shorted to ground, make

sure that this lug is connected to the ground on your

preamplifier. It doesn't actually matter which channel you

invert.

Some pre-amps like the FM Acoustics 222 or the OWL 1 have a

switch that will do this for you without rewiring.  [Gabe]

--

Q9.4 - What exactly are NoNOISE and CEDAR?  How are they used?

NoNOISE and CEDAR are systems for noise removal.  Both of them

approach the same sorts of noise, but use different algorithms

and have different user interfaces, often with differing

effectiveness.

Noise can be broken down into several categories:

IMPULSIVE NOISE: Pops, clicks, thumps, snaps.

CRACKLE:  The low-level "bacon frying" effect heard on

LP's and 78's.

HISS:     Tape hiss, surface noise, amplifier hiss,

broadband noise

BUZZ:     60 Hz hum, any other steady-state noise that

is

relatively narrow-band.

NoNOISE and CEDAR are two (expensive) techniques for removing

many of these ailments.  It is rare that it is possible to remove

all of the problem, nor is it ever possible to remove it with no

degradation of the program material.  [Gabe]

--

Q9.5 - How do noise suppression systems like NoNOISE and CEDAR work?

Digital techniques have been applied to many facets of sound

processing and recording and have, on the whole, been found to

give results far superior to their analogue counterparts. Nowhere

is this more true than in the field of audio restoration, where

excellent processes have been developed for removal of impulsive

noise (thumps, clicks and ticks) and attenuation of continuous

broadband noise (such as tape hiss). Example techniques for these

two are outlined below.

IMPULSIVE NOISE

In this category we include many types of disturbance, from the

click generated by a scratch on a 78rpm disc, to the tiny tick

created by a single corrupt bit in a digital data stream. Also

included is crackly surface noise from 78's (that sounds like a

frying pan), though this requires somewhat different treatment;

however the outline presented below is fairly similar for both

processes. Typically, audible clicks are of a few microseconds to

a few milliseconds in duration, and their density can be up to a

few thousand clicks per second on poor-quality material.

First the audio is split into short blocks of maybe 10ms

duration. A model is fitted to each block; this model can be

thought of as a description of the signal in simple mathematical

terms. The model is chosen such that musical data is a good fit,

but the impulsive noise is a poor fit.

For example, a simple model could be a sum of sinewaves, whose

number, frequencies, amplitudes and phases are the model

parameters. The parameters are calculated such that when these

sinewaves are added together they match the musical parts of the

signal accurately, but match the impulsive noise badly.

Now the model can be thought of as a prediction of the music. In

undamaged sections the prediction is close (since music is known

to consist of a sum of sinewaves, at least approximately); during

clicks and pops etc. the prediction is poor, because the model

has been designed to match the music, and not the noise.

Now we can achieve impulsive noise removal by replacing the data

that fits the model badly (i.e. the clicks) with data predicted

by the model, which is known to be a close approximation to the

music.

BROADBAND NOISE

Broadband noise is usually better tackled in the frequency

domain. What this entails is taking a block of data (as in the

impulsive noise case) but then calculating its spectrum. From the

spectrum an estimate can be made of which frequencies contain

mostly signal, which contain mostly noise.

To help in making this discrimination we first take a

"fingerprint" of the noise from an unrecorded section, such as

the lead-in groove of a record, or a silence between movements of

a symphony. This spectrum of this fingerprint is then compared

with the spectrum of each block of musical data in order to

decide what is noise and what is music.

The de-noising process itself can be thought of as an

automatically controlled, cut-only graphic equalizer. For each

block, the algorithm adjusts the attenuation of each frequency

band so as to let the music through, but not the noise. If the

SNR in a particular band is high (i.e. lots of signal, little

noise) then the gain is left close to unity. If the SNR is poor

in a given band, then that band is heavily attenuated.  [Chris]

--

Q9.6 - What is forensic audio?

Forensic audio is audio services for legal applications.

Forensics breaks down into four main categories.

TAPE ENHANCEMENT: Digital and analog processing to restore verbal

clarity and make tapes easier to understand in a courtroom

situation.

AUTHENTICITY: Electronic and physical microscopic examination of

a tape to prove that it has not been tampered with, altered, or

otherwise changed from its original state.  Another common

authenticity challenge is to determine whether a given was tape

was indeed made on a given machine.

VOICE IDENTIFICATION: Voice ID, or voiceprinting, is the science

that attempts to determine what was said, and by whom.  A variety

of analog and digital analysis processes are used to analyze the

frequency and amplitude characteristics of a human voice and

compare it against known samples.

[More to come on this]  [Gabe]

=====

Section X - Recording technique, Speakers, Acoustics, Sound

--

Q10.1 - What are the various stereo microphone techniques? [9/98]

There are basically three classes of stereo microphone

techniques:

1. Coincident pair (AKA XY)

2. Spaced pair (AKA AB)

3. Near-coincident pair

The technique known as "binaural" is sometimes treated as a

fourth category; in practice, it is strictly related to the

concept behind the near-coincident pair.

The basic ideas behind these three techniques are as follows:

COINCIDENT PAIR

Relies on two identical directional microphones (e.g. cardioid),

mounted as close as possible to each other, angled apart. The

stereo image is produced by intensity difference of the signals

arriving at the two microphones. The stereo spread is wider for

larger angles and for narrower polar patterns.

SPACED PAIR

Relies on two identical microphones, spaced by several feet and

pointing towards the sound source. The most popular method uses

Omni's (spaced Omni's), but, in principle, any polar pattern can

be used. The stereo image is produced by time difference of the

signals arriving at the two microphones. The stereo spread is

wider for larger spacing.

NEAR-COINCIDENT PAIR

Relies on two identical directional microphones angled apart,

with their capsules spaced usually by a few inches. The stereo

image is produced both by intensity and time difference of the

signals arriving at the two microphones. Larger angles and

greater spacing enhance the stereo spread.

BINAURAL (AND RELATED TECHNIQUES)

The baffled Omni's / dummy head technique is based on the idea

that our ears behave as omnidirectional pickups, but their polar

pattern is significantly altered by the head, which acts as a

mechanical equalizer (it filters mid and high frequencies in a

rather complicated way). So, by using two Omni microphones with a

baffle, or even a model head between them, the whole system tends

to behave as our hearing system. It actually happens, and these

techniques give extremely good results on headphones, if they're

used correctly. The result is less satisfactory on loudspeakers,

and sometimes a circuit called "Crosstalk Canceller", which

combines delay and equalization of the L and R signals, is used

to give a proper imaging on loudspeakers as well.

Two more important points:

It is well known that, in general, the spaced pair technique

tends to exaggerate stereo-spread and produce the effect known as

"hole in the middle": the larger the separation between

microphones, the more evident the hole. In order to minimize this

effect, a third mic is often placed between the two in order to

make the central image more solid. An array of three spaced

Omni's (called "Decca tree") is possibly the most famous

arrangement of its kind, and has been extensively used by Decca

for classical music recordings (albeit not exclusively).

The mid-side (MS) technique is actually a special case of the

coincident pair technique. It uses two microphones: one with a

figure-of-eight polar pattern, facing sideways (S signal), and

another microphone of any polar pattern, pointing at the center

of the sound source (M signal). The two signals can be easily

matrixed into L and R signals. More details about this technique

in a special section. [Marco]

Two excellent online sources of mic placement information are

Geoff Martin's pages <www.music.mcgill.ca/~martin/audio.html> and

Frank Lockwood's <www.calcna.ab.ca/~lockwood>

[David]

--

Q10.2 - How do I know which technique to use in a given circumstance?

[9/98]

This largely depends on at least two factors: the kind of sound

you're after and the acoustic behavior of the room or hall you're

recording in. I personally believe that recording an orchestra

or, anyway, an extended set of acoustic instruments still be one

of the most challenging, rewarding and at times bewildering

experiences that an engineer may have. It may be very frustrating

to even come close to what is, by your standards and judgement,

the ideal sound; also because there are no strict rules of thumb.

Anyway, keep in mind a few basic guidelines - and then

experiment!

1. Absolutely basic hint: move them more, equalize them less.

That is - if the tonal balance is wrong, try displacing the

microphones or changing technique, before resorting to EQ. A

flat recording with a proper choice of microphones and

positioning will invariably sound better than a "wrong"

recording, tentatively cured by means of equalization.

2. In most cases, you will get more satisfactory results by using

a well-thought out and well-positioned stereo pair with a

minimum number of spot microphones, rather than putting as

many as 20 of the latter in a formidable attempt to mix them

so to recreate the original sound. The modern digital formats

are very unforgiving when it comes to phase distortions and

weird coloration's of the sound arising from too many

microphones interacting. A band director I worked with was

very puzzled when I told him I had the idea of recording their

album with no more than eight mics (I actually used six),

because their previous work had involved no less than twenty-

six spot microphones. When I listened to it, it sounded awful,

to me - the snare being much more prominent than the whole

trumpet section... so, if it's not necessary, don't do it!

3. Omni's tend to be significantly better behaved than

directional microphones, as far as frequency response goes. In

that sense, they are usually more suitable as ambience

microphones, unless you have to work in an environment which

is acoustically bad (e.g. resonance's, etc.). One problem that

may arise with Omni's is that they tend to pick up a lot of

ambience from the back. This may make it necessary to move

them closer to the source in order to obtain a proper balance

between direct sound and reverberant field - which is not

always ideal when one is recording an ensemble. Directional

microphones are less sensitive to the ambience, and can

therefore be placed farther from the ensemble while retaining

a correct balance.

4. Some people are very fond of the spaced Omni's technique. The

main problem is that the sound may become phase shifted,

especially at high frequencies, and this technique is not

mono-compatible. Also, it is sometimes difficult to fit the

almost universally used third center microphone in the picture

(see Q10.1). The really good side of this approach is that you

get a very broad and yet clear stereo image, and, because

Omni's are used, possibly the most faithful tonal balance. A

spaced pair technique worth mentioning is the Faulkner set-up,

involving two figure-of-eight microphones spaced 20 cm apart

and pointing towards the sound source: one might expect to

have a narrow stereo image, but that's not the case - and the

sonic image is quite focused.

5. The MS technique is quite handy when you are dealing with

ensembles, which require a relatively close microphone

placement (e.g. choirs), and in a good reverberant ambience.

Also, I personally like to think more in terms of stereo width

than L/R position, which is necessarily the case with the MS

technique.

6. With a small ensemble, in a dryish room, the Blumlein

technique (two coincident figure-of-eight microphones angled

90=B0 apart) may yield very good results, mostly in terms of

stereo imaging.

7. It is my very personal opinion that an average method which

will rarely produce awful results is the ORTF (semi-coincident

cardioids spaced by 17 cm and angled 110=B0 apart). A few

recordings I've done with this technique sound superb to me,

others sound decent, very few acceptable. None sounds totally

wrong, though. I'd personally recommend this technique for a

quick recording in a place whose acoustic character is unknown

and as a good starting point, in general.

8. Last but not least, if you're dealing with a well-balanced and

not-too-big ensemble or any small group with instruments whose

loudness is on the soft side). I recommend you try the Jecklin

disc (or OSS technique). Two Omni's separated by a circular

baffle of acoustic foam 28 cm of diameter: a first step towards

binaural recordings which can lead to astonishing results. Be

careful, though positioning is extremely crucial, and can be an

excruciatingly gory affair, at times... Also, this is "not so

recommended" in strongly reverberant halls, as the Omni's tend to

pick up more ambience than necessary.

One last small detail: some Omni's are more suitable for work in

the near field (that is, close to the sound source); whereas

others are happier to operate in the so-called diffuse field

(relatively far from the sound source). For stereo recording of

large ensembles, the latter are recommended. Some manufacturers

(e.g. Bruel & Kjaer) can provide caps, which act as mechanical

equalizers, so that one may adapt the microphones to diverse

situations. [Marco]

--

Q10.3 - How do I soundproof a room?

Despite what you may have seen in the movies or elsewhere, egg

crates on the wall don't work!

First, understand what "soundproofing" means. Here we mean the

means and methods to prevent sound from the outside getting in,

or sound from the inside getting out. The acoustics within the

room are another matter altogether.

There are three very important requirements for soundproofing:

mass, absorption, and isolation.  Actually, there are also three

others: mass, absorption, and isolation. And to finish the job,

you should also use: mass, absorption, and isolation.

Sound is the mechanical vibration propagating through a material.

The level of the sound is directly related to the size of those

vibrations. The more massive an object is, the harder it is to

move and the smaller the amplitude of the vibration set up in it

under the influence of an external sound. That's why well

isolated rooms are very massive rooms.  A solid concrete wall

will transmit much less sound then a standard wood-framed, gypsum

board wall. And a thicker concrete wall transmits less than a

thinner one: not so much because of the distance, but mostly

because it's heavier.

Secondly, sound won't be transmitted between two objects unless

it's mechanically coupled.  Air is not the best coupling

mechanism. But solid objects usually are. That's why well

isolated rooms are often set on springs and rubber isolators.

It's also why you may see rooms-within rooms: The inner room is

isolated from the outer, and there may be a layer of absorptive

material in the space between the two. That's also why you'll

also see two sets of doors into a recording studio: so the sound

does not couple directly through the door (and those doors are

also very heavy!).

If you are trying to isolate the sound in one room from an

adjoining room, one way is to build a second wall, not attached

to the first. This can go a long way to increasing the mechanical

isolation. Try using two sheets of drywall instead of one on each

wall, and use 5/8" drywall instead of 3/8", it's heavier.

But remember: make it heavy, and isolate it. Absorptive materials

like foam wedges or Sonex and such can only control the acoustics

in the room: they will do nothing to prevent sound from getting

in or out to begin with.  [Dick]

There is a very good reference on Studio and Control Room design

with lot's of good information on soundproofing and materials

located on the web at the following URL.

The Recording Studio Design Page

http://www.mcs.net/~malcolm/

--

Q10.4 - What is a near-field monitor?

A near field monitor is one that is design to be listened to in

the near field. Simple, eh?

The "near field" of a loudspeaker is area where the direct,

unreflected sound from the speaker dominates significantly over

the indirect and reflected sound, sound bouncing off walls,

floors, ceilings, the console. Monitoring in the near field can

be useful because the influence of the room on the sound is

minimized.

Near field monitors have to be physically rather small, because

you essentially need a small relative sound source to listen to

(imagine sitting two feet away from an 18" woofer and a large

multi- cellular horn!). The physics of loudspeakers puts severe

constraints on the efficiency, power capabilities and low

frequency response of small boxes, so these small, near-field

monitors can be inefficient and not have the lowest octave of

bass and not play ungodly loud. [Dick]

--

Q10.5 - What are the differences between "studio monitors" and home

loudspeakers?

It depends upon whom you ask.  There are speakers called

"monitor" speakers that are found almost exclusively in homes and

never in studios.

The purpose of a monitor speaker is to monitor the recording and

editing process.  If you buy the concept that they are but one

tool in the process (and probably the most frequently used single

tool at that), and if you buy the concept that your tools should

be flawless, than the requirements for a monitor speaker are easy

to state (but hard to achieve): they should be the most neutral,

revealing and unbiased possible.  They are the final link between

your work and your ears, and if they hide something, you'll never

hear it.  If they color something, you might be tempted to

uncolor it incorrectly the other way.

There is another camp that suggests that monitor speakers should

represent the lowest common denominator in the target audience.

The editing and mix process should be done so that the results

sound good over the worst car speaker or boom box around. While

such an idea has validity as a means of verifying that the mix

will sound good over such speakers, using them exclusively for

the process invites (as has been thoroughly demonstrated in many

examples of absolutely terrible sounding albums) the possibility

of making gross mistakes that simply can't be heard in the mixing

process. [Dick]

--

Q10.6 - My near field monitors are affecting the colors on my video

monitor. What can I do to shield the speakers?

Despite a lot of folk lore and some very impressive sounding

wisdom here on the net and in showrooms, there is effectively

nothing that you can do to the speakers or the monitor, short of

moving them away from one another, that will solve this problem.

The problem comes from the magnetic field created by and

surrounding the magnets in the loudspeaker. It's possible to

design a magnet that has very little external field, but it can

be an expensive proposition for a manufacturer.  If the magnets

do have large external fields, the only technique that works is

by solving the problem at the source: the magnet.  Special

canceling magnets are used, sometimes in conjunction with a "cup"

shield directly around the magnet.

You'll hear suggestions from people about placing a sheet of iron

or steel between the speakers and the monitor. That might change

the field, but it will not eliminate it. As often as not, it will

make it worse.

You'll also here from people about shielding the speaker by

lining the enclosure with lead or copper.  This method is

absolutely guaranteed to fail: lead, copper, aluminum, tin, zinc

and other such materials have NO magnetic properties at all, they

will simply make the speaker heavier and won't solve the problem

at all. There is but one material that has a shot at working:

something called mu-metal, a heavy, very expensive, material

designed for magnetic shield that requires extremely

sophisticated and difficult fabrication and annealing techniques.

Its cost is far greater than buying a new set of speakers that

does not have the problem, and it may not even work if the

majority of the offending field is radiated from the front of the

speaker, which you obviously can't shield.

Try moving the speakers relative to your monitor. Often, moving

them an inch or two is enough to cure the problem or at least

make it acceptable. Sometimes, placing the speakers on their

sides with the woofers (the major offenders in most cases)

farthest away from the monitor works.  [Dick]

-----

Section XI - Industry information

Q11.1 - Is there a directory of industry resources?

Q11.2 - What are the industry periodicals?

Q11.3 - What are the industry trade organizations?

Q11.4 - Are there any conventions or trade shows that deal

specifically with professional audio?

-----

Section XII - Miscellaneous

Q12.1 - How do I modify Radio Shack PZMs?

See http://www2.eng.cam.ac.uk/~cmh/audio_papers/PZM.txt [12/98]

Q12.2 - Can I produce good demos at home? [9/98]

Sure you can. The problem is focusing on what one wants to do,

depending on the kind of music to be recorded and the results one

aims to obtain. There is a whole world of difference between,

say, demoing your latest dance song and recording a tune based on

acoustic guitar and vocals.

So, in order to avoid an inordinate growth of this section, I

will try to give a few hints that might turn in handy if the

purpose is that of making an acceptable recording. It is hard to

state any universal rules, but I guess that there are some

general lines that at least make some sense, no matter whether

you're MIDI-based want to record your own band in the basement.

The following ideas are not, of course, carved in stone, and

someone could argue about some of them; I guess they at least

suggest some basic guidelines, though.

1. Remember that a good demo's purpose is that of giving a fair

idea of what a band sounds like and how a song is written,

arranged and performed. It is not that of sounding like a top-

flight production.

2. If you have to record drums, remember that a few decent

microphones give you a better sound than 10 crappy ones. Try

as follows: two overhead mics above the cymbals (condensers,

if possible; otherwise a couple of SM57s will do); one mic on

the kick, and one of the snare. Try to get a reasonable

balance, and record everything to 2 tracks on your multi-

track, with overheads panned hard left and right and kick and

snare panned dead center. Try adding a dash of suitable

reverbs to the latter, if you wish and a bit of compression as

well, if you have the chance. Even one overhead might work, if

you're short of tracks. You'll end up with mono drums, but I'd

argue that's less relevant than one might think, at demo-

level.

3. On the subject of drum-micing - but also in general: remember

that EQ can be used to cut, as well as boosting! Cutting the

LF a bit on the overheads (and, in general, on signals not

having predominant bass frequencies) may help a lot in terms

of image sharpening.

4. EQ-wise, try to give each part a definite area of the

frequency spectrum to live in, with a pinch of salt. An

electric guitar with a serious bottom end may sound marvelous

on its own, but it might as well become a spike in your side

when the bass is added. You may discover that cutting the LF a

bit on the guitar could help more than boosting it on the

bass.

5. Try not to over-use reverb. If you listen carefully to almost

any pro recording, you will be amazed at how subtly and

sparingly reverbs are used. The same goes for delays and other

effects, unless you're after a deliberate abuse of 'em, of

course.

6. If you have a limited number of tracks and need more, you can

easily bounce more tracks together to free space. On digital

machines as the ADAT this doesn't affect the quality of your

recording significantly. But if you have an analogue multi-

track, after two bounces you're likely to be in trouble

because of layered tape noise. On cheap analogue multi-tracks

the HF signals tend to blur, as well. To avoid this, you may

want to mix the tracks you have onto some digital medium (DAT,

Minidisc, or what-have-you), and then record them back onto a

fresh section of the tape. The bonus is that you don't

actually have to destroy your previous tracks.

7. About compression: in my opinion, if you have a compressor,

you'd better use it on the vocals and on other instruments

when needed, rather than compress the whole mix, unless you

know exactly how to do it. If you have an analogue multi-track

you can try recording every signal a bit hotter than suggested

in your manual. You will probably get a nice, warm and SLIGHT

compression, which usually makes the whole thing sound

tighter.

8. As for mixing:

A: don't mix on headphones

B: keep your loudspeakers at reasonable levels

C: turn them definitely down every once in a while and check if

you can still hear everything

D: if you doubt some part is too loud, try listening to the mix

from an adjacent room: if the part in question sounds louder than

the rest, it is a good idea to turn its level down a bit

E: if you mix onto a digital medium, stay well away from clipping

(0 dB), and mind those transients in particular

F: try, try, try, try again, then try...

8. If you record for purely listening purposes, once you have a

mix you're satisfied with, don't worry about it anymore. If

you want to send your stuff around, though, and you are not

completely satisfied with the quality of your demo, you may

want to invest in the services of a mastering house. A top-

class facility will be probably expensive, but there are

places with good gear and trained engineers who won't leave

you broke. You shouldn't expect miracles, but some little

wonders can be achieved, at mastering stages.

9. Most of all: try to have what everything in music should be

about - Serious Fun. "When you try to drop in / on the head of

a pin / that's Recording". Remember! [Marco]

--

Q12.3 - How do I remove vocals from a song?

You probably want a device called the Thompson Vocal Eliminator

made by LT Sound in Atlanta, Georgia.  The device will cancel out

any vocal that is mono and panned dead center.  The unit works by

filtering out the low frequencies phase canceling the rest of the

signal, and then mixing the filtered bass back in.  The result is

a signal with the center, common information cancelled out.

Sometimes it works well, other times it sounds awful. [Gabe]

-----

Section XIII - Bibliography

Q13.1 - Fundamentals of Audio Technology

Q13.2 - Studio recording techniques

Q13.3 - Live recording techniques

--

Q13.4 - Digital audio theory and practice

Ken C. Pohlmann, "Principles of Digital Audio," SAMS/Prentice

Hall, 1993 [Excellent introduction and explanation of all

aspects of digital audio principals and practices]

John Watkinson, "The Art of Digital Audio," Focal Press, 1989

[ditto]

Francis Rumsey and John Watkinson, "The Digital Interface

Handbook," Focal Press, 1993 [deals with interfacing standards

and protocols for both audio and video]

IEC Standard Publication 958, "Digital Audio Interface,"

International Electro-Technical Commission, 1989, 1993 [THE

standard!]

Claude E. Shannon and Warren Weaver, "The Mathematical Theory

of Communication," U. Chicago Press, 1963.  [The seminal work

on digital sampling...includes the original 1948 BSTJ sampling paper]

--

Q13.5 - Acoustics

Leo Beranek, "Acoustics," New York, American Institue of

Physics, 1986 [The bible, but heavily mathematical, very thick

and obtuse, a good book despite the Lincoln Hall disaster.]

F. Alton Everest, "The Master Handbook of Acoustics," Tab

Books 1989 [A good entry level text on acoustics, studio and

listening room design, not heavily mathematical].

Arthur H. Benade, "Fundamentals of Musical Acoustics," New

York, Dover Publications, 1990 [A thorough book, more on the

acoustics of PRODUCING music rather than REPRODUCING it.]

Any good physics text book is useful for keeping the bunk at bay.

Q13.6 - Practical recording guides

Ront Streicher and Alton Everest, "The New Stereo Soundbook" now

out of print but shortly to be reissued in a much improved hardback

edition, contact Wes Dooley <stereoms@aol.com> for details [David 12/98]

-----

Section XIV - Miscellaneous

Q14.1 - Who wrote the FAQ? [9/98]

GABE WIENER - IN MEMORIUM 1997/04/10

Since his days as a student at Columbia University, Gabe did

wonderful work as a recording engineer, producer, restorer, and

entrepreneur.  The company he founded, Quintessential Sound, is

successful and highly regarded; PGM Records, his label devoted to

under-recorded Baroque music, has received many excellent reviews.

I knew Gabe well from the Internet, and from telephone

conversations.  It is a sign of our times that we could be

comfortable and friendly with one another even though we stood

face to face only on three occasions.  The last of those was on

the day that the first PGM release was recorded.  I consider it a

great privilege to have introduced Gabe to Gavin Black, the

harpsichordist who performed the music on PGM 101; Gabe and Gavin

worked closely together since then.

That day I watched Gabe work.  He had rented the best space he

could find--the American Academy of Arts and Letters, on 155th

St. in Washington Heights, NY--and had brought in his Nagra-D

tape recorder, microphones, a laptop connected to the recorder,

and a few other items.  With him were an assistant, Gavin, and a

harpsichord technician.  Having heard Gavin's playing on many

occasions (we were college roommates almost 20 years ago), I was

familiar with his playing, and was thus able to concentrate on

Gabe's work.

Gabe was amazing.  His focus was beyond compare, his musical

acumen completely blended with his engineering work. Most of all,

his natural ability to work with the people around him carried

the project to completion.  The CD was recorded in a single day.

In mourning him, I can take comfort only in knowing that Gabe

Wiener was given several years of that which most of us dream of

having for a day: doing exactly what he wanted to do, excelling

at it, gaining the respect of his peers, and succeeding in

worldly terms.  However crushing his early death seems, the

memory of this time, all too short, when Gabe's enthusiasm,

drive, skill, and sheer pleasure led to fine music-making and

superb recording for all of us to hear, is something to live by.

Roger Lustig (julierog@ix.netcom.com)

Q14.2 - How do you spell and pronounce the FAQ founder's surname?

Those who paid attention during fourth grade English learned the

rule "I before E except after C."  There is no C in my name, and

therefore the I comes before the E.  My last name is spelled

"Wiener" not "Weiner."

And it rhymes with cleaner, and starts with a W, not a V.  [Gabe]

=====

--

Gabe Wiener Dir., PGM Early Music Recordings

A Div. of Quintessential Sound, Inc., New York

Mastering-Restoration (212) 586-4200

"I am terrified at the thought that so much hideous and bad Recording

music may be put on records forever."--Sir Arthur Sullivan