=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

PC-DRAGON EMULATOR V1.02 by PAUL BURGIN

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Document date: 3/12/93

Introduction

============

Welcome to PC-Dragon, a Dragon or Tandy Color Computer II emulator for IBM

PC's and compatibles. This program allows you to use your Dragon or Tandy

CoCo games and applications software without the need to clutter up your desk

with more than one computer.

Either a Dragon 32, Dragon 64 or Tandy CoCo II with Extended Color Basic is

emulated by PC-Dragon, depending on the ROM file used. The emulator will

automatically detect which architecture and breakpoints to use, although if

you have more than one ROM image you may also choose a machine using the

available command line switches.

This documentation refers in general to the 'Dragon' to mean either of Dragon

32, Dragon 64 or Tandy CoCo II. Where important differences are known they are

stated explicitly.

If you have obtained PC-Dragon from a bulletin board or ftp server then you

cannot use the emulator straight away. This is because the emulator requires

a copy of the ROM chip from your Dragon or Tandy CoCo to function correctly,

and this software cannot be included in the .ZIP for copyright reasons.

Full details of how to transfer the ROM from your machine can be found

later in the document.

Shipping List

=============

The archive contains the following files:

DRAGON.EXE    --  PC-Dragon main executable.

DRAGDEB.EXE   --  Interactive debugging version of PC-Dragon.

CLOADM.EXE    --  Utility to grab a PC-Dragon .CAS virtual cassette file

from a sampled .WAV or .VOC file. Also extracts raw .DGN

or .BAS files from .CAS files.

CSAVEM.EXE    --  Utility to create a 44.1KHz .WAV file from any PC-Dragon

.CAS virtual cassette file.

CAS2PAK.EXE   --  A utility to convert .CAS files to the .PAK snapshot format

used by CoCo2-12.

ROM_STUB.DGN  --  A header file for raw ROM images.

PCDRAGON.DOC  --  This documentation file!

HISTORY.DOC   --  Version history for PC-Dragon.

INSTALL.BAT   --  You can use this file to create an installation disk for

PC-Dragon if you so wish. Simply copy it onto a floppy disk

along with PCDGN102.ZIP, PKUNZIP.EXE, DRAGAMES.ZIP (if

appropriate), and .DGN or .ZIP format copies of any ROM

files that you use. It will also install .CAS files if

included on the disk.

RUNME.BAT     --  Run this file when you first install PC-Dragon to check

that PC-Dragon has been installed properly and that you are

not missing any essential files.

A selection of example cassette files may be found in the directories

CASSETTE, CASSETTE\ARCHIVE and CASSETTE\GAMES, including Shanghai and

previously unreleased Tetras game. These can be used from within the emulator

by typing CLOADM or CLOAD and RUN as appropriate. However, only the BASIC

programs work with Tandy CoCo emulation.

All programs, including the example files, are the sole work of the author.

The freeware distribution status applies to all programs in the archive,

however this status is only granted as a collective package. Please read the

freeware notice at the end of this file.

Minimum Requirements

====================

PC-Dragon runs on any PC with VGA graphics. It can be run full-screen from

Windows, or in a DOS box if graphics modes are not required. It also runs

fine with the Acorn Archimedes 80188 PC emulator.

However, 33MHz machine or faster is recommended.

A Mouse Systems or Microsoft compatible mouse and driver must be installed

to use the mouse support and joystick emulation.

A soundcard and software are required to read and write dragon cassette tapes

using the CLOADM.EXE and CSAVEM.EXE utilities.

The emulator uses at most 240K of base memory.

DRAGON.EXE

==========

General Features

----------------

The following features are emulated:

*   All documented 6809 instructions, addressing modes and registers.

*   Text screen; IBM approximations to Dragon graphic blocks are used.

*   All 56 Dragon 2 and 4 colour graphics modes.

*   Keyboard input, at hardware or operating system level.

*   Cassette input/output redirected to DOS via ROM breakpoints.

*   IRQ interrupt, connected to 50Hz VDG refresh signal (see below).

*   Joystick input at operating system level via ROM breakpoint.

*   Automatically provides D32, D64 or Tandy CoCo II architecture.

*   Reset button.

Other features:

*   Debugging version. Allows single stepping, breakpoints, register and

memory dumps etc.

*   A range of extra PC/DOS emulator services available from the Dragon.

Coming soon:

*   Joystick emulation using the PC's joysticks, rather than mouse.

Possible future features:

*   Joystick input at hardware level.

*   Snapshots, possibly using the existing .PAK format.

*   'Semigraphics' graphics modes.

*   Parallel port.

*   Increased speed. :-)

*   Sound emulation using Sound Blaster (but not before 100% speed)!

*   Additional debugging features, and PC/DOS options and services.

Keyboard

--------

The Dragon keyboard is emulated in two ways; you can choose for keyboard

input to take advantage of the PC's type-ahead buffer and autorepeat features

('PC mode'), or, where necessary, accurate low-level emulation of the Dragon

or Tandy CoCo hardware can be selected ('real mode').

The emulator will use the 'PC mode' by default. This will work for most BASIC

programs and some machine code applications. Where a program will not function

correctly in this input mode simply select 'real mode' by pressing CTRL-F1,

or by giving the /K option on the command line when you start PC-Dragon.

The keyboard keys will produce different effects on-screen depending on

the input mode chosen. There are also slight differences between 'real mode'

and the pseudo-hardware emulation used when a program attempts to read the

keyboard at the hardware level in 'PC mode'.

Common special key mappings used are as follows:

END or ESC        -- Break key.

HOME              -- Clear key.

PG UP             -- Pause (Shift + @ key).

PG DN             -- Continue (Enter key).

INS , DEL         -- I, D keys (as used by EDIT).

CURSOR KEYS       -- Dragon cursor keys.

ALT + CURSOR KEYS -- Shift + Dragon cursor keys (real mode only).

ALT + ESC         -- Escape key (as used by EDIT) (real mode only).

CAPS LOCK         -- Dragon caps lock (Shift + 0 key).

[ , ]             -- ( , ) brackets (real mode only).

The following keys are interpreted by the emulator and are available in both

keyboard modes (response times vary in PC mode):

F1                -- Help page.

ALT   + F1        -- Information page.

SHIFT + F1        -- View this manual (CTRL-C to exit).

CTRL  + F1        -- Toggle PC/real keyboard mode.

F2                -- Toggle cursor on/off.

F3                -- Toggle lower case on/off.

F4                -- Toggle joystick type analogue/digital.

F5                -- Toggle between video palettes.

F6                -- Toggle between 2 and 4 colour graphics.

F7                -- Toggle graphic screen width.

ALT   + F5        -- View text screen at 400h.

ALT   + F6        -- View PMODE3 graphic screen at 600h.

SHIFT + F7        -- Decrease graphic resolution.

SHIFT + F8        -- Increase graphic resolution.

ALT   + F7        -- Graphic screen down by one screen.

ALT   + F8        -- Graphic screen up by one screen.

CTRL  + F7        -- Graphic screen down by 200h.

CTRL  + F8        -- Graphic screen up by 200h.

F8                -- Rewind all virtual cassettes.

SHIFT + F4        -- View CASSETTE directory.

CTRL  + F4        -- View CASSETTE\ARCHIVE directory.

ALT   + F4        -- View CASSETTE\GAMES directory.

F9                -- Warm reset.

ALT   + F9        -- Cold reset.

ALT   + F10       -- MS-DOS command shell.

F10               -- Exit.

SCROLL LOCK       -- Hold video mode.

In real mode SCROLL LOCK will not stay engaged - it is necessary to hold it

depressed. An alternative is to press SHIFT-F4, engage SCROLL LOCK, and then

return to the emulator by pressing F4 again.

To return from MS-DOS shell you should type EXIT as normal. You should return

to the PC-Dragon directory before exiting if you plan to access any files

after doing so.

If the emulator exits to DOS when you press CTRL-C, this may be helped by

removing any 'BREAK=' lines from your CONFIG.SYS file.

Virtual Cassettes

-----------------

PC-Dragon will read and write .CAS virtual cassettes from the directory

CASSETTE below the PC-Dragon directory. Read only cassettes may also be

accessed from the directories CASSETTE\GAMES and CASSETTE\ARCHIVE. The

emulator will derive a valid DOS filename from the Dragon filename before any

cassette operation.

However, since the Dragon can use more characters in filenames than the PC,

there are many possible Dragon filenames that will translate into the same

DOS filenames. Be warned that PC-Dragon asks no questions before overwriting

existing files.

F8 may be pressed at any time to force the emulator to rewind all virtual

cassettes (close DOS files). This is rarely necessary, as it is taken care of

automatically at the end of an operation, or upon reset/exit, but is needed

after receiving a loading error.

Interrupts

----------

PC-Dragon currently runs at about 1/3 to 1/5th of the speed of a real Dragon

32 on a 486DX-33 PC. The IRQ interrupts occur relative to the emulated

machine speed, once every 17,800 emulated clock cycles, thus giving the TIMER

a resolution of about 10 to 14Hz, rather than 50Hz.

If necessary, you can give the /Q option on the command line to quadruple

the interrupt rate. This allows some programs which use the IRQ interrupt

for delay purposes to run faster (eg. Chuckie Egg), as well as reducing the

pauses during sound effects in some programs.

Joysticks

---------

A Mouse Systems or Microsoft compatible mouse is used to emulate the Dragon

joysticks. Either an analogue or digital joystick may be selected; press F4

to alternate between types (analogue is default).

Fast Mode Switching

-------------------

Software which uses fast mode switching techniques to achieve special effects

(such as parts of Rommels Revenge, Space Wrek and Zaxxon) will not be

displayed very smoothly by the emulator as it cannot currently refresh the

graphic screen fast enough. Engage SCROLL LOCK to defeat the effect, fixing

the display into a single video mode.

Command Line Switches

---------------------

The following switches can be given on the command line to start the emulator

with different options, or to select different architectures if you have more

than one ROM file:

/V     Return version number of the emulator.

/W     Wipe RAM at startup. Use this option if the emulator will not

start as a result of the reset vector having being redirected.

/K     Select hardware mapped keyboard emulation.

/Q     Quadruple interrupt frequency.

/32    Emulate a Dragon 32. If the default ROM is not the Dragon 32 ROM

then it should be named D32ROM.DGN.

/64    Emulate a Dragon 64. If the default ROM is not the Dragon 64 ROM

then it should be named D64ROM.DGN. The optional second Dragon 64

ROM should be named D64ROM2.DGN.

/T     Emulate a Tandy CoCo. If the default ROM is not the Tandy CoCo

ROM then it should be named TANDYROM.DGN.

Any other argument is taken to be the filename of an alternative cartridge

file (must be a machine code file in .DGN format). You can also give /? or

/H to obtain a summary of these options. A '-' may be substituted for '/'.

Extended Emulator Services

--------------------------

PC-Dragon also provides a range of services to allow the emulated Dragon

to access selected PC-Dragon configuration options and also to interface to

the PC and DOS hardware and environment.

The services are all accessed by the use of special write-only memory in

the region FFECh to FFEFh and may be used from both BASIC and machine code.

Upon entry to the emulator the services are disabled in order to maintain

software compatibility. To enable the extended services simply write 32h

to address FFEEh and 43h to address FFEFh.

The services are then accessed by reading from address FFEEh or FFEFh. If

FFEEh is used, the 6809 registers A,B,X,Y and U are used to pass any required

parameters to and from the services. Alternatively, if FFEFh is used to

initiate a service then the parameters are read from and written to memory.

Memory locations FFECh and FFEDh should point to the address of the parameters

and they should be formatted as a register dump in the order A,B,X,Y and U.

On startup the contents of memory addresses FFECh to FFEFh are initialised to

FFE00000h.

The following are services available in this version (and above); in all

cases the B register is used to indicate the required service number:

NUMBER | DESCRIPTION

--------+-------------------------------------------------------------------

0   | INSTALLATION/VERSION CHECK

|  Return:         A = number of last service supported

|                  X = 3243h signature

|                  Y = PC-Dragon version (major, minor)

--------+-------------------------------------------------------------------

1   | CLOSE DOS FILES

--------+-------------------------------------------------------------------

2   | PC CURSOR CONTROL

|  Params:   A b0-b6 = 0 off, 1 on (solid),     >1 on (underline)

|            A b7    = 0 change,                1 do not change

|  Return:         A = previous state

--------+-------------------------------------------------------------------

3   | LOWER CASE CONTROL

|  Params:      A b0 = 0 disable,               1 enable

|               A b1 = 0 do not refresh screen, 1 refresh screen

|               A b7 = 0 change,                1 do not change

|  Return:         A = previous state

--------+-------------------------------------------------------------------

4   | JOYSTICK EMULATION CONTROL

|  Params:   A b0-b6 = 0 analogue,              >0 digital

|            A b7    = 0 change,                1 do not change

|  Return:         A = previous state

--------+-------------------------------------------------------------------

5   | GET/SET TEXT MODE COLOURS

|  Params: A b0      = 0 do not refresh screen, 1 refresh screen

|          A b7      = 0 change,                1 do not change

|          X b15-b12 = new foreground colour in 'screen 0,0'

|          X b11-b8  = new background colour in 'screen 0,0'

|          X b7 -b4  = new foreground colour in 'screen 0,1'

|          X b0 -b3  = new background colour in 'screen 0,1'

|  Return:         X = previous values; eg. BLK, GREEN, BLK, BROWN

--------+-------------------------------------------------------------------

6   | GET/SET GRAPHICS MODE COLOURS

|  Params:  A b0     = 0 'pmode 0'    colour set, 1 'pmode 1'

|           A b1     = 0 'screen 1,0' colour set, 1 'screen 1,1'

|           A b2     = 0 do not refresh screen,   1 refresh screen

|           A b7     = 0 change,                  1 do not change

|           X b15-b8 = VGA colour for all of Dragon '00' bit pairs

|           Y b15-b8 = VGA colour for '0' of Dragon '01' bit pairs

|           Y b0 -b7 = VGA colour for '1' of Dragon '01' bit pairs

|           U b15-b8 = VGA colour for '1' of Dragon '10' bit pairs

|           U b0 -b7 = VGA colour for '0' of Dragon '10' bit pairs

|           X b0 -b7 = VGA colour for all of Dragon '11' bit pairs

|  Return:         X = previous values for 00 and 11; eg. GREEN, RED

|                  Y = previous values for 01; eg. YELLOW, YELLOW

|                  U = previous values for 10; eg. BLUE, BLUE

--------+-------------------------------------------------------------------

7   | GET/SET GRAPHICS BORDER COLOURS

|  Params:  A b0     = 0 do not refresh screen,   1 refresh screen

|           A b7     = 0 change,                  1 do not change

|           X b15-b8 = VGA colour for border in 'screen 1,0'

|           X b0 -b7 = VGA colour for border in 'screen 1,1'

|  Return:         X = previous values; eg. GREEN, WHITE

--------+-------------------------------------------------------------------

8   | GET MOUSE POSITION AND BUTTON STATUS

|  Return:      A b0 = 0 left   button not pressed, 1 pressed

|               A b1 = 0 right  button not pressed, 1 pressed

|               A b2 = 0 middle button not pressed, 1 pressed

|                  X = mouse x position

|                       (range 0 to 31  in text mode)

|                       (range 0 to 255 in graphics mode)

|                  Y = mouse y position

|                       (range 0 to 15  in text mode)

|                       (range 0 to 191 in graphics mode)

|              ERROR = 1 if no mouse driver found

|  NB: FF00h is added to X or Y if pointer lies beyond screen edge

--------+-------------------------------------------------------------------

9   | EXIT TO DOS

--------+-------------------------------------------------------------------

10   | MOUSE POINTER CONTROL

|  Params:   A b0-b6 = 0 pointer off,           >0 pointer on

|            A b7    = 0 change,                1 do not change

|  Return:         A = previous state

|              ERROR = 1 if no mouse driver found

--------+-------------------------------------------------------------------

All registers not explicitly specified in the table are not affected, and all

services return 0 upon success, 255 if the requested service is not supported

or an error code as specified.

The following are the available colour values; for graphics colours the value

can be anything in the range 0 to 63, these are merely examples:

TEXT COLOURS     |    BACKGROUND COLOURS    |      GRAPHICS COLOURS

----------------------+--------------------------+--------------------------

BLACK          0   |     BLACK          0     |      BLACK          0

GREEN          2   |     GREEN          2     |      GREEN          2

YELLOW        14   |     BROWN*        14     |      YELLOW        62

BLUE           1   |     BLUE           1     |      BLUE           1

RED            4   |     RED            4     |      RED            4

BUFF           7   |     BUFF           7     |      BUFF           7

CYAN           3   |     CYAN           3     |      CYAN           3

MAGENTA        5   |     MAGENTA        5     |      MAGENTA       61

BROWN          6   |     BROWN          6     |      BROWN          6

DARK GRAY      8   |     BLACK*         8     |      DARK GRAY     56

LIGHT GREEN   10   |     GREEN*        10     |      LIGHT GREEN   58

LIGHT BLUE     9   |     BLUE*          9     |      LIGHT BLUE    57

LIGHT RED     12   |     RED*          12     |      LIGHT RED     60

WHITE         15   |     BUFF*         15     |      WHITE         63

LIGHT CYAN    11   |     CYAN*         11     |      LIGHT CYAN    59

PINK          13   |     MAGENTA*      13     |      DARK MAGENTA   5

*with flashing foreground text

To give quick examples, the following will change the default text screen

colours to BLUE on BUFF from assembly language:

LDD #$3243   ; Enable PC services.

STD $FFEE

LDD #5       ; Service 5: set text colours.

LDX #$1706   ; Setup new colours.

TST $FFEE    ; Call emulator service.

BNE FAILED   ; Branch to FAILED if real Dragon.

To turn off the PC cursor from BASIC:

POKE &HFFEE,ASC("P")                    'Enable PC services.

POKE &HFFEF,ASC("C")

POKE &HFFE1,2                           'Service 2: cursor control.

POKE &HFFE0,0                           'New state is off.

IF PEEK(&HFFEF) THEN ?"Not Emulator"    'Call service, check return.

And finally, a short program which will allow the mouse to draw on a PMODE3

graphics screen. Normal colour will be yellow; hold left mouse button for

blue, right button for red, or both to erase.

10 PMODE3:SCREEN1,0:PCLS

20 POKE&HFFEE,ASC("P")                  'Enable PC services.

30 POKE&HFFEF,ASC("C")

40 POKE&HFFE1,8                         'Service 8: get mouse status

50 IF PEEK(&HFFEF) THEN STOP            'Call service, check return.

60 COLOR (PEEK(&HFFE0)+2) AND 7         'Set colour from button info.

70 LINE-(PEEK(65507),PEEK(65509)),PSET  'Draw line to new position.

80 GOTO 50

Illegal And Undocumented Instructions

-------------------------------------

Unfortunately I have no information regarding how a 6809 responds to illegal

instructions, and no knowledge of any possible undocumented instructions

either. If someone could clarify the situation, please get in touch.

That said, undocumented instruction are hence treated as illegal, and these

come in two flavours:

1. Illegal opcodes. These are all merely treated as NOP instructions.

The emulated 6809 engine simply goes onto the next instruction.

2. Illegal postbytes. With the exception of register transfers with

mismatched sizes (ignored), I have taken no special actions for these

in order to maximise speed. The resulting effects of such postbytes

are hence dependent on the way I have written the decoding.

Eg. STA (,U+) will execute correctly due to indexed mode symmetry

TFR $17   will transfer X to the last valid register used

Update: Thanks to Keiran Anscomb, I now have information regarding the

undocumented RESET opcode, and undocumented effects of illegal register

transfers and exchanges. These will be implemented soon.

Hints For Using Virtual Cassettes

---------------------------------

The .CAS file type used by the emulator is simply a raw bitstream of cassette

data. Because of this it is simple to append one cassette onto the end of

another by using the MS-DOS copy command (the /B switch must be given so that

a binary copy is performed). For example, the following will add the cassette

EXTRA onto the end of the cassette FIRST:

COPY /B   FIRST.CAS + EXTRA.CAS   FIRST.CAS

Since the emulator does not need to physically start and stop a cassette

recorder it does not need a motor on delay when writing data. Nor are file

headers required; you can therefore speed up PC-Dragon writes to cassette (and

save disk space) by issuing the following POKEs before CSAVE, CSAVEM or OPEN

commands:

Dragon (FASTDRAG.CAS):

POKE 144,0 : POKE 145,1    'Reduce number of leader bytes.

POKE 149,0                 'Reduce cassette motor delay.

Tandy CoCo (FASTANDY.CAS):

POKE 146,0 : POKE 147,1    'Reduce number of leader bytes.

'Cannot reduce Tandy CoCo motor delay.

If you are planning to use CSAVEM.EXE to transfer files back to a real Dragon

then you should save the file with sufficient leader bytes for the real

Dragon to detect. For example, you could type

Dragon:

POKE 144,3                 'Greatly increase number of leader bytes.

Tandy CoCo:

POKE 146,3                 'Greatly increase number of leader bytes.

before saving from the emulator. Alternatively, if you have saved your file

with insufficient leader bytes, you may be able to load the file into the

Dragon by using a waveform editor to insert silence before the leaders, or by

typing

Dragon:

POKE 149,0 : POKE 150,1    'Minimise cassette motor delay.

on the receiving Dragon.

Please note that since the DOS filename for a PC-Dragon virtual cassette is

derived from the first Dragon file that it contains, you cannot simply use

the MS-DOS rename command to change the name of a .CAS file. If you do so you

will not be able to access the file from within the emulator. You will need

to use a hex editor to change the name of the Dragon file, and also remember

to update the checksum for the filename block (the last byte before the next

stream of 55h bytes).

Attempting to access a file from the Dragon using an empty filename will

translate to the DOS filename _.CAS; attempting to access a file without

setting any filename will result in the file DEFAULT.CAS being opened.

Machine code files can be used with either Dragon or Tandy CoCo emulation

but are unlikely to execute properly due to the ROM differences. BASIC

programs can be used and executed with either architecture so long as they

are saved in ASCII format, eg.

CSAVE "BAS-PROG", A

but will use more disk space when saved in this format (as well as taking

much longer to save/load).

Cartridge Files

---------------

Any machine code .DGN file may be renamed to CARTRIGE.DGN to act as a default

cartridge upon entry to the emulator. The ROM_STUB.DGN file can be used in

conjunction with the MS-DOS COPY command, as described in the section 'How To

Transfer Your ROM For Use With PC-Dragon' to make .DGN files of any cartridges

that you have transferred to your PC as a raw image (eg. by uploading the

software from an EPROM programmer).

Cartridge files can also be copied to cassette and transferred using the

CLOADM.EXE utility. In most cases you should save either 4K or 8K of data.

For example, to capture Alldream use:

CSAVEM "ALLDREAM", &HC000, &HDFFF, &HC000

(The execution address is not really required, but may prove useful in the

future). Auto-booting cartridges may be captured using a technique similar

to that described in the documentation for CoCo2-12.

The .DGN file format used by PC-Dragon for raw data is a simple format

structured as follows:

-------------+--------------------------------------------------------------

Byte Offset |                       Field Description

-------------+--------------------------------------------------------------

|

00  |  File identification byte $A1.

01 to 16  |  Header information, identical to a cassette header.

01 to 09  |  Name of file (ASCII).

10  |  File type. 0 = BASIC

|             1 = Data*

|             2 = Machine code

11  |  BASIC tokenisation flag.

|             0 = Tokenised

|            -1 = ASCII*

12  |  Block style flag.

|             0 = Standard file.

|            -1 = Gapped BASIC file*,

|                 Gapped data file*,

|                 Distributed format machine code*

13 to 14  |  Execution address for standard machine machine code files*.

15 to 16  |  Load address for standard machine code files**.

17+  |  Raw program data.

|

-------------+--------------------------------------------------------------

*  Items or formats which are ignored or not supported for .DGN files.

** Load addresses are fixed for ROM files, but not for cartridge files.

DRAGDEB.EXE

===========

This debugging version of PC-Dragon provides a number of extra features not

included in the standard version in order to optimise speed.

Additional commands when running (response times may vary):

CTRL-B            -- Break into program.

CTRL-O            -- Output on/off.

Commands when stopped:

ESC               -- Exit loop (go until PCR increases).

CTRL-B            -- Go until breakpoint.

CTRL-D            -- Dump memory.

CTRL-O            -- Go until breakpoint, with output off.

CTRL-P            -- Deposit value in memory.

CTRL-R            -- Modify register.

CTRL-S            -- Set general breakpoint address.

CTRL-X            -- Exit from last BSR/JSR subroutine call.

other             -- Single step.

The debug display shows the last instruction executed. Above the opcode the

registers before execution are listed, and the register values after

execution are listed below. The format of the instruction display if as

follows:

1) OPCODE

2) POSTBYTES

3) MNEMONIC(S)

Where applicable: #$IMMEDIATE_DATA_HEX (#IMMEDIATE_DATA_DECIMAL)

$EFFECTIVE_ADDRESS

[$PREVIOUS_CONTENTS_OF_EFFECTIVE_ADDRESS]

An RTI instruction is shown as the appropriate PULS instructions; SWI is

displayed as PSHS followed by SWI.

The graphics display will not function properly with debug output on (and

vice-versa).

CLOADM.EXE

==========

Cloadm will attempt to extract a sampled Dragon cassette from a .WAV or .VOC

file and produce a .CAS virtual cassette file for use with PC-Dragon. All

types of file can be converted. The program may also be used to strip the

cassette formatting information from a .CAS file, converting to a raw data

file.

The filename of the file to be converted should be given as an argument to

cloadm, or else "DRAGON" will be assumed. If no extension is given .WAV,

.VOC and .CAS files are assumed.

Where the input is a .WAV or .VOC file, the output filename will be derived

from the Dragon filename, as detailed above. In the case of .CAS input files

the output will depend on the type of file being extracted. Machine code or

tokenised basic files are extracted to .DGN raw Dragon data files; ascii

basic files are extracted to .BAS Microsoft Qbasic files. Data files cannot

be converted.

During conversion cloadm will normally report on whether the data conforms to

the standard cassette format; identifying header, data and EOF blocks etc. and

stopping if a loading error is encounterred. However, if required, cloadm may

be instructed to continue despite any errors by giving the /E option on the

command line. This may be useful for any programs which are made up of several

files, or for files which contain unusual block types. The /E option is only

available when converting .WAV or .VOC files to .CAS files, and not when

extracting DOS files from .CAS files.

If the input file does not consist of standard cassette blocks and sync

characters, it may still be converted by specifying /R on the command line.

Cloadm will then scan the file as a raw bitstream and produce an output

file _.CAS; obviously when used in this way cloadm cannot report any loading

errors.

Where loading errors are found to be a problem, the following may help:

*   As the average sinewave wavelength is calculated at runtime by cloadm,

any sampling frequency may be used, but the highest available rate

should be chosen for best results.

*   Best results are obtained when the waveform amplitude comfortably

spans the available values. Try increasing the input volume

if necessary.

*   In addition, the volume sensitivity of cloadm may be specified on the

command line "/Sn" as the first argument, where 0 < n < 127.

*   It is a good idea to clean up the waveform before presenting it to

cloadm. Try cutting any white space, particularly before the first

program block, and removing any noisy parts of the leader.

*   The auto-detection of the average sinewave wavelength may be overriden

on the command line using "/Wn". For a 44.1KHz sample, 29-31 is about

right; for 22KHz the value should be in the range 12-14.

*   Windows software (Eg. Creative Wave Studio) seems to produce better

results than MS-DOS based software.

See the questions at the end of this file for further discussion.

A command line summary for cloadm is available by typing "CLOADM /?".

CSAVEM.EXE

==========

Csavem will produce a 44.1KHz .WAV sample file from any .CAS virtual cassette

file. The filename of the file to be converted should be given as an

argument to csavem, or else "DRAGON" will be assumed. If no extension is

given .CAS is assumed.

The output filename is obtained by simply substituting the .WAV extension

into the input filename.

Csavem does not check the format of the .CAS input file and therefore any

file type or format may be converted.

A command line summary for csavem is available by typing "CSAVEM /?".

See the 'Hints For Using Virtual Cassettes' section above.

How To Transfer Your ROM For Use With PC-Dragon

===============================================

There are several ways to do this. The method suggested here is to use the

CLOADM.EXE utility to transfer the ROM in the same way as other programs.

Some alternative methods are mentioned at the end of this section.

Here is the transfer procedure in full:

1)  Boot up your Dragon 32, Dragon 64 or Tandy CoCo.

2)  Ready your tape recorder for recording and commit the ROM to

cassette by typing:

CSAVEM "DRAGROM", &H8000, &HBFFF, 0

If you have a Tandy CoCo you may optionally name the files TANDYROM

rather than DRAGROM.

3)  Boot up your PC and Sound Blaster waveform editor software. Sample

the cassette recorded in step 2 into a file. A very large output file

should result.

4)  Now use the CLOADM.EXE utility described above to extract the cassette

data. Eg.

CLOADM DRAGROM.WAV

Please do not expect the conversion to work the first time. You are

likely to need to adjust the command line switches until you find a

combination which is suited to your hardware. Repeat step 3 if

necessary. Upon success the file DRAGROM.CAS should result.

5)  Now use CLOADM.EXE again to convert the formatted data to a raw file:

CLOADM DRAGROM.CAS

This should produce the file DRAGROM.DGN in the current directory.

6)  If you have a Dragon 64, you may optionally also wish to transfer the

64K version of the Dragon ROM from your machine; you will not be able

to use the emulator in 64K BASIC mode unless you do so. The procedure

to transfer the second ROM is much as before. Boot up your Dragon 64

and switch to 64K mode by typing:

EXEC 48000

7)  Now commit the second ROM to cassette by typing:

CSAVEM "D64ROM2", &HC000, &HFEFF, 0

8)  Transer this file to your PC by using the procedure described before

in steps 3 and 5. This time you should end up with the .DGN file

D64ROM2.DGN; again you should save this in your PC-Dragon directory.

If you already have your ROM image(s) in MS-DOS format, you can create the

required .DGN files from a raw image by using the included header file

ROM_STUB.DGN. You might, for instance, wish to transfer the ROM from your

Dragon to your PC on floppy disk if you are a disk user.

To convert a raw image to .DGN format simply type:

COPY /B   ROM_STUB.DGN + your_filename   DRAGROM.DGN

This method also works for D64ROM2.DGN, which may be either 3F00h bytes, or

the full 4000h bytes. Note: At the time of writing this feature has not been

tested - it may be necessary to use a D64ROM2 image containing the full

4000h bytes. Cassette support and the o/s breakpoint keyboard will definitely

NOT work in 64K mode.

Another alternative is to transfer the ROM using a 'bit banger' serial link

(for Tandy CoCo II or Dragon 64 owners). This method is described in the

documentation for CoCo2-12 (see later section).

If you have a ROM image in Jeff's .ROM format you can convert it to a raw

file using MS-DOS debug as follows:

DEBUG COCO.ROM

M 102 L 4000 100

W

Q

Following this, you should use the COPY command and the ROM_STUB as described

above.

Please be aware that the ROM software is copyrighted by Microsoft and should

not be distributed.

CAS2PAK.EXE

===========

Cas2Pak is a utility to translate software in the .CAS raw cassette data file

type used by PC-Dragon into the .PAK snapshot format used by CoCo2-12. Using

Cas2Pak you can therefore translate any software you have in the .CAS format

into a format which can be used with Jeffs emulator, or use the program as a

route for transferring tape software to CoCo2-12.

The procedure for transferring files is as follows:

1) Before you use Cas2Pak for the first time you must first perform a

simple patch to the ROM file that you use with CoCo2-12. The patch

hooks in Cas2Pak's cassette emulation routines. Only 4 bytes are

required, and may be added by using the MS-DOS debug facility as

follows (for Dragon users):

DEBUG DRAGON.ROM

E 3EE9

39

E 3EA7

7E C0 2E

W

Q

For the Tandy CoCo II ROM, the input should be:

DEBUG COCO.ROM

E 287E

39

E 2857

7E C0 2E

W

Q

(Press return after each of the above lines, and press the spacebar

once for each gap.)

If you are upgrading from an older version of Cas2Pak you should repeat

this procesure.

2) You can now convert your .CAS file to .PAK format using the syntax:

CAS2PAK <filename>

On success, one or more .PAK files of similar name to the input file are

produced in the same directory.

3) To load the program into CoCo2-12, simply run "COCO" as normal. When the

OK prompt appears load the first .PAK file by pressing F3 L <name>.

4) When the pakfile is activated the prompt returns, with the message:

CASSETTE READY

OK

*

Now simply load your program as normal using CLOADM, CLOAD or OPEN

as appropriate.

5) For large programs the screen will flash when the end of the first .PAK

file is reached. Simply continue by pressing F3 L again, and choosing

the next .PAK file.

6) Single volume cassettes may be rewound using 'EXEC &HC000' if required.

The embedded code generated by Cas2Pak is compatible with both Dragon and

Tandy CoCo ROMs.

File Transfer

=============

If you have a sound card with a suitable ADC and sampling software you should

be able to successfully transfer any Dragon cassette to PC-Dragon, so long

as the recording is of reasonable quality. The same method can also be used

to transfer files directly from a real Dragon's cassette port.

It is also possible to transfer data directly between the serial ports of

the PC and your Tandy CoCo II or Dragon 64, as outlined in the documentation

of CoCo2-12. I would imagine that transfer though the parallel ports should

be possible using a similar technique. Anyone tried this?

If you are a disk user, you should be able to transfer software by using

one of the shareware or freeware programs which can read Dragon or CoCo floppy

disks directly on PC drives.

For anyone who does not own suitable hardware to perform file transfers, I

will gladly attempt to transfer cassette software for a small fee. Simply

send a cassette containing the program that you want transferred to the

address given at the end of this document (cassette will be returned),

together with 4 (UK NDUG), 5 (UK), 10 (Europe, US) or 15 (anywhere else).

This charge is for a single program and it's loaders. Add 2.50 for each

additional program (UK), 4 (Europe, US) or 8 (anywhere else). In return I

will send you a 3.5" disk containing the appropriate .CAS files (together

with the latest version of the emulator). Naturally, If I am unsuccessful I

will refund your money.

Please only send reliable copies, and ensure that the software is suitable

for transfer. It must typically be possible to load the whole program into

memory at once - it must certainly be the case that the cassette never needs

to be rewound or turned over. Return Of The Ring, for example, is not really

suitable.

Also note that I can give no assurance that software successfully transferred

will function correctly with the emulator. Of course, you may wish to try

it with CoCo2-12 instead; or if you can figure out why software will not

perform, then there is always the possibility that any necessary changes can

be accomodated into a future version of PC-Dragon.

Please note the following points:

1. Machine code programs which read the joystick at the hardware level will

not function properly at present.

2. Software which uses semigraphics display modes, or fast mode switching

techniques will not be displayed correctly (see above section).

3. Any software that plays music or sound effects will suffer from long

pauses.

4. In all cases, expect the software to run no faster than 1/4 of the

correct speed, and even slower if your program is graphical (or your PC

is slower)!

If the software that you'd like transferred is a commercial Dragon title then

it may not be necessary for you to send the original cassette, since a vast

number of titles have already been converted to PC format. As this saves me

a lot of hassle I will supply such software very cheaply, but you must provide

some evidence that you already have the software. Please write first for

details if interested.

Other Dragon And Tandy Emulators

================================

Jeff Vavasour has written two excellent emulators which should interest users

of this program. Both are available from the SimTel mirror sites (eg.

oak.oakland.edu) in the emulators directory.

MODEL1-B.ZIP is an emulator for the Z80 based Tandy Model 1 computer of the

mid seventies. The program is shareware and requires a registration fee of

$25.

COCO2-12.ZIP is a Tandy CoCo II emulator, which also includes 64K Dragon

support. The program is freeware. It emulates a different range of features

to PC-Dragon, and also runs faster.

Some Questions And Answers

==========================

Q. I just can't get CLOADM.EXE to work. What can I do?

A. Try different cassette recorders, different cassettes, different volume

settings and different software. It might help to make a new, clean copy

of your software using a copying program and a new, clean cassette, before

sampling it. If you have a Sound Blaster you should try both the MIC and

Line-In inputs.

Quite a few netters have reported that they have had trouble using

CLOADM.EXE. However, if you have tried many volume settings and command

line switch combinations then there is nothing else that I can suggest.

For the best results, a newer sound card should be chosen (eg. SB2.0), or

SB-PRO2, which is what I use. It may also help to run disk caching

software, so that sampling is less adversely affected by hard disk writes.

I have a 2MB hardware disk cache.

Q. I can get almost all my software to load with CLOADM.EXE, but for one or

two programs it just won't work!

A. Yes, indeed some programs appear to be very stubborn. Rommels Revenge is

one such program, but it does work if you strip the autorun protection

from it and transfer it as a simple CLOADM/EXEC file.

Q. My sampling software uses a different file type to .VOC and .WAV; could

you add support for .XXX please?

A. You could try the excellent SOX7DOS.ZIP file conversion package. That is

a simpler option.

Q. How about emulating disk drives?

A. Sorry, I've never owned a Dragon disk drive, so I don't know enough about

them. However, I warmly welcome any other comments or suggestions for new

features.

Q. Do I really need VGA to display such low resolution graphics?

A. Yes, I'm sorry about that. But you can run PC-Dragon on any graphics

card, including Hercules Monochrome, if you avoid switching the emulated

machine into graphics mode.

=========================================================

+++++++++++++++++  Freeware Notice  +++++++++++++++++

=========================================================

This program is released as freeware. The usual conditions and restrictions

apply. The software is supplied 'as is' with no guarantee given against

anti-social behaviour. The author hereby disclaims all responsibility

for losses incurred through the use of this program. You should be aware that

by using the program you are deemed to have accepted these conditions of use.

You may copy and distribute the program freely so long as the software and

documentation remain unchanged. There is no subscription or other fee for

using this software. If the program is obtained from a distribution service

no additional charge may be made for PC-Dragon. PC-Dragon cannot be used

commercially without prior arrangement with the author.

However, if you like PC-Dragon and would like to support further development,

a gift of any amount would be warmly welcomed by this humble student at the

address given below. Failing this, please show your support by sending a

postcard from your home town to the address given below.

I'm also pleased to receive any comments or suggestions regarding PC-Dragon.

Please contact:

Paul Burgin

18 Moorcroft Road

Sheffield

S10 4GS

England

I can also be contacted by e-mail up to June 1994, at pdb6@uk.ac.york.vax or

pdb-a@uk.ac.york.minster.

If you would like to obtain the latest version of PC-Dragon (3.5" disk only),

simply send your name and address together with a UK cheque or IMO for 2

(UK), 5 (Europe, US) or 10 (anywhere else) to the above address.

Please note that this is a 'cost price' offer only and not a shareware

'registration' - you will receive nothing more than the latest version of

PC-Dragon. The standard disclaimer and conditions of use will still apply.

The conditions and prices for all disk offers may be revised or withdrawn at

any time. Please check the document date and write first to confirm details

if in doubt.

The National Dragon Users Group

===============================

If you're keen on the Dragon or Tandy CoCo then why not help to keep them

alive and keep in touch with other users by joining NDUG? Gain access to a

panel of expert volunteers on the helpline, read all the latest news, views

and reviews in the bi-monthly newsletter, or even better why not write some

articles yourself?

A years membership is about 8 for the UK, and new members from either

platform are always very welcome. If you would like details on how to join,

or any further information, then please drop me a line.

Acknowledgements

================

Thanks to Lee Hamel for the keyboard interrupt code.

Thanks also to Jeff Vavasour, Graham Kinns and Keiran Anscomb for your

support.

Legal Nuggets

=============

Dragon, Dragon 32 and Dragon 64 are trademarks of Eurohard S.A. :)

Tandy CoCo is a trademark of Tandy Computers Ltd

Acorn and Archimedes are trademarks of Acorn Computing PLC

MS-DOS, MS-Windows, Microsoft and Qbasic are trademarks of Microsoft Corp

IBM is a trademark of International Business Machines Ltd

Sound Blaster and Sound Blaster Pro are a trademarks of Western Creative Ltd

PKLite is a product and trademark of PKWare Inc