# Q.1.	Write short notes on the following types of memories:

Ans.

Smart Card:

A smart card is a small electronic device about the size of a credit card that contains electronic memory, and possibly an embedded integrated circuit (IC). Smart cards containing an IC are sometimes called Integrated Circuit Cards (ICCs). Defined at its highest level, a smart card is a credit-card sized plastic card with an embedded computer chip. The chip can either be a microprocessor with internal memory or a memory chip with non-programmable logic. The chip connection is either via direct physical contact or remotely via a contactless electromagnetic interface.

There are two general categories of smart cards:

- Contact Smart Cards

## Contactless Smart Cards

Contact Smart Cards:

A contact smart card requires insertion into a smart card reader with a direct connection to a conductive micro module on the surface of the card (typically gold plated). It is via these physical contact points, that transmission of commands, data, and card status takes place.

The above diagram shows the micro module embedded into the plastic substrate or card.  Prior to embedding, a cavity is formed or milled into the plastic card.  Then either a cold or hot glue process bonds the micro module to the card.

Below is a contact micro module that is embedded into a plastic substrate

Contactless Smart Cards:

A contactless card requires only close proximity to a reader. Both the reader and the card have antenna and it is via this contactless link that the two communicate. Most contactless cards also derive the internal chip power source from this electromagnetic signal. The range is typically two to three inches for non-battery powered cards, and this is ideal for applications such as mass transit that require very fast card interface.

This diagram shows the top and bottom card layers which sandwich the antenna/chip module.  The antenna is typically 3 - 5 turns of very thin wire (or conductive ink), connected to the contactless chip.

The chips used in all of these cards fall into two categories as well: microprocessor chips and memory chips. A memory chip can be viewed as small floppy disks with optional security. Memory cards can hold from 103 bits to 16,000 bits of data. They are less expensive than microprocessor cards but with a corresponding decrease in data management security. They depend on the security of the card reader for their processing and are ideal when security requirements permit use of cards with low to medium security.

A microprocessor chip can add, delete and otherwise manipulate information in its memory. It can be viewed as a miniature computer with an input/output port, operating system and hard disk. Microprocessor chips are available 8, 16, and 32 bit architectures. Their data storage capacity ranges from 300 bytes to 32,000 bytes with larger sizes expected with semiconductor technology advances. Their ability to download not just data but applications is being advanced by Sun with Java Card technology and Mondex with Multos.

Standards

The basic smart card standard is the ISO 7816 series, part 1-10. These standards are derived from the financial ID card standards and detail the physical, electrical, mechanical, and application programming interface to a contact chip card.

Applications

Listed below are some of the major applications seen around the world.

There are over 300,000,000 GSM mobile telephones with smart cards that contain the mobile phone security and subscription information. The handset is personalized to the individual by inserting the card that contains its phone number on the network, billing information, and frequently call numbers.

Almost every small dish TV satellite receiver uses a smart card as its removable security element and subscription information. There are over 4 million in the US alone between DirectTV, USSB and Echo Star. There are millions more in Europe and Asia.

The Financial industry has been quick to adopt smart card technology in various countries around the world. Every French Visa Debit card (over 25,000,000) has a chip in it. In Germany, about 40,000,000 banking cards have been issued. EuroPay, MasterCard, and Visa all have smart card programs for their bank members. In the Portugal and Singapore, the national banking networks have launched electronic purse projects. Proton has worked with its banking partners to issued over 25,000,000 electronic purse cards in several countries.

Other applications for smart cards include computer/internet user authentication and non-repudiation, retailer loyalty programs, physical access, resort cards, mass transit, electronic toll, product tracking, national ID, drivers license, pass ports, and the list goes on.

PC Card:

A computer device packaged in a small card about the size of a credit card and conforming to the PCMCIA standard.

In the early 90’s, the rapid growth of mobile computing drove the development of smaller, lighter, and more portable tools for information processing. One of the most exciting of these innovations was PC Card technology. The power and versatility of PC Cards quickly made them standard equipment in mobile computers. The rapid development and worldwide adoption of PC Card technology has been due in large part to the standards efforts of the Personal Computer Memory Card International Association (PCMCIA).

The association’s PC Card Standard is now bringing the benefits of these same PC Cards to a variety of industries and vertical applications, including smart cards, set-top boxes, automobiles, and others. The PC Card technology’s compact size and ruggedness make it the ideal technology for a wide variety of applications.

The PC Card Standard provides physical specifications for three types of PC Cards, with additional provisions for extended cards. All three card types measure the same length and width and use the same 68-pin connector. The only difference between the card types is thickness. The thicknesses are 3.3, 5.0, and 10.5 millimeters for Type I, Type II, and Type III cards respectively. Because they differ only in thickness, a thinner card can be used in a thicker slot, but a thicker card cannot be used in a thinner slot.

The card types each have features that fit the needs of different applications. Type I PC Cards are typically used for memory devices such as RAM, Flash, OTP, and SRAM cards. Type II PC Cards are typically used for I/O devices such as data/fax modems, LANs, and mass storage devices. Type III PC Cards are used for devices whose components are thicker, such as rotating mass storage devices. Extended cards allow the addition of components that must remain outside the system for proper operation, such as antennas for wireless applications.

The rapid rate of adoption of PC Card slots has driven a steady stream of card and host implementations. During that time, PC Cards containing new technologies were introduced and significant new capabilities were added to the Standard. At the same time considerable experience was gained by card, host, and software vendors, and opportunities to improve compatibility were recognized.

PCMCIA’s goal is to make the technology as easy to use as possible, however, the Standard can only provide guidelines in some areas so there will be manufacturers who do not follow the Standard exactly or have interpreted it differently. Therefore, development planned for flexibility and adaptability will allow for the greatest compatibility. One way to be prepared for the variety of the real world is to perform exhaustive testing of designs with all the significant components from software functions and modules to entire platforms.

### Features of PC Card Technology

Card Information Structure: The CIS describes the characteristics and capabilities of the card so the host system can automatically configure it. CIS is defined in the Metaformat Specification.

CardBus: CardBus allows PC Cards and hosts to use 32-bit busmastering and to operate at speeds up to 33MHz.

DMA:  The Standard allows cards to utilize Direct Memory Access technology directly in the hardware when matched with a corresponding host system.

eXecute In Place (XIP): Allows operating system and application software to run directly from the PC Card, eliminating the need for large amounts of system RAM.

Low Voltage Operation: The Standard enables 3.3 and 5-volt operation. A physical keying mechanism for 3.3-volt cards protects them from being damaged in a 5-volt slot.

Multiple Function PC Cards:  The Standard enables truly standardized multiple function PC-Cards.

Plug and Play: PC Cards can be inserted or removed while the system is on because power and ground contact pins are the longest contacts, ensuring that disconnect signals disengage first, preserving data integrity.

Power Management: The Standard provides a means to interface to APM (Advanced Power Management) through the Card Services Specification.

Zoomed Video (ZV): Zoomed Video is a connection between a PC Card and host system that allows the card to write video data directly to the VGA controller. The data is transferred with no buffering requirements because it is transferred over the ZV bus and not the system bus.

| PC Card Physical Characteristics |  |
| --- | --- |
| Physical Interface | 68 Pins |
| Back End I/O Conn. | Proprietary * |
| Length | 85.6 mm |
| Width | 54.0 mm |
| Thickness | Type I = 3.3 mm Type II = 5.0 mm Type III = 10.5 mm |
| Operating Temp. | 0 to 55 C |
| Storage Temp. | -20 to 65 C |
| Minimum Insertions | Office Env. 10,000 Harsh Env. 5,000 |

* Two standardized connectors are available as part of the optional PCMCIA Specific Extensions Specifications.

Small Computer Systems Interface (SCSI):

Abbreviation of Small Computer System Interface. Pronounced "scuzzy," SCSI is a parallel interface standard used by Apple Macintosh computers, PCs, and many UNIX systems for attaching peripheral devices to computers. Nearly all Apple Macintosh computers, excluding only the earliest Macs and the recent iMac, come with a SCSI port for attaching devices such as disk drives and printers.

SCSI interfaces provide for faster data transmission rates (up to 80 megabytes per second) than standard serial and parallel ports. In addition, you can attach many devices to a single SCSI port, so that SCSI is really an I/O bus rather than simply an interface.

SCSI's most obvious strength is the number of devices it can control. Where IDE interfaces were restricted to two disk drives and today's EIDE interfaces are restricted to four devices (including hard disks and CD-ROM drives), a single SCSI controller can handle up to eight devices (including the host adapter card, which counts as a device). Furthermore, the devices can vary from hard disks and CD-ROM drives to compact disk-recordable units, scanners, printers, optical drives, and more. High-end servers frequently use a SCSI interface exclusively, in large part because of its easy expandability. But the exclusivity is unnecessary. IDE/EIDE and SCSI interfaces can be freely combined inside a PC, offering expanded possibilities for hardware configuration. In PCs, the SCSI host adapter takes up a hardware interrupt (IRQ), but the devices attached to the card do not, which significantly increases expandability. In fact, it's possible to add a second SCSI card for 7 additional devices. Better still, a "twin-channel" SCSI card takes up only one IRQ and handles 15 peripheral devices.

Although it's considered a single standard, SCSI actually comes in a variety of types. Among these are SCSI, SCSI-2, SCSI-3, Fast SCSI, Fast/Wide SCSI, Ultra SCSI, Wide Ultra SCSI, Ultra2 SCSI, Wide Ultra2 SCSI, and more. The latest specification is SCSI-3.

Interfaces:

Low Voltage Differential (LVD)
Introduced with the Ultra2 SCSI, LVD now increases data transfer rates to 160Mbytes/sec, and maintains the cable length at 12 meters.

Single-Ended SCSI
Supports transfer rates to 40 Mbytes/sec and cable lengths to 3 meters.

High Voltage Differential (HVD)
Supports data transfer rates to 40 Mbytes/sec and cable lengths to 25 meters. However HVD cards and disk drives are expensive and seldom used (less than 5% of SCSI disk drives).

Summary of SCSI Transfer Modes and Protocols

The table below provides a quick summary of the different SCSI interface protocols and their main characteristics

| Interface Protocol | Defining Standard | Bus Speed (MHz) | Bus Width (bits) | Transfer Rate (MB/s) | Cabling | Maximum Single-Ended Cable Length (m) | Number of Devices Per Bus |
| --- | --- | --- | --- | --- | --- | --- | --- |
| "Regular" SCSI | SCSI-1 | 5 | 8 | 4.77 | "A" or "P" | 6 | 8 |
| Wide SCSI | SCSI-2 | 5 | 16 | 9.54 | "A"+"B" or "P" (or SCA-2) | 6 | 16 |
| Fast SCSI | SCSI-2 | 10 | 8 | 9.54 | "A" or "P" | 3 | 8 |
| Fast Wide SCSI | SCSI-2 | 10 | 16 | 19.07 | "A"+"B" or "P" (or SCA-2) | 3 | 16 |
| Ultra SCSI | SCSI-3 | 20 | 8 | 19.07 | "A" or "P" | 1.5 | 8 |
| Ultra Wide SCSI | SCSI-3 | 20 | 16 | 38.15 | "A"+"B" or "P" (or SCA-2) | 1.5 | 16 |
| Serial SCSI (Firewire) | SCSI-3 | 100 to 400+ | 1 | 12 to 50+ !? | 6-pin Firewire Cable? | !? | !? |

Here's the general rule for determining the maximum usable cable length for each type of SCSI.

| Type | Cable Length |
| --- | --- |
| SCSI-1 | 6 meters (19 feet) |
| SCSI-2 (Fast SCSI) | 3 meters (9 feet) |
| Ultra SCSI | 1.5 meters (4.5 feet) |
| Wide Ultra SCSI | 1.5 meters for greater than 4 devices 3 meters for less than 3 devices |
| Ultra2 SCSI Ultra160 SCSI | 12 meters (39 feet) |

Q.2. Briefly explain the steps how do the following peripherals work.

Ans.

- A Floppy Disk Drive:

Floppy drives are notable for the use of open loop tracking: they don’t actually look for the tracks, but simply order the head to move to the ‘correct’ position. Hard disk drives, on the other hand, have servo motors embedded on the disk which the head uses to verify its position, allowing track densities many hundred times higher than is possible on a floppy disk.

When a 3.5in disk is inserted, the protective metal slider is pushed aside and a magnet locks onto the disk’s metal center plate. The drive spindle goes in the center hole, and the drive pin engages the rectangular positioning hole next to this. The drive pin is spun by the drive motor, a flat DC servo device locked to a spin of 300rpm by a servo frequency generator.

The head is moved by a lead-screw which in turn is driven by a stepper motor; when the motor and screw rotate through a set angle, the head moves a fixed distance. The data density of floppies is therefore governed by the accuracy of the stepper motor, which means 135 tracks per inch (tpi) for 1.44MB disks. A drive has four sensors: disk motor; write-protect; disk present; and a track 00 sensor, which is basically an edge stop.

The magnetic head has a ferrite core, with the read/write head in the center and an erase head core on either side. The erase heads wipe out a strip either side of a new data track to avoid interference from old data tracks. Bits are stored as magnetic inversions, the inversion intervals being two to four microseconds (µs). The read signal goes to a peak detector and is processed to produce a binary signal by the electronics in the drive. This is then sent back to the PC.

Most floppy drives have two read/write heads, meaning that it is double-sided. These heads are used to read and write data to the diskette. Like a hard drive, the head mechanism is moved by the head actuator. It is a stepper motor design, much like the old hard drives. This little motor moves the heads in and out, giving them the ability to position themselves over any track on the disk. The heads use a recording method known as tunnel erasure. Basically, this recording method is a way to keep each track of data separate from the others. As the head writes the data to a track, the tunnel erase heads come along and erase the outer edges of the track, thereby creating a sharp-edged track, very distinct from the others. This keeps the data on one track from being confused with data from another track, thereby eliminating problems.

The heads are spring-loaded; therefore, they are physically contacting the disk while they are reading and writing. The drive spins the disk at about 300RPM; therefore this contact is not a problem to the data because friction is minimum. Some diskette makers coat the disk with Teflon, further reducing friction between the heads and the disk. Although a good idea, eventually a build-up of Teflon will form on the heads, requiring us clean them.

The spindle motor spins the disk. As said before, it spins the disk at 300 RPM. On old 5.25" drives, the disk is spun at 360 RPM. Older drives had the spindle motor attached to a belt system that spun the disks. These were not very reliable, and the RPM was not constant depending on the disk. Today's drives use a direct-drive system with no belts. This system has automatic torque compensation so that sticky disks are spun with greater force than a slippery one, thereby maintaining a constant 300-RPM with all disks.

The newer drives have this automatic ability. The older ones require periodic adjusting. This is done using little strobe marks that are on the motor. We run the drive in fluorescent light and adjust the RPM until the marks look still, just like a car rim or wagon wheel when turned fast enough.

b) A laser reads data on a compact disc:

Apart from far more sophisticated error-checking techniques, the innards of a CD-ROM drive are pretty much the same as those used in CD audio players. Data is stored in the same way on all CDs. The information is stored in sequential 2KB sectors that form a single spiral track that starts at the center of the disc and wraps around many times until it reaches the outer edge of the disc.

A player reads information from the CD's spiral track of pits and lands, starting from the center of the disc and moving to the outer edge. It does this by firing an infrared laser - 780 nano-millimetres wide and generated by a small gallium arsenate semiconductor - through the clear optical grade polycarbonate plastic layer and onto the metallic sheet. Although it’s of very low power, it’s strong enough to damage the eye if shined directly into it. As the disc rotates at between 200 and 500rpm, the light bounces off the pits and the frequency of the light changes.

The areas around the pits, called lands, also play a part in the process. The reflected light passes through a prism and onto a photosensor, the output of which is proportional to the amount of light it receives. Light reflected from a pit is 180 degrees out of phase with the light from the lands, and the differences in intensity are measured by the photoelectric cells and converted into electrical pulses. The result is that the series of pits and lands of varying lengths stamped into the surface of the disc are interpreted as a series of corresponding 0s and 1s from which the data - or, via a digital-to-analogue converter (DAC), the audio - stored on the disc is recreated. And since just a weak bandwidth laser is the only thing to touch the surface of the CD directly, there’s none of the wear and tear of traditional analogue media to contend with.

b) A Hard Disk Drive:

Hard disks are rigid platters, composed of a substrate and a magnetic medium. The substrate - the platter's base material - must be non-magnetic and capable of being machined to a smooth finish. It is made either of aluminum alloy or a mixture of glass and ceramic. To allow data storage, both sides of each platter are coated with a magnetic medium - formerly magnetic oxide, but now, almost exclusively, a layer of metal called a thin-film medium. This stores data in magnetic patterns, with each platter capable of storing a billion or so bits per square inch (bpsi) of platter surface.

Platters vary in size and hard disk drives come in two form factors, 5.25in or 3.5in. The trend is towards glass technology since this has the better heat resistance properties and allows platters to be made thinner than aluminum ones. The inside of a hard disk drive must be kept as dust-free as the factory where it was built. To eliminate internal contamination, air pressure is equalized via special filters and the platters are hermetically sealed in a case with the interior kept in a partial vacuum. This sealed chamber is often referred to as the head disk assembly (HDA).

Typically two or three or more platters are stacked on top of each other with a common spindle that turns the whole assembly at several thousand revolutions per minute. There’s a gap between the platters, making room for magnetic read/write head, mounted on the end of an actuator arm. This is so close to the platters that it’s only the rush of air pulled round by the rotation of the platters that keeps the head away from the surface of the disk - it flies a fraction of a millimeter above the disk. On early hard disk drives this distance was around 0.2mm. In modern-day drives this has been reduced to 0.07mm or less. A small particle of dirt could cause a head to ‘crash’, touching the disk and scraping off the magnetic coating. On IDE and SCSI drives the disk controller is part of the drive itself.

There’s a read/write head for each side of each platter, mounted on arms that can move them towards the central spindle or towards the edge. The arms are moved by the head actuator, which contains a voice-coil - an electromagnetic coil that can move a magnet very rapidly. Loudspeaker cones are vibrated using a similar mechanism.

The heads are designed to touch the platters when the disk stops spinning - that is, when the drive is powered off. During the spin-down period, the airflow diminishes until it stops completely, when the head lands gently on the platter surface - to a dedicated spot called the landing zone (LZ). The LZ is dedicated to providing a parking spot for the read/write heads, and never contains data.

When a disk undergoes a low-level format, it is divided it into tracks and sectors. The tracks are concentric circles around the central spindle on either side of each platter. Tracks physically above each other on the platters are grouped together into cylinders that are then further subdivided into sectors of 512 bytes apiece. The concept of cylinders is important, since cross-platter information in the same cylinder can be accessed without having to move the heads. The sector is a disk's smallest accessible unit. Drives use a technique called zoned-bit recording in which tracks on the outside of the disk contain more sectors than those on the inside.

Data is recorded onto the magnetic surface of the disk in exactly the same way as it is on floppies or digital tapes. Essentially, the surface is treated as an array of dot positions, with each ‘domain’ of magnetic polarization being set to a binary ‘1’ or ‘0’. The position of each array element is not identifiable in an ‘absolute’ sense, and so a scheme of guidance marks helps the read/write head find positions on the disk. The need for these guidance markings explains why disks must be formatted before they can be used.

When it comes to accessing data already stored, the disk spins round very fast so that any part of its circumference can be quickly identified. The drive translates a read request from the computer into reality. There was a time when the cylinder/head/sector location that the computer worked out really was the data’s location, but today’s drives are more complicated than the BIOS can handle, and they translate BIOS requests by using their own mapping.

Allocating and tracking individual data sectors on a large drive would require a huge amount of overhead, causing file handling efficiency to plummet. Therefore, to improve performance, data sectors are allocated in groups called clusters. The number of sectors in a cluster depends on the cluster size, which in turn depends on the partition size. The table below indicates the possibilities supported by the Windows 98 FAT32 file system:

| Partition size | FAT32 cluster size |
| --- | --- |
| 3GB - 7GB | 4KB |
| 8GB - 16GB | 8KB |
| 16GB - 32GB | 16KB |
| Larger than 32GB | 32KB |

When the computer wants to read data, the operating system works out where the data is on the disk. To do this it first reads the FAT (file allocation table) at the beginning of the partition. This tells the operating system in which sector on which track to find the data. With this information, the head can then read the requested data. The disk controller controls the drive's servomotors and translates the fluctuating voltages from the head into digital data for the CPU.

More often than not, the next set of data to be read is sequentially located on the disk. For this reason, hard drives contain between 64KB and 1MB of cache buffer in which to store all the information in a sector or cylinder in case it’s needed. This is very effective in speeding up both throughput and access times. A hard drive also requires servo information, which provides a continuous update on the location of the heads. This can be stored on a separate platter, or it can be intermingled with the actual data on all the platters. A separate servo platter is more expensive, but it speeds up access times, since the data heads won’t need to waste any time sending servo information.

However, the servo and data platters can get out of alignment due to changes in temperature. To prevent this, the drive constantly rechecks itself in a process called thermal recalibration. During multimedia playback this can cause sudden pauses in data transfer, resulting in stuttered audio and dropped video frames. Where the servo information is stored on the data platters, thermal recalibration isn’t required. For this reason the majority of drives embed the servo information with the data.

The drive is connected to the computer using one of a few possible interfaces. MFM and RLL are now obsolete, IDE is somewhat antiquated and has a problem in coping with the big modern (gigabyte-plus) hard disks and today’s drives use either EIDE or SCSI.

Q.3. Write a list of ten considerations for.

a) Purchasing a Printer:

Following are the factors that should be considered while purchasing a printer.

    - Type (Dot Matrix, Laser, Inkjet etc)

    - Resolution

    - Memory

    - Interface

    - Price

    - Cartridge/Toner Life

    - Paper-Handling Requirements

    - Performance Requirements

    - Extra Features (Scanner, Copier add-ons etc)

    - Warranty/After Sale Service

b) Purchasing a laptop computer:

Following factors should be considered when purchasing a laptop computer:

- Weight

- Display

- Memory

- Hard Drive

- Battery

- Input Device

- CPU

- Ports, Slots, and Bays

- Comfort

- Durability

c) Maintaining a PC:

Following points are necessary for maintaining a PC:

- Regular Cleaning

- Proper Maintenance

- Software Upgrades

- Hardware Upgrades

- Defragmentation

- Periodic Backups

- Periodic Antivirus Checks

- Protection from dust etc.

- Deleting unnecessary files

- Proper Security measures

Q.4. Briefly explain how do the following mechanisms work in security.

Ans.

- Biometric Pens:

Biometric Pens are used in signature verification systems. Signature verification systems have one major thing going for them: public acceptance. On everything from the Declaration of Independence to a credit card slip, people tend to accept a person's signature as proof of identity. Actually, signature recognition systems, also called dynamic signature verification systems, go far beyond simply looking at the shape of a signature: They measure both the distinguishing features of the signature and the distinguishing features of the process of signing. These features include pen pressure, speed, and the points at which the pen is lifted from the paper. These behavioral patterns are captured through a specially designed pen or tablet (or both) and compared with a template of process patterns.

The problem is that our signatures vary significantly over time and from one instance to another, so strong accuracy requires multiple samples and an extended verification process.

- Fingerprint:

This system--which consists of a hardware scanner and recognition software--records specific fingerprint characteristics, saves each user's data in a template, and then refers to the templates when the user next tries to gain access. Fingerprint systems are accurate, but they can be affected by changes in the fingerprint (burns, scars, and so on) and by dirt and other factors that distort the image.

The one-time registration of an individual's fingerprint takes only minutes. A tiny camera in the Fingerprint Identification Reader captures an image of the fingerprint and uses complex algorithms to convert the image into a unique "map" of minutiae points. This map of minutiae points (not the actual fingerprint) is then encrypted and stored within the network. Individuals need not be concerned about personal privacy issues as the fingerprint is not stored and cannot be recreated from the minutiae points.

c) Keystroke Analysis Design:

Instead of replacing the user name/password login system, this technology--called keystroke dynamics--works in conjunction with the login information. When you type your user name and password, the software measures your typing rhythm and compares it with your profile.

Keyboard dynamics technology measures dwell time, the length of time you hold down each key, as well as flight time, the time it takes you to move between keys. Taken over the course of several login sessions, these two metrics produce a measurement of rhythm unique to each user.

d) Retinal Scanning:

Probably the single most secure of all, these biometric systems work with the retina, the layer of blood vessels located at the back of the eye. The retinal image is difficult to capture, and during enrollment the user must focus on a point while holding very still so the camera can perform the capture properly. The only thing that is actually determined is the pattern of the blood vessels, but since this pattern is unique in each person, identification can be precise.