Discuss the internal structure of bus.

# Bus Types

## Bus lines are divided into two types.

- Dedicated

- Multiplexed

# Dedicated Bus Line

Dedicated bus line is permanently assigned either to one function or a physical subset of computer components.

An example of functional dedication is the use of separate dedicated address and data lines, which is common to many buses. However it is not essential. For example, address and data information may be transmitted over the same set of lines using an valid control line. At the beginning of data transfer, the data transfer, the address is placed on the bus and the address valid line is activated. At this point, each module has a specified period of time to copy the address and determine if it is the addressed module. This address is then removed from the bus the same bus connections are used for the subsequent read or write data transfer.

# Multiplexed Bus Line

The above method of using the same lines for multiple purposes is known as time multiplexing. The advantage of time multiplexing is the use of fewer lines, which saves space and usually cost. The disadvantage is that more complex circuitry within each module. Also there is a potential reduction in performance since certain events that share the same lines cannot take place in parallel.

Physical dedication refers o the use of multiple buses, each of which connects only a subset modules. A typical example is the use of an I/o bus to interconnect all I/O modules; this bus is then connected to the main bus through some type of I/O adapter module. The potential advantage of physical action is high throughput, because there is less bus connection. A disadvantage is the increased size and cost of the system.

# Method of Arbitration

In all but the simplest systems, more than one module may need control of the bus. For example, an I/O module may need to read or write directly to memory without sending the data to the CPU since only one unit at time can successfully transmit over the bus, some method of arbitration is needed. The various methods can roughly be classified as being centralized or distributed.

In a centralized scheme, a single hardware device, referred to as a bus controller or an arbiter is responsible for allocating time on the bus. The device may be separate module or pat of the CPU. In a distributed scheme, there is a no central controller. Rather, each module contains access control logic and the modules act together to share the bus. With both methods of arbitration, the purpose is to designate one device, either the CPU or an I/O module, as master. The master may then initiate the data transfer (e.g., read or write) with some other device, which act as slave for this particular exchange.

# Timing

Timing refers to the way in which events are coordinated on the bus. With Synchronous Timing, a clock determines the occurrence of events on the bus. The bus includes a clock line upon which a clock transmits a regular sequence of alternating 1s and 0s of equal duration. A single 1 – 0 transmission is referred to as a clock cycle or bus cycle and defines a time slot. All other devices on the bus read clock line and all events start at the beginning of a clock cycle. Other bus signals may change at the leading edge of the clock signal. Most events occupy a single clock cycle.

With Asynchronous Timing, the occurrence of one event on bus follows and depends on the occurrence of a previous event.

Synchronous Timing is simpler to implement and test. However, it is less flexible than Asynchronous timing. Because all devices on a Synchronous bus are tied to a fixed clock rate, the system cannot take advantage of advances in device performance. With Asynchronous Timing a mixture of slow and fast devices using older and newer technology, can share bus.

# Bus Width

The bus width of the data bus has an impact on system performance: the wider the data bus, the greater the number of bits transferred at one time. The width of the address bus has an impact on system capacity: the wider the bus address bus, the greater the range of locations that can be referenced.

# Data Transfer Type

# A bus supports various data transfer types. All buses support both write and read transfers. In the case of a multiplexed address / data bus, the bus is first used for specifying the address and then for transferring data. For a read operation there is typically a wait while the data is being fetched from the slave to be put on the bus. For either a read or write there may also be delay if it necessary to go through arbitration to gain control of the bus for the remainder of the operation (i.e., seize the bus to request a read or write, then seize the bus again to perform a read or write).

In the case of dedicated address and data buses, the address is put on the address bus remains there, while the data are put on the data bus. For write operation master the data on to data bus as soon as the address has stabilized and the slave has had the opportunity to recognize its address. For a read operation the slave puts the data onto the data bus as soon as it has recognized its address and has fetched the data.

There are also several combination operations that some buses allow. A Read Modify Write operation is simply a read followed immediately by a write to the same address. The address is only broadcast once at the beginning of the operation. The whole operation is typically indivisible in order to prevent any access the data element by other potential bus masters. The principle purpose of this capability is to protect shred memory resources in a multiprogramming system.

Read-After-Write is an indivisible operation consisting of a write followed immediately by a read from the same address. The read operation may be performed for checking purposes.

Some bus systems also support a Block Data transfer. In this case one address cycle is followed by n data cycles. The first data item is transferred to or from the specified address; the remaining data items are transferred to or from subsequent addresses.

Execution Unit, Bus Interphase Unit

Explain the functions of following registers in detail.

AX.BX.CX.DX, SP, BP, SI, DI, FR, PC, IR, MAR, MBR, SS, DS, SS, AND ES.

# The Bus Interphase Unit

The major parts of the BIU are the four-byte instruction queue, the segment registers (ES, CS and DS), the instruction pointer (IP) and the address-summing block (). The internal data buses and the Q bus interconnect the BIU and the EU.

# Instruction Queue

The instruction queue increases the average speed with which a program is executed by storing up to four prefetched instructions.

# Segment Registers

The segment registers (CS, DS and ES) are all 16-bit registers used for addressing the 1 Mbytes (1,048,576 bytes) of memory space.

The memory space is divided into groups of 64-Kbytes (65,536-bytes) segments. The program assigns every segment a base address, which is its starting location in the memory space. The segment registers point to four currently addressable segments. These registers can be changed by the program to point to other desired segments.

Currently addressable memory segments are those defined by the base addresses contained in CS (code segment) register, the DS (data segment) register, the SS (stack segment) register, and the ES  (extra segment) register.

# Instruction Pointer and Address Summing Block

The 16-bit IP is analogous to the program counter in other

microprocessors; it points to the next instruction in memory. The IP contains the offset address of the next instruction, which is the distance in bytes from the beginning or base address of the current code segment.

To achieve the 20-bit physical address that goes out on the address bus, the 16-bit offset address in the IP is added to the segment base address which has been shifted four bits to the left.

Execution Unit					Bus Interphase Unit

Address bus (20 bits)

General Registers



| AH | AL |
| --- | --- |
| BH | BL #### CS DS SS ES IP ### Internal Communications Registers |
| CH | CL |
| DH | DL |
| SP |  |
| BP |  |
| DI |  |
| SI |  |

ALU data bus (16-bits)

Temporary registers

Bus instruction Queue

ALU

1     2     3    4

Q bus (8-bits)

### EU

Control

System

### Flags

The internal organization of the 8088 microprocessor.

# Execution Unit

The EU decodes instructions fetched by the BIU, generates appropriate control signals, and executes the instructions. The main parts of the EU are the arithmetic logic unit, the general registers and the flags.

# The ALU

This unit does all the computational and logic operations, working with either 8-bit or 16-bit operands.

# The General Registers

This set of 16-bit registers is divided into two sets of four registers. One set consists of the data registers and the other set consists of the pointer and index registers.

Each of the 16-bit data registers has two separately accessible 8-bit sections and can be used either as a 16-bit register or as two 8-bit registers. The pointer and index registers are used only as 16-bit registers.

The low order bytes of the data registers are designated as AL, BL, CL and DL. The high order bytes are designated as AH, BH, CH and DH. These registers can be used in most arithmetic and logic operations in any manner specified by the programmer for storing data prior to and after processing. Also some of these registers are used specially by certain program instructions.

The pointer and index registers are the stack pointer (SP), the base pointer (BP), the source index (SI) and the destination index (DI). These registers are used in various forms of memory addressing under control of the EU

15	        8  7            0

Accumulator

##### Base

Count

Data

| AH | AL |
| --- | --- |
| BH #### Data set | BL |
| CH | CL |
| DH | DL |

15	                          0

Stack pointer

Base pointer

Source index

Destination index

SP #### Pointer #### & #### Index set

BP

SI

DI

# The Flags

The flag register contains the nine independent status and control bits. A status flag is one bit indicator and is used to reflect a certain condition after an arithmetic or logic operation by the ALU, such as carry (CF), a zero result (ZF), or the sign result (SF) among others. The control flags are used to alter processor operations in certain situations.

| TF | DF | IF | OF | SF | ZP | AF | PF | CF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Carry

Parity

Aux carry

Zero

Sign

Overflow

Interrupt enable

Direction

Trap