1.	Introduction

- The hallmark of a von Neumann machine is a large random-access memory. Each cell in the memory has a unique numerical address, which can be used to access or replace the contents of that cell in a single step. In addition to its ability to address memory locations directly, a von Neumann machine also has a central processing unit (the CPU) that possesses a special working memory (register memory) for holding data. Data that are being operated on and a set of built-in operations that is rich in comparison with the Turing machine. The exact design of the central processor varies considerably, but typically includes operations such as adding two binary integers, or branching to another part of the program if the binary integer in some register is equal to zero. The CPU can interpret information retrieved from memory either as instructions to perform particular operations or as data to apply the current operation to. Thus portion of memory can contain a sequence of instructions, called a program, and another portion of memory can contain the data to be operated on by the program. The CPU repeatedly goes through a fetch-execute cycle. A von Neumann machine runs efficiently because of its random access memory and because its architecture can be implemented in electronic circuitry that makes it very fast.

# 1.1	Components

All computers can be summarized with just two basic components:

- Primary storage or memory

- A central processing unit or CPU.

- The CPU is the "brains" of the computer. Its function is to execute programs that are stored in memory. The CPU fetches an instruction stored in memory then executes the retrieved (fetched) instruction within the CPU before proceeding to fetch the next instruction from memory This process continues until they are told to stop. These programs may involve either its data being process in some manner or results of an operation being stored. Thus we can summarize:

- Fetch instruction:		read instructions from the memory

- Interpret instructions:	decode the instruction to determine the operation to    		                	be performed

- Process data:		execute the instruction

- Write data:			results may mean writing data to memory or an

I/O device

- The ability to process and store data then entails more essential components, the architecture for which is the basic make-up of any computer. The CPU typically consists of:

- A control unit

- An Arithmetic Logic Unit (ALU)

- Registers

1.1.1	CPU Structure

## PC

## DR

## Arithmetic Logic Unit

## AC

## MQ

Instruction Decoder

Legend:

### Control Unit:

## IR

## MAR

## PSW

MAR: memory address register

IR:      instruction register

MBR

MBR: memory buffer register

PC:     program counter

MM

### PSW: program status word

SB

ALU Unit:

AC:   accumulator register

I/O Module

I/O

Module

### DR: general-purpose register

MQ: multiplier-quotient register

###### MM: Main Memory

SB: System Bus

- Organization of a CPU Structure

- Control Unit (CU)

Circuitry located on the central processing unit, which coordinates and controls all hardware. This is accomplished by using the contents of the instruction register to decide which circuits are to be activated. The control unit is also responsible for fetching instructions from the main memory and decoding the instruction.

- Arithmetic-Logic Unit (ALU)

Performs arithmetic operations such as addition and subtraction as well as logical operations such as AND, OR and NOT. Most operations require two operands. One of these operands usually comes from memory via the memory buffer register, while the other is the previously loaded value stored in the accumulator. The results of an arithmetic-logic unit operation are usually transferred to the accumulator (AC).

- Registers

Registers are the smallest units of memory and are located internally within the CPU. And are most often used to either temporarily store results or control information. Registers within the CPU serve two basic functions:

- User-visible registers: these enable to machine/assembly language programs to minimize main memory references by optimizing use of registers.

- Control and Status registers: used by the control unit to control the operation of the CPU and by privileged, operating system programs to control the execution of programs.

Examples include those usually found in the fetch-execute cycle:

- Accumulator (AC, DR)

A register located on the central processing unit. The contents can be used by the arithmetic-logic unit for arithmetic and logic operations, and by the memory buffer register. Usually, all results generated by the arithmetic-logic unit end up in the accumulator.

- Instruction Register (IR)

A register located on the central processing unit, which holds the contents of the last instruction fetched. This instruction is now ready to be executed and is accessed by the control unit.

- Memory Address Register (MAR)

A register located on the central processing unit, which is in turn, connected to the address lines of the system. This register specifies the address in memory where information can be found and can be also used to point to a memory location where information is to be stored.

- Memory Buffer Register (MBR)

A register located on the central processing unit, which is in turn connected to the data lines of the system. The main purpose of this register is to act as an interface between the central processing unit and memory. When the control unit receives the appropriate signal, the memory location stored in the memory address register is used to copy data from or to the memory buffer registers.

- Program Counter (PC)

Contains the memory address of the next instruction to be executed. The contents of the program counter are copied to the memory address register before an instruction is fetched from memory. At the completion of the fetched instruction, the control unit updates the program counter to point to the next instruction, which is to be fetched.

- Program Status Word (PSW)

Generally referred to as a Status Register (SR), this register encapsulates key information used by the CPU to record exceptional conditions. Conditions such as CPU-detected errors (an instruction attempting to divide by zero), hardware faults detected by error checking circuits, and urgent service requests or interrupts generated by the I/O modules.

1.2	Memory

- Memory is made up of a series of zero's (0) and one's (1) called bits or binary. These individual bits are grouped together in lots of eight and are referred to as a byte. Every byte in memory can be accessed by a unique address that identifies its location. The memory in modern computers contains millions of bytes and is often referred to as random-access memory (RAM).

- Memory organization

- This memory is called a N-word m-bit memory

- N generally is in a power of 2, i.e. N=2n

- Size of address is n bits

- Size of word is m bits

- Example: A 4096-word 16-bit memory

Word Length = m bits

Address 0

Address 1						N Words

MAR

2n Words

Uni-directional

Address N-2						address bus

Address N-1

Bit 1

m bits

## MBR

Bi-directional data bus

- A diagram showing the structure of a Main Memory layout

- Memory Read

- Address is placed into the MAR

- Read control is asserted

- Contents of desired location is placed into MBR

- Memory Write

- The word to be written is placed into the MBR

- Address of memory location to be written is specified in MAR

- Write control line is asserted

- Content of MBR is transferred into memory location specified by the MAR

1.3	Input / Output Devices

- Input Devices

- Accepts outside information

- Converts it into digital signals suitable for computation by the CPU

- Output Devices

- Communicates data stored in memory or processed data, to the outside world

- May be in various forms such as a computer Monitor or a Hardcopy

1.4	Bus Interconnection

- A bus is a communication pathway connecting two or more devices

- It is a shared transmission medium. Multiple devices connect to the bus, and a signal transmitted by any one device is available for reception for all other devices attached to the bus.

- Only one device at a time can successfully transmit

- A bus usually consists of multiple communication pathways, or lines, each transmitting either 1 or 0, e.g. an 8-bit unit of data can be transmitted over eight bus lines

- A system bus connects all the major components, i.e. CPU, CU, I/O

- Bus structure

- A system bus typically consists of 50-100 separate lines. Each of which is split amongst the Data, Address and Control lines. The number of lines is usually referred to as the width of the bus.

- Each line is assigned to a particular function

- The bus lines can be classified into 3 functional groupings

- Data Lines

- Address Lines

- Control Lines

## Control Unit

## ALU

## Registers

Main memory

Secondary memory

Input Devices

Output Devices

Data Bus	         Address bus      Control bus

- Diagram of the Bus Interconnection Structure

- Data Lines

- Provides a path for moving data between system modules

- Address Lines

- Used to designate the source or destination of the data on the data bus

- The width of the address bus determines the maximum possible capacity of the system

- Control Lines

- Used to control the access to and the use of data and address lines

- Control signals transmit both command and timing information between system modules

- Timing signals indicate the validity of data and address information

- Command signals specify the operations to be performed

1.5	Instruction-Execution Cycle

All computers have an instruction execution cycle. A basic instruction execution cycle can be broken down into the following steps:

1.Fetch cycle

2.Execute cycle

Start

Fetch instruction

Execute instruction

## Stop

- The fetch-Execute cycle

1.5.1 Fetch Cycle

- To start off the fetch cycle, the address, which is stored in the program counter (PC), is transferred to the memory address register (MAR). The CPU then transfers the instruction located at the address stored in the MAR to the memory buffer register (MBR) via the data lines connecting the CPU to memory. The control unit (CU) coordinates this transfer from memory to CPU. To finish the cycle, the newly fetched instruction is transferred to the instruction register (IR) and unless told otherwise, the CU increments the PC to point to the next address location in memory.

- Steps:

- [PC] -> [MAR]

- [MAR] -> Address bus

- Read control is asserted

- [MEM] -> Data bus -> [MBR]

- [MBR] -> [IR]

- [PC] + 1 -> [PC]

After the CPU has finished fetching an instruction, the CU checks the contents of the IR and determines which type of execution is to be carried out next. This process is known as the decoding phase. The instruction is now ready for the execution cycle.

1.5.2 Execute Cycle

- Once an instruction has been loaded into the instruction register (IR), and the control unit (CU) has examined and decoded the fetched instruction and determined the required course of action to take, the execution cycle can commence. Unlike the fetch cycle (and the interrupt cycle, both of which have a set instruction sequence, which we will see later) the execute cycle can comprise some complex operations (commonly called opcodes).

- Steps:

- [IR] -> decoding circuitry

- If required data are not available in instruction, determine the location

- Fetch the data, if any

- Execute the instruction

- Store results, if any

- The actions within the execution cycle can be categorized into the following four     groups:

- CPU - Memory: Data may be transferred from memory to the CPU or from the CPU to memory.

- CPU - I/O: Data may be transferred from an I/O module to the CPU or from the CPU to an I/O module.

- Data Processing: The CPU may perform some arithmetic or logic operation on data via the arithmetic-logic unit (ALU).

- Control: An instruction may specify that the sequence of operation may be altered. For example, the program counter (PC) may be updated with a new memory address to reflect that the next instruction fetched should be read from this new location.

- For simplicity reasons, the following examples will deal with two operations that can occur. The [LOAD ACC, memory] and [ADD ACC, memory], both of which could be classified as memory reference instructions. Instructions, which can be executed without leaving the CPU, are referred to as non-memory reference instructions.

- LOAD ACC, memory

- This operation loads the accumulator (ACC) with data that is stored in the memory location specified in the instruction. The operation starts off by transferring the address portion of the instruction from the IR to the memory address register (MAR). The CPU then transfers the instruction located at the address stored in the MAR to the memory buffer register (MBR) via the data lines connecting the CPU to memory. The CU coordinates this transfer from memory to CPU. To finish the cycle, the newly fetched data is transferred to the ACC.

- Steps:

- [IR] {address portion} -> [MAR]

- [MAR] -> [MEM] -> [MBR]

- [MBR] -> [ACC]

#### ADD ACC, memory

##### This operation adds the data stored in the ACC with data that is stored in the memory location specified in the instruction using the ALU. The operation starts off by transferring the address portion of the instruction from the IR to the MAR. The CPU then transfers the instruction located at the address stored in the MAR to the MBR via the data lines connecting the CPU to memory. This transfer from memory to CPU is coordinated by the CU. Next, the ALU adds the data stored in the ACC and the MBR. To finish the cycle, the result of the addition operation is stored in the ACC for future use.

- Steps:

- [IR] {address portion} -> [MAR]

- [MAR] -> [MEM] -> [MBR]

- [MBR] + [ACC] -> [ALU]

- [ALU] -> [ACC]

After the execution cycle completes, if an interrupt is not detected, the next instruction is fetched and the process starts all over again.

- Example:

Draw a diagram (including MAR, MBR, IR and PC), show how the instruction MOV AX [7000] is being fetched if the starting address for the instruction is 1FFF and the content of the location at 7000 is ABCD.

- Instruction – Fetch-cycle

PC		  MAR				          MEMORY

1FFF

1FFF

ABCD

(1)		          (2)					7000

IR		   MBR

MOV AX [7000]

MOV AX [7000]

Mov AX[7000]

1FFF

(5)			(4)

(3)

(6) [PC] +1 -> [PC]

Address bus     Data bus

- Instruction – Execution-cycle

IR

(1) Decoding circuitry 	          MEMORY

MOV AX [7000]

ABCD

MAR							7000

(2)			(3)

7000

AX		  MBR

ABCD

ABCD

(4)

(6)			 (5)

Address bus   Data bus

- Fetch –cycle:

- [PC] -> [MAR]				MAR = 1FFF

- [MAR] -> Address bus

- Read control line is asserted

- [MEM]1FFF -> Data bus -> [MBR]		MBR = MOV AX [7000]

- [MBR] -> [IR]					IR      = MOV AX [7000]

- [PC] + 1 -> [PC]

- Execute-cycle:

- [IR] -> decoding circuitry			MAR = 7000

- [MAR] <- 7000

- [MAR] -> Address bus

- Read control line is asserted

- [MEM]7000 -> Data bus -> [MBR]		MBR = ABCD

- [MBR] -> AX					AX     = ABCD

1.5.3 Interrupt Cycle

- An interrupt can be described as a mechanism in which an I/O module etc. can break the normal sequential control of the central processing unit (CPU). And thus alter the way we view the traditional sequence of the fetch and execute cycle. The main advantage of using interrupts is that the processor can be engaged in executing other instructions while the I/O modules connected to the computer are engaged in other operations.

Start

Fetch instruction

Fetch Cycle

Halt

Execute instruction

Execute Cycle					Disabled

Enabled

Check for interrupt

Interrupt Cycle

- Instruction cycle with interrupts

- Common interrupts that the CPU can receive

- Program:	Generated by some condition that occurs as a result of an instruction execution, such as arithmetic overflow, division by zero, attempt to execute an illegal machine instruction, and reference outside a user's allowed memory space.

- Timer:	Generated by a timer within the processor. This allows the operating system to perform certain functions on a regular basis.

- I/O:	Generated by an I/O controller, to signal normal completion of an operation or to signal a variety of error conditions.

- Hardware failure:	Generated by a failure such as power failure or memory parity error.

- Up until now we have dealt with the instruction execution cycle on the hardware level. When interrupts are introduced, the CPU and the operating system driving the system, is responsible for the suspension of the program currently being run, as well as restoring that program at the same point before the interrupt was detected. To handle this, an interrupt handler routine is executed. This interrupt handler is usually built into the operating system.

1.6	Modern Processors

- When central processing units (CPU's) were first developed they processed the first instruction before starting the second. For example, the processor fetched the first instruction, decoded it, and then executed the fetched instruction, before fetching the second instruction and starting the process over again. In a processor, such as the one just described, the CPU itself is the weak link. The external bus operates for at least one cycle (clock pulse) in the three, but has to wait the remaining cycles for the CPU.

Cycle 1	Cycle 2	Cycle 3	Cycle 4

##### Instruction 1	    Fetch	 Execute

Instruction 2				              Fetch 	            Execute

- Sequential execution of program instructions

- Modern processors on the other hand, have developed what are called pipelines. Pipelines are the most common implementation technique in a CPU today that increases the performance of the system. The idea behind the pipeline is that while the first instruction is being executed, the second instruction can be fetched, or in simple terms, instructions overlap.

- The first pipelines to be introduced where a simple three-stage pipeline. While this utilizes all the resources of the system, conflict of resources can occur, resulting in instructions being held until the previous instruction has completed its current stage. Apart from these minor hiccups, it is possible for the CPU to complete an instruction every cycle as opposed to the earlier processors that required three cycles per instruction.

- To overcome the delays associated with the three stage pipeline, modern processors have broken down the execute cycle into a number of phases, some have even broken down the fetch cycle in the fight to overcome delays in their processors. No matter how many phases the cycle is broken down to, the end result is that only one instruction can be completed every cycle.

Cycle 1	Cycle 2	Cycle 3	Cycle 4

##### Instruction 1	    Fetch	 Execute

Instruction 2			   Fetch		Execute

Instruction 3					  Fetch		Execute

- Execution cycle in a three-stage pipelined processor

- Enter the world of superscalar pipeline, where more than one instruction can be issued per clock cycle. Intel describes their processors by different levels. For example a level 2 (L2) processor (Pentium) can issue two instructions per clock cycle and their level 3 (L3) processor (Pentium Pro) can issue 3. The topic of pipelined architecture will be covered in more detail in later chapters.

- Exercises:

- With a diagram, introduce into the fetch-execute cycle a provision for an interrupt.

- The instruction (e.g. LOAD LABEL) loads the data at the address specified in the accumulator, which is a hexadecimal value. With a diagram show how many times the instruction must be fetched. Given that both the opcode field and the main memory are 1 byte wide. The starting address is FEDC16 and the instruction is LOAD 0110.

- The operation ADD ACC, Memory adds the data stored in the ACC with the data that is stored in the memory location specified in the instruction using the ALU. Show the steps of the execution-cycle on the operation.

- With the aid of a diagram, show how the instruction ADD [5000], 10, [1000] is loaded into main memory if the starting address for the instruction is 0200 and the data stored in location 5000 and 1000 are 8 and 8 respectively.

Then…

Show how the instruction, which will add the data at address 5000 with the data in address 1000 and the number 10, and then store the result in address 5000, will be executed.

- Give the definitions and purposes of the following:

- PSW

- ALU

- MM

- MAR

- MBR

- Give a brief description on the concept of pipelined architecture. How does a traditional sequence like the fetch-execute differ in this type of architecture?

- Student Notes:

- Student Notes