2.0	Introduction

- A typical program involves performing a number of functionally different steps, such as adding two numbers, testing for a particular condition, reading a character from the keyboard, or sending a character to be displayed on a monitor. A computer must have instructions capable of performing these four types of operations:

- Data transfer between the main memory and the CPU registers

- Arithmetic and logic operations on data

- Program sequencing and control

- I/O transfers

- The format of an instruction is depicted in a rectangular box symbolising the bits of the instruction code. The bits of the binary instruction are divided into groups called fields. The most common fields found in instruction formats are:

- An operation code field that specifies the operation to be performed.

- An address field that designates either a memory address or a code for choosing a processor registers.

- A mode field that specifies the way the address field is to be interpreted.

- Other special fields are sometimes employed under certain circumstances, as for example a field that gives the number of shifts in a shift type instruction (a concept discussed in more detail in LD201), or an operand field in an immediate type instruction.

The operation code field of an instruction is a group of bits that define various processor operations, such as Add, Subtract, Complement, and shift.

The bits that define the mode field of an instruction code specify a variety of alternatives for choosing the operands from the given address field. The various addressing modes will be discussed in the next chapter.

2.1	Address Field

- Operations specified by computer instructions are executed on some data stored in memory or in processor registers. Operands residing in memory are specified by their addresses. Operands residing in processor registers are specified by a register address. A register address is a binary code of n-bits that specifies one of 2n registers in the processor. Thus a computer with 16 processor registers R0 through to R15 will have in its instruction code a register address field of four bits. The binary code 0101, for example, will designate register R5.Computers may have instructions of several different lengths containing varying number of addresses. The number of address fields in the instruction format of a computer depends upon the internal organisation of its registers. Most instructions fall in one of three types of organisation:

- Single accumulator organisation.

- Multiple register organisation.

- Stack organisation.

2.1.1 Accumulator Organisation

- An accumulator type organisation is the simplest of computer organisations. All operations are performed with the implied accumulator register. The instruction format in this type of computer uses one memory address field.

For example, the instruction that specifies an arithmetic addition has only one address field symbolised by X.

ADD X

- ADD is the symbol for the operation code of the instruction and gives X the address of the operand in memory. This instruction results in the operation AC <- AC + M[X]. AC is the accumulator register and M[X] symbolises the memory word located at address X.

2.1.2	Multiple Register Organisation

- A processor unit with multiple registers usually allows for greater programming flexibility. The instruction format in this type of computer needs three registers. Thus, the instruction for arithmetic addition may be written in symbolic form as

ADD R1, R2, R3

- to denote the operation R3 <- R1 + R2. However, the number of register address fields in the instruction can be reduced from three to two if the destination register is the same as one of the source registers.

ADD R1, R2

- Thus, the instruction will denote the operation R2 <- R2 + R1. Registers R1 and R2 are the source registers and R2 is also the destination register.

Computers with multiple processor registers employ the MOVE instruction to symbolise the transfer of data from one location to another. The instruction

MOVE R1, R2

- Denotes a transfer R2 <- R1. Transfer type instructions need two address fields to specify the source operand and the destination of transfer.

2.1.3	Stack Organisation

- The stack organisation will also be presented in further detail in the next chapter, and later in this section. Computers with stack organisation have instructions that require one address field for transferring data to and from the stack. Operation type instructions such as ADD do not need an address field because the operation is performed directly with the operands in the stack.

- To illustrate the influence of the number of address fields on computer programs, we will evaluate the arithmetic statement

X = ( A + B ) * ( C + D )

Using the One, Two, and Three address instructions.

2.2	Three Address Instructions

OPCODE

OPER 1

OPER 2

OPER 3

OPER 1				DATA SOURCE 1

OPER 2				DATA SOURCE 2

OPER 3				DESTINATION

- Computers with three address instruction formats can use each address field to specify either a processor register or a memory address for an operand. The program in symbolic form evaluates X = ( A + B ) * ( C + D ) is shown below, together with an equivalent register transfer statement for each instruction.

ADD A, B, R1			R1 <- M [A] + M [B]

ADD C, D, R2			R2 <- M [C] + M [D]

MUL R1, R2, X			M [X] <- R1 * R2

- It is assumed that the computer has two processor registers, R1 and R2. The symbol M[A] denotes the operand stored in memory at the address symbolised by A.

- Advantage:	Programming flexibility

- Disadvantage:	Long instruction word

2.3 	Two Address Instructions

OPCODE

OPER 1

OPER 2

OPER 1				DATA SOURCE 1

OPER 2				DATA SOURCE 2

- Implied destination, either oper 1 or oper 2

- Two address instructions are the most common in commercial computers. Here again each address field can specify either a processor register or a memory address. The program to evaluate X = ( A + B ) * ( C + D ) is as follows:

MOVE A, R1			R1 <- M[A]

ADD B, R1			R1 <- R1 + M[B]

MOVE C, R2			R2 <- M[C]

ADD D, R2			R2 <- R2 + M[D]

MUL R2, R1			R1 <- R1 * R2

MOVE R1, X			M[X] <- R1

- The MOVE instruction moves or transfers the operands to and from memory and processor registers. The second listed in the symbolic instruction is assumed (implied) to be the destination where the result of the operation is transferred.

- Advantage:	Smaller instruction word

- Disadvantage:	Implied destination may not be the desired address, needs a data transfer to the actual location.

2.4	One Address Instructions

OPCODE

OPER 1

OPER 1				DATA SOURCE 1

- The other data source is an accumulator or stack

- Implied destination of operation

- A computer with a one-address instruction uses an implied AC register. The program to evaluate the arithmetic statement is as follows:

LOAD A				AC <- M[A]

ADD B				AC <- AC + M[B]

STORE T				M[T] <- AC

LOAD C				AC <- M[C]

ADD D				AC <- AC + M[D]

MUL T				AC <- AC * M[T]

STORE X				M[X] <- AC

- All operations are done between the AC register and a memory operand. The symbolic address T designates a temporary memory location required for storing the intermediate result.

- Advantage:	Smaller instruction word.

- Disadvantage:	Data needs to be loaded into the accumulator first. And the implied destination may not be the desired address, therefore it requires a data transfer to the actual location.

2.5	Zero Address ( Stack ) Instructions

OPCODE

- Most computers have a facility for a memory stack but only a few commercial computers have the appropriate instructions for evaluating arithmetic expressions. Such computers have a stack organised CPU with the top locations of the stack as registers. The rest of the stack is in memory. In this way, the operations that must be performed with the top two elements of the stack are available in processor registers for manipulation with arithmetic circuits.

The PUSH and POP instructions require one address field to specify the source or destination operand. Operation type instructions for the stack such as ADD and MUL imply two operands on top of the stack and do not require an address field in the instruction. The following program shows how the expression will be evaluated, X = ( A + B ) * ( C + D ):

PUSH A			TOS <- A

PUSH B			TOS <- B

ADD				TOS <- ( A + B )

PUSH C			TOS <- C

PUSH D			TOS <- D

ADD				TOS <- ( C + D )

MUL 				TOS <- ( C + D ) * ( A + B )

POP X				M[X] <- TOS

- Advantage:	Smaller instruction word.

- Disadvantage:	Data needs to be loaded into the stack first. And the implied destination may not be the desired address, therefore it requires a data transfer to the actual location.

2.6	Using Different Instruction Formats

- Given the scenario of a computer system, which has a 40-bit wide instruction word, there are 12 instructions in the instruction set (4 bits for the opcode and 36 bits left for the operand). How many bits would by allocated to each of the instruction formats.

2.6.1 	Three Address Format

- 12 bits per operand field

- The number of addressable ranges would be 2n12 = 4096

OPCODE

OPER 1

OPER 2

OPER 3

4 Bits		12 Bits	   12 Bits	     12 Bits

0

## Main Memory

4096

2n12  -1

2.6.2 	Two Address Format

- 18 bits per operand field

- The number of addressable ranges would be 2n18 = 256K

OPCODE

OPER 1

OPER 2

4 Bits		18 Bits	   18 Bits

0

## Main Memory

256K

2n18  -1

2.6.2 	Two Address Format

- 36 bits per operand field

- The number of addressable ranges would be 2n36 = 64M

OPCODE

OPER 1

4 Bits	  36 Bits

0

## Main Memory

64M

2n36  -1

- Exercises:

- Write programs to compute X= ( A + B ) * ( C * D / E ) on each of the following machines:

i)	0-address format

ii)	1-address format

iii)	2-address format

iv)	3-address format

The instruction available for use is as follows:

| 0-address | 1-address | 2-address | 3-address |
| --- | --- | --- | --- |
| PUSH | LOA M | ADD A, B | ADD A, B, B |
| POP | STO M | DIV D, E | DIV D, E, E |
| MUL | ADD M | MUL C, D | MUL C, E, E |
| DIV | SUB M | MUL A, C | MUL B, E, X |
| ADD | MUL M | MOV X, A |  |
| SUB | DIV M |  |  |

- Assume a system has a 24-bit wide instruction word. Calculate the minimum number of bits needed for opcode in order to evaluate the above expression for various machines in part (a). Hence calculate the maximum number of bits for the operand for each of the machines.

- A computer system has a 128-bit instruction word and uses a 3-address format. There are 155 different general-purpose registers available. Assume that the use of any of these registers is required in a particular instruction – i.e. the appropriate register must be specified in the instruction word as a special field. If there are 200 different opcodes/instructions available for the system, what is the instruction format like?

- Student Notes

- Student Notes