3.0	Introduction

- Address fields in a typical instruction format are quite limited in addressable range, therefore it would be better to give them the ability to reference a large range of locations in main memory or, from some systems virtual memory. To achieve this objective, a variety of addressing techniques has been employed. They all involve some trade-off between address range and/or addressing flexibility on the one hand, and a number of memory references and/or complexity of address calculation on the other.

- In many computer systems, the computer will have several different programs.  To efficiently load and remove these programs from memory or different locations, addressing techniques are provided that make the program re-locatable, meaning that the same program can be run in many different sections at memory.

3.1	Addressing Techniques

- All computers provide more than one type of addressing mode. The question arises as to how the control unit can determine which address mode is being used in a particular instruction. Several approaches are taken. Often, different opcodes will use different addressing modes. Also, one or more bits in the instruction format can be used as mode field. The value of the mode field determines which addressing mode to be used. Another thing to note is the effective address, either being a main memory address or a register address.

# 3.1.1	Immediate Addressing

- The operand is given explicitly in the instruction. This mode is used in specifying address and data constants in programs:

MOV 200immediate, R0

- The instruction places the value 200 in register R0. The immediate mode is used to specify the value of a source operand. It makes no sense as a destination because it does not specify a location in which an operand can be stored. Using a subscript to denote the immediate mode is not appropriate in an assembly language. Sometimes, the value is written, as it is, e.g. 200. But this can be confusing. A common convention is to use the pound sign # in front of the value of an operand to indicate that this value is to be used as an immediate operand. Using this convention, the above instruction is written as:

MOV	#200, R0

- Example:

- Load 5000: Move the value 5000 into the accumulator

Accumulator

5000

Load 5000

Main memory

- Immediate addressing technique

3.1.2 Direct Addressing

- To specify a memory address in an instruction word, the most obvious technique is simply to give the address in binary form. Although direct addressing provides the most straightforward (and fastest) way to give a memory address, several other techniques are also used. It requires only one memory reference and no special calculation. However, because of this it has only a limited address space. A typical instruction using direct addressing may look as follows:

MOV AX, [7000]

- In this case the instruction specifies that the contents of [7000], which is a memory location, must be retrieved and placed into the accumulator AX. Common convention for denoting an instruction using direct address is to place a bracket by the side, e.g. [address].

- Example :

- Load 5000: Move the content at address 5000 into the accumulator

Load [5000]

Accumulator

ABCD

ABCD

5000

Main memory

- Direct addressing technique

# 3.1.3	Indirect Addressing

- The effective address of the operand is the contents of a main memory location, the location whose address appears in the instruction. We denote indirection by placing the name of the memory location (or register address as we will see later in this chapter) with a @ symbol. The memory location that contains the address of the operand is called a pointer. In direction is an important and powerful concept in programming. Consider the analogy of a treasure hunt: In the instructions for the hunt you may be told to go to a house at a given address. Instead of finding the treasure there, you find a note that gives you another address where you will find the treasure. By changing the note, the location of the treasure can be changed, but the instructions for the hunt remain the same. Changing the note is equivalent to changing the contents of a pointer in a computer program. An example instruction may look like:

MUL #10, @1000, [2000]

- In this instruction we are asked multiply the value 10 with the contents of whatever is found to be the address at location @1000. For example, after we first point to @1000, the contents are the address 3000, of which we will go there to retrieve a value. The final part of the above instruction now asks us to store the resultant operation in address 2000, found in main memory.

- Example

- Load @5000: Access main memory at address 5000 to get address of actual data. Access main memory at this address to retrieve actual data into accumulator.

Load @5000

ABCD

5000

Accumulator

FACE

ABCD

FACE

Main memory

- Indirect addressing technique

# 3.1.4	Register Addressing

- Similar to direct addressing, however the operand field refers to a register address containing the actual data, instead of a main memory address. If register address is heavily used in an instruction set, this implies that the CPU registers will be heavily used. Because of the severely limited number of registers (compared to main memory locations), their use in this fashion makes sense only if they are employed efficiently. The advantages of register addressing is the access time, however the obvious disadvantage to register addressing is their limited size. A typical instruction could be similar to that of direct address, for example:

MOV AX, R1

- In this case the instruction specifies that the contents of R1, which is a memory location, must be retrieved and placed into the accumulator AX. Common convention for denoting an instruction using register address is to place an R followed by the register location by the side, e.g. R3.

- Example:

- Load R5: Move the content in the register 05 into the accumulator

R1

R2

R3

R4

R5

R6

R7

Load [R5]

Accumulator

Main memory

ABCD

ABCD

- Register addressing technique

# 3.1.5	Register Indirect Addressing

- Similar to indirect addressing, however the operand field points to a register containing the effective address of data. The effective address may be a memory location or a register location. The same advantages and disadvantages that could be said of indirect addressing could also be true here. A typical instruction might be, again similar to indirect addressing, as such:

ADD #20, @R8, R3

In this instruction we are asked add the value 20 with the contents of whatever is found to be in the register address @R8. For example, after we first point to @R8, the contents are the address 2000 (in the this case the effective address is a memory location), of which we will go there to retrieve a value. The final part of the above instruction now asks us to store the resultant operation in register location R3.

- Example

- Load @R3: Access the register 05 to get the address of actual data (Assume this address is a register). Access this register to retrieve actual data into accumulator

R1

R2

R3

R4

R5

R6

R7

Load @R3

R5

Accumulator

Main memory

FACE

FACE

- Register indirect addressing technique

# 3.1.6	Displacement Addressing

- Operand field contains offset or displacement. Uses a special –purpose register whose contents are added to the offset to produce the effective address of data, Basically it combines the capabilities of direct addressing and register indirect addressing. Under displacement addressing, there are three basic techniques, although it should be noted that all are following the same generic method of specifying an effective address:

- Relative addressing (PC)

- Based addressing (BX)

- Indexed addressing (SI)

- Example

- Load [displacement register e.g. PC, BX, or SI + 5]: Move the data at location e.g. (PC + 5) into the accumulator.

## Offset

5

PC

## PC

5001

5000

5001

5002

5003

5004

5005

5006

Load [PC + 5]

5001

+

5006 (Relative Address)

### Offset

ABCD

Accumulator

ABCD

- Displacement addressing technique

3.2	Assembly Language

- Machine instructions are represented by patterns of 0’s and 1’s. Such patterns are awkward to deal with when writing programs. Therefore we use symbols to represent the patterns.

- For example, in the case of the Move and Add instructions, we use the symbolic names MOVE and ADD to represent the corresponding operation code patterns. Similarly we use the notation R3 to refer to register number three.

A complete set of such symbolic names and rules for their use constitutes a programming language generally referred to as assembly language. The symbolic names are called mnemonics; the set of rules for using the mnemonics in the specification of complete instructions and programs is called the syntax of the language.

- Programs written in an assembly language can be automatically translated into a sequence of machine instructions by a special program called an assembler. The assembler, like any other program, is stored as a sequence of machine instructions in the main memory. A user program is usually entered into the computer, at this point the program is simply a set of lines of alphanumeric characters. When the assembler program is executed, it reads the user program, analyses it, and then generates the desired machine language program. The latter contains patterns of 0’s and 1’s specifying the instructions that will be executed by the computer. The user program in its original alphanumeric text format is called the source program, and the assembled machine language program is called the object program.

3.2.1 Instruction Notation

- The assembly language syntax may require us to write the MOVE instruction as

MOVE 	R0, SUM

- The mnemonic, MOVE, represents the operation performed by the instruction. The assembler translates this mnemonic into a binary code that the computer understands. The binary code is usually referred to as the opcode, because it specifies the operation denoted by the mnemonic.

The opcode mnemonic is followed by at least one blank space. Then the information that specifies the operand is given. In the example above, the source operand is in register R0. This information is followed by the specification of the destination operand, separated from the source operand by a comma. The destination operand is in the memory location that has its address represented by the name SUM.

Since there are several possible addressing modes for specifying operand locations, the assembly language syntax must indicate which mode is being used. For example, a numerical value or a name used by itself, such as SUM in the preceding instruction may be used to denote the direct mode. The pound sign usually denotes an immediate operand. Thus the instruction

ADD	#5, R3

- Adds the number 5 to the contents of register R3 and puts the result back into register R3. The pound sign is not the only way to denote immediate addressing. In some cases, the intended addressing mode is indicated by the opcode used. The assembly language may have different opcode mnemonics for different addressing modes.

For example, the previous Add instruction may have to be written as

ADDI	5, R3

- The mnemonic ADDI states that the source operand is given in the immediate addressing mode.

Putting parentheses around the name or symbol denoting the pointer to the operand usually specifies indirect addressing. For example, if the number 5 is to be placed in a memory location whose address is held in register R2, the desired action can be specified as

MOVE	#5, (R2)	or perhaps	    MOVEI	5, (R2)

3.2.2	Number Notation

- When dealing with numerical values, it is often convenient to use the familiar decimal notation. Of course, these values are stored in the computer as binary numbers. In some situations it is more convenient to specify the binary patterns directly. Most assemblers allow numerical values to be specified in different ways, using conventions that are defined by the assembly language syntax. Consider for example, the number 93, which is represented by the 8-bit binary number 01011101. If this value is to be used as an immediate operand, it can be given as a decimal number, as in the instruction

ADD	#93, R1

Or as a binary number identified by a percent sign, as in

ADD 	#%01011101, R1

- Binary numbers can be written more compactly as hexadecimal numbers, in which four bits are represented by a single hexadecimal digit. The hexadecimal notation is a direct extension of the BCD code (binary coded decimal). Where the first ten patterns 0000, 0001, …., 1001 are represented by the digits 0, 1, …., 9, as in BCD. The remaining six 4-bit patterns, 1010, 1011, …., 1111, are represented by the letters A, B, …., F.

- Thus, in hexadecimal representation, the decimal value becomes 5D. In assembler syntax, a hexadecimal representation is often identified by a dollar sign. Therefore we can write

ADD	#$5D, R1

- Exercises

- Student Notes: