5.0	Introduction

- The input and output subsection of a computer provides an efficient mode of communication between the central processing unit and the outside environment. Programs and data must be entered into the computer memory for processing and results obtained from computations must be recorded or displayed to the user. Among the input and output devices that are commonly found in computer systems are keyboards, display terminals, printers and disks. Other input and output devices encountered are magnetic tape drives digital plotters, optical readers, analog-to-digital converters, and various data acquisition equipment. Computers can be used to control various processes such as machine tooling, assembly line procedures, and industrial control.

- The input and output facility of a computer is a function of its intended application. The difference between a small and large system is partially dependent on the amount of hardware the computer has available for communicating with other devices and the number of devices connected to the system, and thus various modes of transfer, or architectures will differ.  However, it can be said that an I/O module does have some generic functions.

5.1	I/O Module Functions

- Control and Timing

To co-ordinate the flow of traffic between the computers internal resources and those of external devices, because of the fact that the CPU will be communicating with multiple devices. Therefore the internal resources, such as main memory and the system are must be shared among a number of activities including data I/O.

- CPU Communication

The CPU must be able to decode specific commands and also send commands to the I/O module such as READ SECTOR and WRITE SECTOR for example. These commands are sent via the control bus. The data bus is utilised for the transfer of data between the CPU and the I/O module. The address bus is used for address recognition, where each I/O device has an address unique to that particular device which is being controlled. Lastly there is status reporting, usually noted in the PSW, to report the current status of an I/O module and to report various error conditions that may have occurred.

- Device Communication

Communicating to external devices in terms of commands, status information of various devices or of the I/O module, and the exchange of data between the I/O module and CPU and vice versa.

- Data Buffering

Data buffering is required because of the different data transfer rate of the CPU, memory and the particular I/O devices. An I/O module must be able to operate at both device and memory speeds.

- Error Detection

Any error that has been detected will be recorded and reported to the CPU, examples of these are a paper-jam, bad disk track, transmission error and so on.

5.2	Accessing I/O Devices

- Most modern computers use a single bus arrangement as shown below:

## Processor

## Memory

BUS

I/O Device 1

I/O Device n

….

- A single bus structure

The processor, memory, and the I/O devices are connected to this bus, which consists of three sets of lines as discussed before, namely the address, data and control lines. Each I/O device is assigned a unique set of addresses. When the processor places a particular address on the address lines, the device that recognises this address responds to the commands issued on the control lines. The processor requests either a read or a write operation, and the requested data is then transferred over the data lines.  When I/O devices and memory share the same address space, the arrangement is called memory-mapped I/O.

- Example of Memory-mapped:

With memory-mapped I/O, any machine instruction that can access memory can be used to transfer data to or from an I/O device. For example, if DATAIN is the address of the input buffer associated with the keyboard, the instruction

MOVE DATAIN, R0

Reads the data from DATAIN and stores them in processor register R0. Similarly, the instruction

MOVE R0, DATAOUT

Sends the contents of register R0 to location DATAOUT, which may be the output data buffer of a monitor or a printer. The memory concept usually applies to the application of programmed I/O in I/O operations, and this is discussed in the next section.

In another type of organisation called I/O mapped, the memory and the I/O address spaces are separate. In this case the CPU must execute separate I/O instructions to activate either the read I/O or write I/O lines, which cause a word to be transferred between the addressed I/O and the CPU.

    - Programmed I/O

- Programmed I/O is most useful in small, low-speed systems where hardware costs must be minimised. Programmed I/O requires that all I/O operations be executed under the direct control of the CPU; in other words, every data-transfer operation involving an I/O device requires the execution of an instruction by the CPU. Typically the transfer is between two programmable registers: one a CPU register and the other attached to the I/O device. The I/O device does not have a direct access to main memory. A data transfer from an I/O device to memory requires the CPU to execute several instructions, including an input instruction to transfer a word from the I/O device to the CPU and a store instruction to transfer the word from CPU to memory.

- The procedural representation of programmed I/O is as follows:

CPU asserts read command to I/O module

CPU  I/O

The CPU reads status of I/O module

I/O  CPU

Not ready

Ready status?

Error Condition

Ready

CPU reads a word from I/O module

I/O  CPU

CPU writes one word to I/O module

CPU  Memory

Transfer complete?

No

Yes

- Programmed I/O flowchart

    - Interrupt Driven I/O

- The problem with programmed I/O is that the CPU has to wait a long time for the I/O module either in the case of transmission or reception of data. The CPU, while waiting, must repeatedly interrogate the status of the I/O module. As a result, the level of the performance of the entire system is severely degraded.

An alternative is for the CPU to issue an I/O command to a module and then goes on to do some other useful work. The I/O module will then interrupt the CPU to request service when it is ready to exchange data with the CPU. The CPU then executes the data transfer, as before, and then resumes its former processing.

CPU asserts read command to I/O module

CPU  I/O

CPU does something else

Read status of I/O module

I/O  CPU

Interrupt

Check status?

Error Condition

Ready

CPU reads a word from I/O module

I/O  CPU

CPU writes one word to I/O module

CPU  Memory

Transfer complete?

No

Yes

- Interrupt Driven I/O flowchart

    - Direct Memory Access (DMA)

- Removing the CPU from the path and letting the I/O device manage the memory buses directly would greatly improve the speed of transfer, as compared to the previous two techniques. This transfer technique known as direct memory access or DMA takes over the buses to manage the transfer directly between the I/O device and memory.

- The CPU may be placed in an idle state in a variety of ways. One common method is to disable the buses through special control signals. The bus request input is used by the DMA to request from the CPU to relinquish control of the buses. When this input is active, the CPU terminates the execution of its present instruction and places the address bus, data bus and the read/write lines into a high-impedance state. After this is done, the CPU activates the bus granted output to inform the DMA that it can take control of the buses. When the bus granted line is enabled, the DMA takes control of the bus system to communicate directly with the memory. The transfer can be made for an entire block of memory words, suspending the CPU operation until the entire block is transferred. This is referred to as burst transfer. The transfer can be made one word at a time between CPU instruction executions as well, this is known as cycle stealing. This is more commonly used to raise the bus request line to the CPU so suspension can begin. The CPU merely delays its operations for one memory cycle to allow the DMA transfer to steal one memory cycle, the effect is more of a pause and hence this is not an interrupt.

CPU generates a read 'block of data' command to DMA module

CPU is free to do anything

DMA interrupts CPU

CPU reads DMA status

Next instruction to be executed

- Cycle Stealing

- The DMA module needs to take control of the bus in order to transfer data to and from memory and only when the CPU does not need it. The diagram below shows the possible breakpoints for the DMA to interrupt the CPU.

INSTRUCTION CYCLE

Processor Cycle

Processor Cycle

Processor Cycle

Processor Cycle

Processor Cycle

Processor Cycle

Fetch Cycle

Decode instruction

Fetch operand

Execute instruction

Store result

Process interrupt

DMA Breakpoints

- Cycle Stealing in DMA

5.3 Interrupts

- Interrupt is an asynchronous event that suspends a CPU from its ordinary operations and quickly jumps to a pre-programmed routine called a handler. The CPU must be able to return to its original operation after executing the service routine.

- The concept of a program interrupt is used to handle a variety of problems that arise out of normal program sequence. Program interrupt refers to the transfer of program control from a currently running program to another service program as a result of an externally or internally generated request. Control returns to the original program after the service program is executed. The interrupt procedure is in principle similar to a subroutine call except for three variations:

- The interrupt is initiated by an external or internal signal rather than from the executions of an instruction.

- The address of the service program that processes the interrupt request is determined by a hardware procedure rather than from the address field of an instruction.

- In response to an interrupt it is necessary to store all the information that defines the state of the computer rather than storing only the program counter.

- After the computer has been interrupted and the corresponding service program has been executed, the computer must return to exactly the same state that it was before the interrupt occurred. Only if this happens will the interrupted program be able to resume exactly as if nothing has happened. The state of the computer at the end of an execution of an instruction is determined from the contents of the program counter and other processor registers and the values of various status bits. The collection of all status bits is sometimes called the program status word (PSW) or the status register (SR). Typically, it includes the status bits from the last ALU operation and it specifies what interrupts are allowed to occur and whether the computer is operating in a user or system mode. Many computers have a resident operating system that controls and supervises all other programs. When the computer is executing programs that are part of the operating system, the computer is placed in system mode, and the computer is set in user mode when user application programs are running. The mode of the computer at any given time is determined from special status bits in the PSW.