6.0	Introduction

- Every computer contains several types of devices to store the instructions and data required for its operation. These storage devices plus the algorithms-implemented by hardware and/or software-needed to manage the stored information form the memory system of the computer.

- A CPU should have rapid uninterrupted access to the external memories where its programs and the data they posses are stored so that the CPU can operate at or near its maximum speed. Unfortunately, memories that operate at speeds comparable to processor speeds are expensive, and generally only very small systems can afford to employ a single memory using just one type of technology. Instead, the stored information is distributed, often in complex fashion, over various memory units that have very different performance and cost.

6.1	Memory Hierarchy

MEMORY HIERARCHY

Increased speed creased cost

CPU REGISTERS

Internal memory type

CACHE MEMORY

MAIN MEMORY

SECONDARY MEMORY

External memory type					increased size

- Representation of the Memory Hierarchy

- CPU Registers:

These high-speed registers in the CPU serve as the working memory for temporary storage of instructions and data. They usually form a general purpose register file for storing data as it is processed. Each register can be accessed, that is, read or written into, within a single clock cycle.

- Cache memory:

Most computers now employ another layer of memory known as the cache, which is positioned logically between the main and secondary memories. A cache's storage capacity is less than that of main  memory, but with an access time of one to three cycles faster.

- Main memory:

Main memory stores programs and data that are in active use. Storage locations in main memory are addressed directly by the CPU's load and store instructions.

- Secondary memory:

This memory type is much larger in capacity but also much slower than main memory. Secondary memory stores system programs, large data files, and the like that are not continually required by the CPU. It also acts as an overflow memory when the capacity of main memory is exceeded. Information in secondary storage is considered to be on-line but accessed indirectly via input/output programs that transfer information between main and secondary memory.

    - Memory performance

The issue of speed, cost and size are always paramount in discussing memory systems. An ideal memory would be fast large and inexpensive. But these chips are expensive because for cost reasons, packing a large number of cells into a single chip is impractical. Of course there are alternative memory types, but these are slower such as secondary storage. During program execution, the speed of memory access is of utmost importance. The key to managing the operation of the hierarchical memory system is to bring the instructions and data that will be used in the near future as close to the CPU as possible, using the mechanisms presented when we discuss about Cache memory can do this. A more formalised categorisation of performance can be viewed from these characteristics:

- Access time: for random-access memory, this is the time it takes to perform a read or write operation, that is, the time from the instant that an address is presented to the memory to the instant that data have been stored or made available for use. For non-random access memory, access time is the time it takes to position the read-write mechanism at the desired location.

- Memory cycle time: this concept is primarily applied to random-access memory and consists of the access time plus any additional time required before a second access can commence. The additional time may be required for transients to die out on signal lines or to regenerate data if they are read destructively.

- Transfer rate: This is the rate at which data can be transferred into or out of a memory unit. For random access memory, it is equal to 1/(cycle time). For random access memory, average time to read/write being equal to average access time + number of bits / transfer rate.

6.1.2	Access Methods

- One of the sharpest distinctions among memory types is the method of accessing units of data. Four types may be distinguished:

- Sequential access: memory is organised into units of data, called records. Access must be made in a specific linear sequence. Stored addressing information is used to separate records and assist in the retrieval process. A shared read/write mechanism is used, and this must be moved from its current location to the desired location, passing and rejecting each intermediate record. Thus the time to access an arbitrary record is highly variable.

- Direct access: as with sequential access, direct access involves a shared read/write mechanism. However, individual blocks or records have a unique address based on physical location. Access is accomplished by direct access to reach a general vicinity plus sequential searching, counting, or waiting to reach the final location. Again, access time is variable.

- Random access: each addressable location in memory has a unique, physically wired-in addressing mechanism. The time to access a given location is independent of the sequence of prior accesses and is constant. Thus, any location can be selected at random and directly addressed and accessed.

- Associative access: this is random-access type of memory that enables one to make a comparison of desired bit locations within a word for a specified match, and to do this for all words simultaneously. Thus a word is retrieved based on a portion of its contents rather than its address. As with ordinary random-access memory, each location has its own addressing mechanism, and retrieval time is constant independent of location or prior access patterns.

6.2	Cache Memory

- The effectiveness of the cache mechanism is based on a property of computer programs called the locality of reference. Analysis of programs shows that most of their execution time is spent on routines in which many instructions are executed repeatedly. These instructions may constitute a simple loop, nested loop, or a few procedures that repeatedly call each other. The actual detailed pattern of instruction sequencing is not important-the point is that many instructions in localised areas of the program are executed repeatedly during some period, and he remainder of the program is accessed relatively infrequently. This is referred to as locality of reference and manifests itself in two ways: temporal and spatial locality.

- Temporal locality means that a recently executed instruction is likely to be executed again very soon.

- Spatial locality means that instructions in close proximity to a recently executed instruction (with respect to the instructions' addresses) are also likely to be executed soon.

- If the active segments of a program can be placed in a fast cache memory, then the total execution time can be reduced significantly. Conceptually, operation of a cache memory is very simple. The memory control circuitry is designed to take advantage of the property of locality of reference. The temporal aspect of the locality of reference suggests that whatever an information item (instruction or data) is first needed, this item should be brought into the cache where it will hopefully remain until it is needed again. The spatial aspect suggests that instead of bringing just one item from the main memory to the cache, it is wise to bring several items that reside at adjacent addresses as well.

    - Cache Organisation

- Memory words are stored in a cache data memory and are grouped into small pages called cache blocks. The contents of the cache's data memory are thus copies of a set of main memory blocks. Each cache block is marked with its block address, referred to as a tag, so the cache knows to what part of memory space the block belongs. The collection of tag addresses currently assigned to the cache, which are noncontinguous, is stored in a special memory, the cache tag memory.

- A diagram of what the cache memory looks like is as follows:

Cache

Data

Memory

Cache

Hit

Cache Tag

Memory

Address

Control

Data

- Basic structure of a cache memory

- One general way of introducing a cache into a computer is as such: a look-aside buffer. The cache and the main memory are directly connected to the system bus. In this particular design the CPU initiates a memory access by placing a (real) address on the memory address bus at the start of a read or write cycle.. The cache compares the address to the tag address currently residing in its tag memory. If a match is found, that is, a cache hit occurs, the access is completed by a read or write operation executed in the cache; main memory is not included. If no match is found in the cache, that is, a cache miss occurs, then the desired access is completed by a read or write operation directed to memory. In response to a cache miss, a block (line) of data from memory is transferred into the cache. This transfer is fast, taking advantage of the small block size and fast random access memory methods, such as a page mode, which allows the cache block to be filled in a single short burst. The cache implements some replacement policy such as LRU to determine where to place an incoming block. When necessary, the cache block is replaced and the block being replaced is transferred from cache back to main memory. Note that cache misses, even though they are infrequent, result in block transfers that tie up the system bus, making it unavailable for other uses like I/O operations.

- This particular operation can be seen in the following diagram:

CPU

MAIN

MEMORY

CACHE

HIT

MISS

Cache access

Block replacement

Main memory access

SYSTEM BUS

- Look-aside cache implementation

    - Replacement Policies

- The placement of blocks of information in a memory system is called memory allocation. The method of selecting the part of memory in which an incoming block is to be placed is the replacement policy. Simple replacement policies assign a block to memory only when an unoccupied or inactive region of sufficient size is available. More aggressive policies pre-empt occupied blocks to make room for the block. The main goal in choosing a replacement policy is to maximise the hit ratio of the faster memory and minimise the miss ratio. Two useful replacement policies are first-in-first-out (FIFO) and least recently used (LRU).

- For example consider a paging system in which the cache has a capacity of three pages. The execution of a program requires reference to five distinct pages. The page address stream formed by the program is 2, 3, 2, 1, 5, 2, 4, 5, 3, and 2.

TIME.		1	2	3	4	5	6	7	8	9	10

ADDRESS	2	3	2	1	5	2	4	5	3	2

TRACE.

2

2

5

3

3

5

5

5

2

2

FIFO

3

3

3

3

2

2

2

2

2

2

1

1

4

4

4

4

HIT				        HIT		HIT

- The action of a FIFO replacement policy

TIME.		1	2	3	4	5	6	7	8	9	10

ADDRESS	2	3	2	1	5	2	4	5	3	2

TRACE.

2

2

2

3

3

2

2

2

2

2

LRU

3

3

3

5

5

5

5

5

5

1

1

1

4

4

4

2

HIT		        HIT	         HIT		HIT

- The action of a LRU replacement policy

    - Hit Rate & Miss Penalty

- An indicator of the effectiveness of a particular implementation of the memory hierarchy is the success rate in accessing information at various levels of the hierarchy. Recall that a successful access to data in a cache memory is called a hit. The number of hits stated as a fraction of all attempted accesses is called the hit rate, and the miss rate is the number of misses stated as a fraction of attempted accesses.

- Ideally, the entire memory hierarchy would appear to the CPU as a single memory unit that has the access time of a cache on the CPU and the size of a device in secondary storage. How close we get to this ideal depends largely on the hit rate at different levels of the hierarchy.

- Performance is adversely affected by the actions that must be taken after a miss. The extra time needed to bring the desired information into the cache is called the miss penalty. This penalty is ultimately reflected in the time that the CPU is stalled because the required instructions or data are not available for execution. In general, the miss penalty is the time needed to bring a block of data from a slower unit in the memory hierarchy to a faster unit. The miss penalty is reduced if efficient mechanisms for transferring data between the various units of the hierarchy are implemented.

- The basic equation for the calculation of access time given the hit and miss concept is as follows. Let h be the hit rate, M the miss penalty, that is, the time to access information in the main memory, and C the time to access information in the cache. The average access time experienced by the CPU is:

tave = hC + ( 1 - h ) M

6.3	Virtual Memory

- In most modern computer systems, the physical main memory is not as large as the address space spanned by an address issued in the processor. When a program does not completely fit into main memory, the parts of it not currently being executed are stored on secondary storage devices. Of course, all parts of a program that are eventually executed are first brought into main memory. When a new segment of a program is to be moved into full memory, it must replace another segment already in memory. In modern computers, the operating system moves programs and data automatically between the main memory and secondary memory. Thus, the application programmer does not need to be aware of limitations imposed by available main memory.

- Techniques that automatically move program and data blocks into the physical main memory when they are required for execution are called virtual-memory techniques. Programs, and hence the processor, reference an instruction and data space that is independent of the available physical main memory space. The binary addresses that the processor issues for either instructions or data are called virtual or logical addresses. These addresses are translated into physical address by a combination of hardware and software components. If a virtual address refers to a part of the program or data space that is currently in the physical memory, then the contents of the appropriate location in the main memory are accessed immediately. On the other hand, if the referenced address is not in the main memory. Its contents must be brought into a suitable location in the memory before they can be used.

- A special hardware unit, called the memory management unit (MMU), translates virtual addresses into physical addresses. When the desired data (or instructions) are in the main memory, these data are fetched and if the data are not main memory, the MMU causes the operating system to bring the data into the main memory from the secondary storage.

PROCESSOR

Virtual address

MMU

Data

Physical address

CACHE

Physical address

Data

MAIN MEMORY

Transfer

SECONDARY

- Virtual memory organisation

- When a program generates an access request to a page that is not in the main memory, a page fault is said to occur - similar to a cache miss. The whole page must be brought in from secondary storage into memory before access can proceed. When it detects a page fault, the MMU asks the operating system to intervene by raising an exception (interrupt). Processing of the active task is interrupted, and control is transferred to the operating system. The operating system then copies the requested page from the disk into main memory and returns control to the interrupted task. Because a long delay occurs while the page transfer takes place, the operating system may suspend execution of the task that caused the page fault and begin execution of another task whose pages are in main memory.

- If a new page is brought from secondary memory when the main memory is full, it must replace one of the resident pages. The problem of choosing which page to remove is just as critical here as it is in the cache, and the idea that programs spend most of their time in a few localised areas also applies. Because main memories are considerably larger than cache memories, it should be possible to keep relatively larger portions of a program in the main memory. This will reduce the frequency of transfers to and from secondary storage.

    - Paging

- A page is a fixed-length block that can be assigned to fixed regions of physical memory called page frames. The advantage of paging as opposed to other virtual memory techniques is that data transfer between memory levels is simplified. An incoming page can be assigned to any available page frame. In a pure paging system, each virtual address consists of two parts: a page address and a displacement (offset), this is found within the page table. Pages themselves should neither be too large or to small. If too large, they heavily utilise valuable space in the main memory, yet too small, many page faults are likely to be incurred.

    - Address Translation

- A method for translating virtual addresses into physical addresses is to assume that all programs and data are composed of pages, the definition of which is given above. They constitute the basic unit of information that is moved between the main memory and the secondary memory whenever the translation mechanism determines that a move is required.

- A virtual-memory address translation method based on the concept of fixed-length pages is shown in the diagram on the next page (6-12). Each virtual address generated by the processor, whether it is for an instruction fetch or an operand fetch/store operation, is interpreted as a virtual page number followed by an offset that specifies the location of a particular word within a page. Information about the main memory location of each page is kept in the page table. This information includes the main memory address where the page is stored and the current status of the page. An area in the main memory that can hold one page is called a page frame. The starting address of the page table is kept in a page table base register. By adding the virtual page number to the contents of this register, the address of the corresponding entry in the page table is obtained. The contents of this location give the staring address of the page if that page currently resides in the main memory.

Virtual address from processor

Page table base register

Page Table Address

Offset

Virtual page number

+

PAGE TABLE

Control

Bits

Page frame in memory

Page frame

Offset

Physical address in main memory

- Virtual-memory address translation

- Exercises: