7.0	Introduction

- Briefly mentioned at the end of chapter one was the concept of a pipelined architecture. In this chapter we will see how pipelining can lead to improved performance. Pipelining is a particularly effective way of organising parallel activity in a computer system. The basic idea is very simple. It is frequently encountered in manufacturing plants, where pipelining is commonly known as an assembly-line operation. For example, in car manufacturing, the first station in an assembly line may prepare the chassis of the car, the next station adds the body, the next one installs the engine and so on. While one group is installing the engine on one car, another group is fitting a car body on the chassis of another car. It make take a car days to be assembled, but it is possible to have a car rolling off the assembly line every few minutes due to this parallel form of sequencing.

  - Basic Concepts

- Consider how the idea of pipelining can be used in a computer. The processor executes a program by fetching and executing instructions, one after the other:

TIME

Fetch

Execute

Fetch

Fetch

Execute

Execute

INSTRUCTION 1

INSTRUCTION 2

INSTRUCTION 3

- A sequential execution

- Now consider a computer that has two separate hardware units, one for fetching instructions and another for executing them:

Storage Buffer

Instruction

Fetch

unit

Instruction

Execution

unit

- Hardware organisation

- The instruction fetched by the fetch unit is deposited in an intermediate storage buffer. The results of execution are deposited in the destination location specified by an instruction. The computer is controlled by a clock whose period is such that the fetch and execute steps of any instruction can each be completed in one clock cycle. In the first clock cycle, the fetch unit fetches an instruction and stores it in the buffer at the end of the clock cycle. In the second clock cycle, the instruction fetch unit proceeds with the fetch operation for the second instruction. Meanwhile, the execution unit performs the operation specified in the first instruction that is available in the buffer. The end of the second clock cycle completes the execution of the first instruction and instruction number two is now available in the buffer. In this manner, both the fetch and the execute units are kept busy all the time.

CLOCK CYCLE		    1	         2	  3             4

INSTRUCTION

1

2

3

F1

E1

E2

F2

E3

F3

- Pipelined execution

- The processing of an instruction need not be divided into only two steps. For example, a pipelined processor may process each instruction in four steps, as follows:

- F: fetch-read the instruction from memory.

- D: decode-decode the instruction and fetch the source operand(s).

- O: operate-perform the operations.

- W: write-store the results in the destination location.

- The sequence of events in this case would look like:

CLOCK CYCLE	    1	         2	  3             4	          5  	  6    	    7

INSTRUCTION

1

2

3

F1

W1

O1

D1

D2

F2

W2

O2

F3

F3

O3

D3

- Instruction execution divided into four parts

- This means that four distinct hardware units are needed. These units must be capable of operating in parallel. Each unit operates on different data, and the result of which is passed to the next unit downstream through a storage buffer. Each buffer holds the information needed by the units downstream to complete execution of an instruction.

Storage Buffer B1

Storage Buffer B2

Storage Buffer B3

Fetch Instruction

F

Decode Instruction

D

Perform Operation

O

Write Results

W

- Hardware organisation

- For example, during clock cycle 4, the information in the buffers are as follows:

- Buffer B1 holds instruction number 3, which was fetched in cycle 3 and being decoded by the instruction-decoding unit.

- Buffer B2 holds both the source operands for instruction 2 and the specification of the operation to be performed, which were produced by the decoding hardware in cycle 3. It also holds the information needed for the write step of instruction 2. This information is not needed by stage 3 of the pipeline, but it must be passed on to stage 4 in the following clock cycle to enable that stage to perform the required write operation.

- Buffer B3 holds the results produced by the operation unit and the destination information for instruction 1.

- With a four-stage pipeline, the rate at which instructions are executed is four times that of sequential operation. It is important to understand that pipelining does not result in individual instructions being executed faster, rather, it is the throughput that increases, where throughput is measured by the number of instructions per second whose execution is completed.

- Based on what we have seen, the increase in performance resulting from pipelining appears to be proportional to the number of pipeline stages. This would be true if pipelined operations could be sustained throughout program execution. Unfortunately, this is not the case. For a variety of reasons, one of the pipeline stages may not be able to complete its processing task for a given instruction in the time allotted. At any time one of the stages in the pipeline cannot complete its operation in one clock cycle, the pipeline stalls. This can be caused by a time-consuming arithmetic operation or by having to access the main memory following a cache miss. Whenever the pipeline is stalled, some degradation in performance occurs. An important goal in designing a pipelined processor is to identify ways to minimise their impact on performance.

CLOCK CYCLE	    1	         2	  3             4	          5  	  6    	    7

INSTRUCTION

1

2

3

F1

W1

O1

D1

D2

F2

W2

O2

F3

O3

D3

- Effect of an operation that takes more than one clock cycle to complete

    - Dependency Constraints

- Consider a program with two instructions. When this program is executed in a pipeline, the execution of the second instruction can begin before the execution of the first instruction is completed. This means that the results generated by the first instruction may not be available for use by the second instruction. We must ensure that the results obtained when instructions are executed in a pipelined processor are identical to those obtained when the same instructions are executed sequentially. Just imagine what would happen to our car in the assembly line if the components were placed in all different orders. The potential for obtaining errors, in the sense of incorrect results, when operations are performed in parallel can be demonstrated with a simple example. Assume that A=5, and then consider this simple operation:

A  3 + A

B  4 * A

- When these operations are performed in the order given, the result is B=32. But if they are performed in parallel, the value of A used in computing B is the original value 5; this leads to an incorrect result. If these two operations are performed by instructions in a program, then the instruction must be executed one after the other, because the data used in the second instruction depends on the result of the first instruction. On the other hand, the two operations can be performed in parallel, because these operations are independent:

A  5 * C

B  20 + C

- This example illustrates a basic constraint that must be enforced to guarantee correct results. No two operations that depend on each other can be performed in parallel. This rather obvious condition has far-reaching consequences. Understanding the implications of this condition is the key to understanding the variety of design alternatives and trade-offs encountered in pipelined computers. In the case of such an incorrect result occurring in a pipelined processor, the pipeline will stall.

- Hence, the dependency just described arises when the destination of one instruction is used as a source in a subsequent instruction. Such dependency can seen as follows:

MUL	R2, R3, R4

ADD	R5, R4, R6

- The result of the multiply instruction is placed into register R4. Assuming that the multiply operation takes one clock cycle to complete, execution of the MUL instruction would then proceed. As the decode unit begins decoding the ADD instruction in cycle 3, it realises that R4 is used as a source operand. Hence, the D step of that instruction cannot be completed until the W step of the multiply instruction has been completed. As a result the pipelined execution has stalled.

- Exercises

- What is pipelining?

- What is meant by dependency constraints?

- Consider the following sequence of instructions:

ADD	#20, R0, R1

MUL	#3, R2, R3

ADD 	R0, R2, R5

In all instructions, the destination operand is given last. Initially, registers R0 and R2 contain 2000 and 50 respectively. These instructions are executed in a computer that has a four-stage pipeline similar to that in your study guide. Assume that the first instruction is fetched in clock cycle 1, and that instruction fetch only requires only one clock cycle.

- Draw a diagram to describe the operation being performed by each pipeline stage during each of the clock cycles 1 through 4, regardless if you manage to execute all instructions.

- Give the contents of the interstage buffers, B1, B2, and B3, during clock cycles 2, 4, and 5.

- Repeat problem (3) for these instructions in Program X (the instructions are related to this program), but in this instance, draw the accompanying memory map if, memory addresses [1000], [2000], and [3000] contain the values 3000, 2000, and 1000 respectively.

MUL	#20, #20, [1000]

- ADD	[1000], [2000], [3000]

- Student Notes: