### Number Systems

123.625x

- If

- x=8, convert to base 2.

Ans.

As we know that,

1=001

2=010

3=011

So number becomes,

123=001010011

and for the fractional part

6=110

2=010

5=101

So the number becomes,

123.625(8)  =  1010011.110(2)

- x =10,  convert to base 2.

Ans.

6	   5	4	3	2	1	0

64	  32	16	8	4	2	1

1	   1	1	1	0	1	12

0.625 x 2

1.250 x 2

0.500 x 2

1.000

123.62510 =  1111011.1012

- x = 16, convert to base 2

Ans.

As we know that,

1 = 0001

2 = 0010

3 = 0011

So the number becomes

123 = 000100100011

and for the fractional part

6 = 0110

2 = 0010

5 = 0101

So the number becomes

123.625(16)  = 100100011.110(2)

- Add  - 1101.0110(2) to part (i) in two’s complement using 10/6 format.

Ans.

123.6258  =001010011.110010101 2

101.00111100101012  *   24

Sign bit

| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sign bit

-1101.01102

0.110101100   *   24

0.001010100 *  24

Sign bit

| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sign bit

Now adding the mantissa of the above two numbers.

- 001111001

- 001010100

1   011001101

Now storing the number in 10/6 format in two’s complement format.

Sign bit

| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sign bit

c. What is the percentage error in storing using the 10/6 format.

The percentage error of the first number is given below.

001111001

* 100 = 6.23%

011110010101.001111001

###### B. Boolean Algebra

- The diagram below illustrates a seven segments LCD display design and a partial truth table. The column in segment a is ‘1’ if the digit 0,2,3,5,7,8,9 are displayed else don’t care ‘x’ or ‘0’.

- Construct the entire truth table for segment “a” only.

Ans.

| ### Decimal Number | #### Input |  |  |  | Output |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | a |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 2 | 0 | 0 | 1 | 0 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 |
| 7 | 0 | 1 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 |
| 10 | 1 | 0 | 1 | 0 | X |
| 11 | 1 | 0 | 1 | 1 | X |
| 12 | 1 | 1 | 0 | 0 | X |
| 13 | 1 | 1 | 0 | 1 | X |
| 14 | 1 | 1 | 1 | 0 | X |
| 15 | 1 | 1 | 1 | 1 | X |

- Obtain the unsimplified expression.

Ans.

ABCD +  ABCD + ABCD + ABCD + ABCD + ABCD + ABCD

- Simplify the expression using Karnaugh Map and get the SOP.

Ans.		   AB      CD

00           01         11         10

00

01

11

10

| 1 ## BD |  | 1 | 1 |
| --- | --- | --- | --- |
|  | 1 | 1 |  |
| X ## A | X | X | X |
| 1 | 1 | X | X |

BD

CD

Minimum SOP expression = A + B   D + BD + CD

- Use Boolean algebra to change the SOP expression to the POS.

Ans.

Maximum POS expression = ( A + B + C + D )  (  A + B + C + D )  (  A + B + C + D )

The simplified POS expression = (A + C + D) (A + B + C + D )

- Draw the circuit for the simplified SOP.

Ans.  The following is the circuit for the simplified SOP

a

A

C

D

B

D

DEBUGGING

Briefly describe what this device is and the purpose of the logic gates which are numbered in the device.

Ans.

The device shown is a SR Flip Flop

It shares two properties:

- These are synchronous bitable devices. Which exists in one of two states and in the absence of input, remains in the same state. Thus it can function as a 1-bit memory.

- The flip-flop has two outputs, which are always the complement of each other. These are generally labeled Q and Q.

The truth table for the circuit is given below.

| Input | Read write | Select | Output |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 |

From the above truth table we can see that the flip-flop is working in the reset state. The reset state is one in which the Q output of the S-R flip-flop is low.

Transistor-Transistor Logic

With the rapid development of integrated circuits (ICs), new problems were encountered and new solutions were developed. One of the problems with DTL circuits was that it takes as much room on the IC chip to construct a diode as it does to construct a transistor. Since "real estate" is exceedingly important in ICs, it was desirable to find a way to avoid requiring large numbers of input diodes. But what could be used to replace many diodes? Well, looking at the DTL NAND gate below, we might note that the opposed diodes look pretty much like the two junctions of a transistor.

In fact, if we were to have an inverter, it would have a single input diode, and we just might be able to replace the two opposed diodes with an NPN transistor to do the same job. In fact, this works quite nicely. What's even better is that we can easily construct a transistor with multiple emitters, using almost the same space on the IC as a standard transistor. The figure below shows the result.

One problem shared by all logic gates with a single output transistor and a pull-up collector resistor is switching speed. The transistor actively pulls the output down to logic 0, but the resistor is not active in pulling the output up to logic 1. Due to inevitable factors such as circuit capacitances and a characteristic of bipolar transistors called "charge storage," it will take a certain amount of time for the transistor to turn completely off and the output to rise to a logic 1 level. This limits the frequency at which the gate can operate. . 1

Emitter-Coupled Logic

Emitter-Coupled Logic is based on the use of a differential amplifier to amplify the digital signal, and emitter followers to logically combine input signals. As a result, none of the transistors in the gate ever enter saturation, nor do they ever get turned completely off. The transistors remain entirely within their active operating regions at all times. As a result, the transistors do not have a charge storage time to contend with, and can change states much more rapidly. Thus, the main advantage of this type of logic gate is extremely high speed. This type of circuit measures logic levels according to the amount of current flowing, rather than according to a voltage level. Accordingly, Emitter-Coupled Logic is also known as Current Mode Logic (CML). This leads us to a major drawback of this type of gate: it draws a great deal of current from the power supply, and hence tends to dissipate a significant amount of heat. To minimize this problem, some devices such as frequency counters use an ECL decade counter at the input end of the circuitry, followed by TTL or high-speed CMOS counters at the later digit positions. This puts the fast, expensive IC where it is absolutely required, and allows us to use cheaper ICs in locations where the signal will never be at that high a frequency2

Metal-Oxide Semiconductor (MOS)

The operation of an MOS transistor is considerably easier to explain than that of a bipolar transistor. The voltage-controlled switches correspond directly to MOS transistors. An MOS transistor is nothing more than a voltage-controlled switch. It has three connection points: a source, a drain, and a gate (a transistor gate bears no resemblance to a logic gate, an unfortunately ambiguous use of the term). A cross section of the metal-oxide-silicon sandwich that forms the transistor is shown below

The bottommost material layer is made of silicon, an insulating oxide layer sits on top of it, and the topmost layer is the metal gate. (More modern integrated circuit processes have replaced the metal layer with a material called polycrystalline silicon, but the older "metal gate" terminology still holds.) The source and drain regions contain silicon material with a large excess of electrons separated by the slightly positively charged bulk silicon. The source and drain are called diffusion regions because of the chemical process used to create them. Negatively charged ions (atoms with extra valence electrons) are placed onto the silicon surface and are diffused into the surface by heating the silicon material. The materials of the source and drain are identical. By convention, the source is the electrical node with the lower of the two voltage potentials at either end of the channel.

The electrical behavior of the transistor is generally as follows. When a positive voltage is placed on the gate, electrons from the silicon bulk are attracted to the transistor channel, an initially non conducting region between the source and drain very close to the silicon surface. When the gate voltage becomes sufficiently positively charged, enough electrons are pulled into the channel from the bulk to establish a charged path between the source and the drain. Electrons flow across the transistor channel, and the voltage-controlled switch is conducting. If a 0 or very small voltage is placed on the gate, no electrons (or at least very few) are attracted to the channel. The source and drain are disconnected, no current flows across the channel, and the switch is not conducting.

Because they are made from materials with different affinities for electrons, the two transistor types behave quite differently. The transistor operation described above is actually for the nMOS transistor. The bulk is positively charged, while the diffusion is negatively charged. The transistor switch is "closed" (conducting) when a logic 1 is placed on its gate and "open" (non conducting) when the gate is connected to a logic 0. The pMOS transistor is complementary. The diffusion regions are positively charged and the silicon bulk is negatively charged. A pMOS transistor behaves in a complementary way: It is "closed" (conducting) when a logic 0 is placed on the gate and is "open" (non conducting) when a logic 1 is placed there.

The symbols for the two different kinds of transistors make it easy to remember how they operate. An nMOS transistor conducts when the gate voltage is asserted in positive logic. The pMOS transistor conducts when the gate is asserted in negative logic. This is why there is a polarity bubble on the gate of the pMOS transistor's symbol.

Any logic gate can be constructed from a combination of nMOS and pMOS transistors.

The figure above shows transistor networks for (a) an inverter, (b) a two-input NAND gate, and (c) a two-input NOR gate. The power supply (+5 V) and ground (0 V) represent logic 1 and logic 0, respectively. The inverter is constructed from an nMOS and a pMOS transistor connected in series between power and ground. It operates as follows (see figure below).

When A is a logic 1, the nMOS transistor is conducting and the pMOS transistor is not. The only unbroken connection path is from ground to the output node. Thus, a logic 1 at the input yields a logic 0 at the output. Now let's look at the case in which A is a logic 0. Now the pMOS transistor conducts while the nMOS transistor does not. The output node is connected to a logic 1. A 0 at the input yielded a 1 at the output. The series transistors implement an inverter."3

Complimentary Metal-Oxide Semiconductor

CMOS logic is a newer technology, based on the use of complementary MOS transistors to perform logic functions with almost no current required. This makes these gates very useful in battery-powered applications. The fact that they will work with supply voltages as low as 3 volts and as high as 15 volts is also very helpful.  CMOS gates are all based on the fundamental inverter circuit shown below.

Note that both transistors are enhancement-mode MOSFETs; one N-channel with its source grounded, and one P-channel with its source connected to +V. Their gates are connected together to form the input, and their drains are connected together to form the output. The two MOSFETs are designed to have matching characteristics. Thus, they are complementary to each other. When off, their resistance is effectively infinite; when on, their channel resistance is about 200 . Since the gate is essentially an open circuit it draws no current, and the output voltage will be equal to either ground or to the power supply voltage, depending on which transistor is conducting. When input A is grounded (logic 0), the N-channel MOSFET is unbiased, and therefore has no channel enhanced within itself. It is an open circuit, and therefore leaves the output line disconnected from ground. At the same time, the P-channel MOSFET is forward biased, so it has a channel enhanced within itself. This channel has a resistance of about 200  , connecting the output line to the +V supply. This pulls the output up to +V (logic 1).  When input A is at +V (logic 1), the P-channel MOSFET is off and the N-channel MOSFET is on, thus pulling the output down to ground (logic 0). Thus, this circuit correctly performs logic inversion, and at the same time provides active pull-up and pull-down, according to the output state. This concept can be expanded into NOR and NAND structures by combining inverters in a partially series, partially parallel structure.

The circuit above and below are a practical example of a CMOS 2-input NOR gate.

In this circuit, if both inputs are low, both P-channel MOSFETs will be turned on, thus providing a connection to +V. Both N-channel MOSFETs will be off, so there will be no ground connection. However, if either input goes high, that P-channel MOSFET will turn off and disconnect the output from +V, while that N-channel MOSFET will turn on, thus grounding the output.  The structure can be inverted, as shown below.

Here we have a two-input NAND gate, where a logic 0 at either input will force the output to logic 1, but it takes both inputs at logic 1 to allow the output to go to logic 0. This structure is less limited than the bipolar equivalent would be, but there are still some practical limits. One of these is the combined resistance of the MOSFETs in series. As a result, CMOS totem poles are not made more than four inputs high. Gates with more than four inputs are built as cascading structures rather than single structures. However, the logic is still valid. Even with this limit, the totem pole structure still causes some problems in certain applications. The pull-up and pull-down resistances at the output are never the same, and can change significantly as the inputs change state, even if the output does not change logic states. The result is uneven and unpredictable rise and fall times for the output signal. This problem was addressed, and was solved with the buffered, or B-series CMOS gates.  The technique here is to follow the actual NAND gate with a pair of inverters. Thus, the output will always be driven by a single transistor, either P-channel or N-channel. Since they are as closely matched as possible, the output resistance of the gate will always be the same, and signal behavior is therefore more predictable. One of the main problems with CMOS gates is their speed. They cannot operate very quickly, because of their inherent input capacitance. B-series devices help to overcome these limitations to some extent, by providing uniform output current, and by switching output states more rapidly, even if the input signals are changing more slowly. 4

I2L CIRCUITS

Integrated injection logic (I2L) Is a bipolar technology that allows an extremely high component density on a chip (up to ten times that of TTL). The I2L technology it being used for same complex LSI functions, such as microprocessors, and is Simpler to fabricate than either TTL or MOS. It also has a low power requirement and reasonably good switching speeds that are improving all the time.

The basic I2L gate is extremely simple, as indicated in Figure. Transistor Q1 act as a current source and active pull-up, and the multiple-collector transistor Q2 Operates as an inverter.

I2L Gate

Notes

1John Bayko

2PC Guide

3 J. Gosper

4Hookey

5Hookey

6Hookey

7Hookey

8Hookey

9Hookey

10Katz Appendix B

11Hookey

12 J. Gosper

13Hookey

14Hookey

Works Cited

J. Gosper, Jeffrey Dr.  "Simple Logic Gates."  September 1998.  www.brunel.ac.uk/~castjjg/hndcfund/material/logic/part5.htm.

"Basic Logical Functions and Gates."  http://www.play-hookey.com/digital/basic_gates.html.

Charles M. Kozierok. .  "Functions and Digital Logic."  August 15, 1999.  http://www.pcguide.com/ref/cpu/rootsFunctions-c.html.

John Bayko.  "Great Microprocessors of the Past and Present ."  January 1999.  http://www.cs.uregina.ca/~bayko/cpu.html.

Katz, Randy H. .  Contemporary Logic Design.  California: Addison Wesley Publishing Company, 1993.