The MOSFET:

Metal-oxide semiconductor field-effect transistors (MOSFETs) are the active switching elements in CMQS-circuits. These devices differ greatly in construction and internal operation from BLT’s but the switching action is basically the same: They function ideally as open or closed switches, depending on the input.

Figure (a) shows the symbols for both n-channel and p-channel MOSFETs. As indicated, the three terminals of a MOSFET are gate, drain, and source. When the gate

of an n-channel MOSFET is more positive than the source, the MOSFET is on, and

there is a low value resistance between the drain and the source (RNO). When the gate-to-source voltage is zero, the MOSFET is off and there is an extremely high resistance between the drain and the source (ROFF). This operation is illustrated in Figure. The p-channel MOSFET operates with opposite voltage polarities, as shown in part (c). Ideally, RON and ROFF can be neglected, and the MOSFET can he considered as a dosed switch when it is on and an open. switch when it is off.

CMOS Inverter:

Complementary MOS (CMOS) logic uses the MOSFET as its basic element. It uses both p-channel ant n-channel enhancements MOSFET as shown in the inverter circuit in –Figure.

CMOS inverter circuit

When a HIGH is applied to the input, the p-channel MOSFET Q1 is off, and the n-channel MOSFET Q2 is on. This condition connects the output to ground through the on resistance of Q2, resulting in a LOW output. When a LOW is applied to the input, Q1 is on and Q2 is off. This condition connects the output +VDD through the on resistance of Q1, resulting in a HIGH output. The operation is illustrated in Figure

Operation of CMOS inverter

CMOS NAND Gate:

The operation of a CMOS NAND gate is as follows:

When both inputs are LOW, Q1 and Q2 are on, and Q3 and Q4 are off. The output is pulled HIGH through the on resistance of Q1 and Q2 in parallel.

When input A is LOW and input B is HIGH Q1 and Q4 are on, and Q2 and Q3 are off. The output is pulled HIGH through the low on resistance of Q1.

When input A is HIGH and input B is LOW, Q1 anal Q4 are of, and Q2 and Q3 are on. The output is pulled HIGH through the low on resistance of Q2.

Finally, the both inputs are HIGH, Q1 and Q2 are off and Q3 and Q4 are on. In this case, the output is putted LOW through the on resistance of Q3 and Q4 in series to ground.

CMOS NAND Gate Circuit

CMOS NOR Gate:

The operation of a CMOS NOR gate is as follows:.

- When both inputs are low, Q1 and Q2 are on, and Q3 and Q4 are off. As a result, the output is pulled HIGH through the on resistance of Q1 and Q2 in series.

	When input A is LOW and input B is HIGH, Q and Q4 are on, and Q2 and Q3 are off. The output is pulled LOW through the low on résistance of Q4 to ground.

	When input A is-HIGH and input B is Low, Q1 and Q4 are off, and Q2 and Q3 are on. The output is pulled LOW through the on resistance of Q3 to ground.

	When both inputs are HIGH, Q1and Q2 are off, and Q3 and Q4 are on. The output is pulled Low through the on resistance of Q3 and Q4 in parallel to ground.

CMOS NOR Gate Circuit

Open-Drain Gates:

An open drain gate is the CMOS- counterpart of an open-collector TTL gate. An open-drain output circuit is a single n-channel MOSFET as shown in Figure (a). As with open-coIIector TTL, a pull-up resistor must he used, as shown in part (b), to produce a HIGH output state. Also, like open-collector TTL, open-drain outputs can he connected in a wired-AND configuration.

Open drain gates

Tristate Gates:

The circuitry in tristate CMOS gate allows each of the output transistors Q1 and Q2 to he turned off at the same time, thus disconnecting the output from the rest of the circuit.

When the enable input is Low, the device is enabled for normal logic operation. When the enable is HIGH, both Q1 and Q2 are off, and the circuit is in the high-Z state.

Precaution for handling CMOS:

-	All CMOS devices are subject to damage from electrostatic discharge (ESD). Therefore, they must he handled with special care. Note the following precaution:

- All CMOS devices are shipped in conductive foam to prevent electrostatic charge buildup. When they are removed from the foam, the pins should not be touched.

- The devices should be placed with pins down on a grounded surface such as a metal plate, when removed from protective material. Do not place CMOS devices in polystyrene foam or plastic trays.

- All tools, test equipment, and metal, workbenches should be earth grounded. A person working with CMOS devices should in certain environments have his or her wrist grounded with a length of cable and a large value resistor The resistor prevents serve shock should the person come in contact with a voltage source.

- Do not insert CMOS devices (or any other ICs) into sockets or PC boards with the power on.