TTL CIRCUITS:

The integrated circuit operation of standard TTL  logic gates with totem-pole outputs is covered.

The Bipolar junction Transistor:

The bipolar junction transistor (BJT) is the active switching element used in all TTL circuits Figure shows the symbol for an npn BJT with its three terminals: base, emitter, and collector. A BIT has two junctions, the base-emitter junction and the base-collector junction.

The basic switching operation is as follows. When the base is approximately 0.7 V more positive than the emitter is and when sufficient current is provided into the base, the transistor turns on and goes into saturation. In saturation, the transistor ideally acts like a closed switch between the collector and the emitter. When the base is- less than 0.7 V more positive than the-emitter, the transistor turns off and becomes-an open-switch between the collector and the emitter. To summarize in general terms, a HIGH on the base, turns the transistor on and makes it a closed switch. A LOW on the base turns the transistor off and makes it an open switch. In TTL, some BJTs have multiple emitters.

The symbol of BJT

TTL Inverter:

Transistor-Transistor logic (TTL) is one of the most widely used integrated circuit technologies. The logic function of an inverter or any type of gate is always the same, regardless of the type of circuit technology that is used. Using TTL technology is only one to build an inverter or any other logic circuit. Figure shows a standard TTL circuit for an inverter. In this figure, Q1 is the input coupling transistors and D1 is the input clamp diode. Transistor Q2 is called phase splitter and the combination of Q3 and Q4 forms the output circuit often referred to as a totem pole arrange.

When the input is d HIGH, the base-emitter Junction of Q1 is reverse biased, and the base collector junction is forward biased. This condition permits current through R1 and the base collector junction of Q1 into the base of Q2, thus driving Q2 into, saturation. As a Result, Q3 is turned on by Q2, and its collector voltage, which is the output, is near ground potential. We-therefore have a LOW output for a HIGH input. At the same time, the collector of Q2 is at a sufficiently low v6ltage level to keep Q4 off.

When the input is IOW, the base-emitter junction of Q1 is forward biased, and the base-collector junction is reverse-biased There is current through R1 and the base-emitter junction of Q1 to the LOW input. A LOW provides a path to ground for the current. There is no current into the base of Q2, so it is off. The collector of Q2 IS HIGH, thus turning Q4 on. A saturated Q4 provides a low-resistance path from VCC to the output; we therefore have a HIGH on the output for a LOW on the input. At the same time, the emitter of Q2 is at ground potential keeping Q2off.

Diode D1 in the TTL circuit prevents negative spikes of voltage on the input -from damaging Q1. Diode D2 ensures that Q4 will turn off when Q2 is on (HIGH input). In this condition the collector voltage of Q2 is equal to the bate-to-emitter voltage, VBE, of Q3 plus the collector-to-emitter voltage VCE of Q2. Diode D2 provides an additional VBE equivalent drop in series with the base-emitter junction of Q4to ensure its turn-off when Q2 is on.

The operation of the TTL inverter for the two input -states is illustrated in Figure. In the circuit in part (a), the base of Q1 is 2.1 V above ground, so Q2and Q3 are on. In the circuit in part (b), the base of Q1 is about 0.7 V above ground-not enough to turn Q2and Q3 on.

A standard TTL inverter circuit

Operation of TTL inverter

TTL NAND Gate:

A 2-input standard TTL NAND gate is shown in Figure, It is the same as the inverter circuit except for the additional input emitter of Q1 In TTL technology multiple-emitter transistors are used for the input devices. These multiple-emitter transistors can be compared to the diode arrangement, as shown in Figure.

Perhaps, you can understand the operation of this circuit better by visualizing Q1 in Figure 15-19 replaced by the diode arrangement. A LOW on either input A or input B forward-biases the respective diode and reverse-biases D3 (Q1 base-coIIector junction). This action keeps Q2 off and results in a HIGH output in the same way as described for the TTL inverter. Of course LOW on both inputs will do the same thing.

A HIGH On both input reverse-biases both input diodes and forward-biased D3 (Q1 base-collector junction). This action turns Q2 on and results in a LOW output in the same, way as described for the TTL inverter.

You should recognize this operation as that of the NAND function. The output is LOW if and only if inputs are HIGH.

A standard TTL NAND Gate circuit

TTL NOR Gate:

A 2-input standard TTL NOR gate is shown in Figure. Compare this with the NAND gate circuit, and notice the additional transistors. Both Q1 and Q2 are input transistors. Transistors Q3 and Q4 are in parallel and acts as phase splitter. You should recognize Q5 and Q6 as the ordinary TTL totem-pole output circuit.

TTL NOR Gate Circuit

The operation of the TTL- NOR gate is as follows:

	When both inputs are LOW, the forward-biased base-emitter junctions of the input transistors pull current away from the phase-splitter transistors, Q3, and Q4, keeping them off. As a result, Q5 is on and Q6 is off, producing a HIGH output.

	When input A is LOW and input B is HIGH, Q3 is off andQ4 is on. Transistor Q4 turns on Q6 and turns off Q5, producing a LOW output.

- When input A is HIGH and input B is LOW, 'Q3 is on and Q4 is off. Transistor Q4 turns on Q6 and turns off Q5, producing a LOW output.

	When both inputs are HIGH, both Q3 and Q4 are on. Having both on has the same effect as having either one on, turning Q6 on and Q5 off. The result is still LOW output.

We should recognize this operation as that of the NOR function. The output is LOW when any of the input is HIGH.

Open-collector gates:

The TTL gates described in the previous sections all had the totem-pole output circuit. Another type of output available in, TTL integrated circuits is the Open-collector output. A standard TTL inverter with an open-collector is shown in Figure l5-23(a). The other types of gates are also available with open-collector outputs.

Notice that the output is, the collector of transistor Q3 with nothing connected to it hence the name open-collector. In order to get the proper HIGH and LOW, logic levels out of the circuit, an external pull-up resistor must be connected to VCC from the collector of Q3as also shown in Figure. When Q3 is off, the output is pulled-up to VCC through the external resistor. When Q3 is on, the output is connected to near-ground through the saturated transistor.

The ANSI/IEEE standard symbol that designates as open-collector output is shown in Figure turns in inverter.

# TTL inverter with open-collector output

Gates with Tristate Outputs

The tristate output combines the advantages of the totem-pole Open-collector circuits. The three output states are HIGH, LOW, and high intendance (high-Z). When selected for normal logic-level operation, as determined by the state of the enable input a tristate circuit operates in. the same way as a regular gate. When tristate circuit is selected for high-Z operation the output is effectively disconnected from the rest of the circuit. Figure 15-25 illustrates the Operation of a tristate circuit. The inverted triangle designates a tristate output.

Figure shows the basic circuit for a TTL tristate inverter. When the enable input is LOW, Q2 is off, and the output circuit Operates as a normal totem-pole configuration, in which the output state depends on the input state. When the enable input is HIGH, Q2 is on. There is thus LOW on the second emitter of Q1 causing Q3 and Q5 to turn off, and diode D1 is forward biased causing also to turn off. When totem-pole transistors are off, they are effectively open and the output is completely disconnected from the internal circuitry, as illustrated in figure.

Low-Power IR (54L/74L):

The 54L/4L series of TTL circuit is designed for low power consumption. A typical gate circuit is shown in Figure 15-29. Notice that the circuit’s resistor values are considerably higher than those of the standard gate in Figure 15-28.The higher resistance, Of course, results in less current and therefore, less power but increases the switching time of the gate. The typical power dissipation of a standard 54/74 gate is 10mW, and that of a 54L/4L gate is 1rnW. The saving of power, however, is paid for in loss of speed. A typical standard 54/74 gate has a propagation delay time of 10ns, compared with 33ns for a 54L/4L gate.

54/74L series TTL NAND Gate

SchottkyTTL (54S/74S):

The 5451745 series of TTL circuits provides a faster switching time by incorporating Schottky diodes to prevent the transistors from going into saturation, thereby decreasing the time for a transist6r to turn on or off Figure 15-30 shows a Schottky gate circuit. Notice the symbols for the Schottky transistor and Schottky diodes. Typically, propagation delay for a 54S/74S gate is 3ns and the power dissipation is 19 mW.

## 54S/74S TTL NAND Gate

Low-Power Schotty TTL (54LS/74LS) :

This TTL series compromises speed to obtain lower power dissipation: A NAND gate circuit is shown in figure 15-31. Notice that the input uses Schottky diodes rather than the conventional transistor input. The typical power dissipation for a gate is 2 mW, and the propagation delay is 10 ns.

54LS/74LS TTL NAND Gate

Advanced Schottky and Advance Low-Power Schottky (AS/ALS):

These technologies are advanced versions of the S and Ls series. The typical static power dissipation is 8.5mW for the AS series and 1 mW for the ALS series. The typical propagation delay time is 1.5 ns for the AS and 4 ns for the ALS. One version of the AS series is called the F or FAST series.